3GPP TSG-RAN WG4 Meeting #91                 	R4-1906366

Reno, Nevada, US, 13th – 17th May 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PRACH simulation results

Agenda item:	6.12.2.4.1

Document for:	Discussion

Introduction

In the RAN4#88bis meeting, a way forward was agreed for NR PRACH demodulation in [1]. In this contribution we provide updates to our previous simulation results with the simulation assumptions agreed in the WF.
A finalized procedure to arrive at performance requirement SNR numbers from the contributions of all companies was furthermore agreed in [2] and the corresponding script is provided in [3].

Simulation Results

Simulation Parameters

In the following we will provide simulation results based on the simulation assumptions as agreed earlier for NR PRACH in [1].

Table 1: Simulation Parameter for NR PRACH

|  |  |  | Ncs | v | Frequency Offset (Hz) | Channel type |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 4 GHz | Format 0 | 1.25kHz |  | 32 | 400 | AWGN and TDL |
|  | Format A1 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format A2 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format A3 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format B4 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format C0 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
|  | Format C2 | 15kHz | 23 | 0 | 400 | AWGN and TDL |
|  |  | 30kHz | 46 | 0 | 400 | AWGN and TDL |
| FR2 30 GHz | Format A1 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format A2 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format A3 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format B4 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format C0 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |
|  | Format C2 | 60kHz | 69 | 0 | 4000 | AWGN and TDL |
|  |  | 120kHz | 69 | 0 | 4000 | AWGN and TDL |

Further parameters and details:

Propagation condition details:

FR1 PRACH: AWGN, TDL-C 300ns, 100 Hz

FR2 PRACH: AWGN, TDL-A 30ns, 300Hz

System Bandwidth:

FR1: 10MHz/15kHz; 40MHz/30kHz; 40MHz/1,25kHz

FR2: 100MHz/60kHz; 100MHz/120kHz

Target false alarm probability = 0.05%

The performance measure is:

SNR [dB] with Missed detection probability 1%.

Simulation results PRACH FR1 1T2R

Simulation results for PRACH FR1 1T2R and with the parameters defined are shown in Table 2.

Table 2: Simulation results for PRACH FR1 1T2R

| PRACH FR1 1T2R |  | Propagation condition | Frequency offset (Hz) | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal | Impairment |
| Format 0 | 1.25kHz | AWGN | 0 | -16,35 | -13,85 |
|  |  | TDLC300-100 | 400 | -9,03 | -6,53 |
| Format A1 | 15kHz | AWGN | 0 | -11,74 | -9,24 |
|  |  | TDLC300-100 | 400 | -4,39 | -1,89 |
|  | 30kHz | AWGN | 0 | -11,57 | -9,07 |
|  |  | TDLC300-100 | 400 | -4,92 | -2,42 |
| Format A2 | 15kHz | AWGN | 0 | -14,71 | -12,21 |
|  |  | TDLC300-100 | 400 | -7,26 | -4,76 |
|  | 30kHz | AWGN | 0 | -14,55 | -12,05 |
|  |  | TDLC300-100 | 400 | -7,82 | -5,32 |
| Format A3 | 15kHz | AWGN | 0 | -16,06 | -13,56 |
|  |  | TDLC300-100 | 400 | -8,62 | -6,12 |
|  | 30kHz | AWGN | 0 | -15,88 | -13,38 |
|  |  | TDLC300-100 | 400 | -9,35 | -6,85 |
| Format B4 | 15kHz | AWGN | 0 | -18,77 | -16,27 |
|  |  | TDLC300-100 | 400 | -11,37 | -8,87 |
|  | 30kHz | AWGN | 0 | -18,62 | -16,12 |
|  |  | TDLC300-100 | 400 | -12,00 | -9,50 |
| Format C0 | 15kHz | AWGN | 0 | -8,70 | -6,2 |
|  |  | TDLC300-100 | 400 | -1,24 | 1,26 |
|  | 30kHz | AWGN | 0 | -8,55 | -6,05 |
|  |  | TDLC300-100 | 400 | -1,85 | 0,65 |
| Format C2 | 15kHz | AWGN | 0 | -14,76 | -12,26 |
|  |  | TDLC300-100 | 400 | -7,18 | -4,68 |
|  | 30kHz | AWGN | 0 | -14,56 | -12,06 |
|  |  | TDLC300-100 | 400 | -7,84 | -5,34 |

Simulation results PRACH FR1 1T4R

Simulation results for PRACH FR1 1T4R and with the parameters defined are shown in Table 3.

Table 3: Simulation results for PRACH FR1 1T4R

| PRACH FR1 1T4R |  | Propagation condition | Frequency offset (Hz) | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal | Impairment |
| Format 0 | 1.25kHz | AWGN | 0 | -18,77 | -16,27 |
|  |  | TDLC300-100 | 400 | -14,56 | -12,06 |
| Format A1 | 15kHz | AWGN | 0 | -14,27 | -11,77 |
|  |  | TDLC300-100 | 400 | -9,49 | -6,99 |
|  | 30kHz | AWGN | 0 | -14,10 | -11,60 |
|  |  | TDLC300-100 | 400 | -9,39 | -6,89 |
| Format A2 | 15kHz | AWGN | 0 | -17,32 | -14,82 |
|  |  | TDLC300-100 | 400 | -12,30 | -9,80 |
|  | 30kHz | AWGN | 0 | -17,16 | -14,66 |
|  |  | TDLC300-100 | 400 | -12,35 | -9,85 |
| Format A3 | 15kHz | AWGN | 0 | -18,48 | -15,98 |
|  |  | TDLC300-100 | 400 | -13,62 | -11,12 |
|  | 30kHz | AWGN | 0 | -18,35 | -15,85 |
|  |  | TDLC300-100 | 400 | -13,48 | -10,98 |
| Format B4 | 15kHz | AWGN | 0 | -20,99 | -18,49 |
|  |  | TDLC300-100 | 400 | -16,09 | -13,59 |
|  | 30kHz | AWGN | 0 | -20,89 | -18,39 |
|  |  | TDLC300-100 | 400 | -16,07 | -13,57 |
| Format C0 | 15kHz | AWGN | 0 | -11,31 | -8,81 |
|  |  | TDLC300-100 | 400 | -6,35 | -3,85 |
|  | 30kHz | AWGN | 0 | -11,14 | -8,64 |
|  |  | TDLC300-100 | 400 | -6,39 | -3,89 |
| Format C2 | 15kHz | AWGN | 0 | -17,32 | -14,82 |
|  |  | TDLC300-100 | 400 | -12,31 | -9,81 |
|  | 30kHz | AWGN | 0 | -17,15 | -14,65 |
|  |  | TDLC300-100 | 400 | -12,36 | -9,86 |

Simulation results PRACH FR1 1T8R

Simulation results for PRACH FR1 1T8R and with the parameters defined are shown in Table 4.

Table 4: Simulation results for PRACH FR1 1T8R

| PRACH FR1 1T8R |  | Propagation condition | Frequency offset (Hz) | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal | Impairment |
| Format 0 | 1.25kHz | AWGN | 0 | -21,09 | -18,59 |
|  |  | TDLC300-100 | 400 | -18,69 | -16,19 |
| Format A1 | 15kHz | AWGN | 0 | -16,67 | -14,17 |
|  |  | TDLC300-100 | 400 | -13,58 | -11,08 |
|  | 30kHz | AWGN | 0 | -16,58 | -14,08 |
|  |  | TDLC300-100 | 400 | -13,08 | -10,58 |
| Format A2 | 15kHz | AWGN | 0 | -19,70 | -17,20 |
|  |  | TDLC300-100 | 400 | -16,51 | -14,01 |
|  | 30kHz | AWGN | 0 | -19,61 | -17,11 |
|  |  | TDLC300-100 | 400 | -16,04 | -13,54 |
| Format A3 | 15kHz | AWGN | 0 | -20,70 | -18,20 |
|  |  | TDLC300-100 | 400 | -17,49 | -14,99 |
|  | 30kHz | AWGN | 0 | -20,61 | -18,11 |
|  |  | TDLC300-100 | 400 | -16,99 | -14,49 |
| Format B4 | 15kHz | AWGN | 0 | -23,19 | -20,69 |
|  |  | TDLC300-100 | 400 | -19,84 | -17,34 |
|  | 30kHz | AWGN | 0 | -23,03 | -20,53 |
|  |  | TDLC300-100 | 400 | -19,53 | -17,03 |
| Format C0 | 15kHz | AWGN | 0 | -13,70 | -11,20 |
|  |  | TDLC300-100 | 400 | -10,67 | -8,17 |
|  | 30kHz | AWGN | 0 | -13,60 | -11,10 |
|  |  | TDLC300-100 | 400 | -10,08 | -7,58 |
| Format C2 | 15kHz | AWGN | 0 | -19,69 | -17,19 |
|  |  | TDLC300-100 | 400 | -16,47 | -13,97 |
|  | 30kHz | AWGN | 0 | -19,58 | -17,08 |
|  |  | TDLC300-100 | 400 | -15,98 | -13,48 |

Simulation results PRACH FR2 1T2R

Simulation results for PRACH FR2 1T2R and with the parameters defined are shown in Table 5.

Table 5: Simulation results for PRACH FR2 1T2R

| PRACH FR2 |  | Propagation condition | Frequency offset (Hz) | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal | Impairment |
| Format A1 | 60kHz | AWGN | 0 | -11,52 | -8,42 |
|  |  | TDLA30-300 | 4000 | -3,70 | -0,60 |
|  | 120kHz | AWGN | 0 | -11,43 | -8,33 |
|  |  | TDLA30-300 | 4000 | -4,15 | -1,05 |
| Format A2 | 60kHz | AWGN | 0 | -14,46 | -11,36 |
|  |  | TDLA30-300 | 4000 | -5,96 | -2,86 |
|  | 120kHz | AWGN | 0 | -14,45 | -11,35 |
|  |  | TDLA30-300 | 4000 | -7,01 | -3,91 |
| Format A3 | 60kHz | AWGN | 0 | -15,82 | -12,72 |
|  |  | TDLA30-300 | 4000 | -7,74 | -4,64 |
|  | 120kHz | AWGN | 0 | -15,78 | -12,68 |
|  |  | TDLA30-300 | 4000 | -8,54 | -5,44 |
| Format B4 | 60kHz | AWGN | 0 | -18,61 | -15,51 |
|  |  | TDLA30-300 | 4000 | -10,13 | -7,03 |
|  | 120kHz | AWGN | 0 | -18,48 | -15,38 |
|  |  | TDLA30-300 | 4000 | -11,01 | -7,91 |
| Format C0 | 60kHz | AWGN | 0 | -8,43 | -5,33 |
|  |  | TDLA30-300 | 4000 | -1,00 | 2,10 |
|  | 120kHz | AWGN | 0 | -8,43 | -5,33 |
|  |  | TDLA30-300 | 4000 | -1,13 | 1,97 |
| Format C2 | 60kHz | AWGN | 0 | -14,54 | -11,44 |
|  |  | TDLA30-300 | 4000 | -5,99 | -2,89 |
|  | 120kHz | AWGN | 0 | -14,42 | -11,32 |
|  |  | TDLA30-300 | 4000 | -7,05 | -3,95 |

Conclusion

In this contribution we have provided updates to our simulation results for NR PRACH test cases.

- References

- R4-1813757, Way forward on NR PRACH demodulation requirements, Ericsson, 3GPP TSG-RAN WG4 RAN#88bis, Chengdu, China, 8 – 12 October 2018.

- R4-1904713, WF on requirement SNR derivation procedure, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, RAN4#90b.

- R4-1904714, Excel Script for Deriving NR Performance Requirements, ZTE, RAN4#90b.