3GPP TSG-RAN WG4 Meeting #91                 	R4-1906368

Reno, Nevada, US, 13th – 17th May 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-15 HST evaluation results

Agenda item:	6.12.2.1.1

Document for:	Discussion

Introduction

In the RAN4#90bis meeting, a WF [1] was agreed that outlines a set of assumptions for optional performance requirements for high speed train (HST) scenarios in Rel-15, which are for evaluation until the RAN4#91 meeting:

- The performance requirements for HST are optional, and the applicability depends on the declaration. - Assumptions for the following HST evaluations are for NR Rel-15 HST for RAN4#91 meeting. Bases on the evaluation results, companies can share their views on the HST requirements definition.

In this contribution we will deliver all our evaluation results for both PUSCH and PRACH in HST scenarios for NR Rel-15. Separate contributions [2, 3] will discuss the implications of these evaluations and will reproduce subsets of the complete results given here.

Simulation results HST PUSCH

Setup

Following the WF by Docomo [1], we ran simulations with the following setup. The mentioning of both the transmission scheme and naming of FL (front-loaded) DMRS are debatable and are discussed in [2]

Table 1: Simulation setup Rel-15 HST PUSCH

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR1 |
| Transform precoding | Disabled | Enabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | Baseline: DMRS 1+1, the position of FL DMRS l0= 3 for both 15kHz and 30kHz SCS DMRS 1+1+1 for 30kHz SCS is welcome to provide evaluation results | Baseline: DMRS 1+1, the position of FL DMRS l0=3 for both 15kHz and 30kHz SCS DMRS 1+1+1 for 30kHz SCS is welcome to provide evaluation results |
| symbols length | 14 | 14 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type A |
| Frequency domain resource | Full applicable test bandwidth | 15kHz: 25 PRB; 30kHz: 24 PRB (middle of test BW) |
| MCS index | 2 | 2 |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 2.1GHz or 3.6GHz | 15kHz SCS: 2.1GHz 30kHz SCS: 2.1GHz or 3.6GHz |
| Propagation condition | Scenario 3 with AWGN | Scenario 3 with AWGN |
| SCS and BW | 15kHz: 10MHz 30kHz: 40MHz | 15kHz: 5MHz; 30kHz: 10MHz |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |

Table 2: Propagation condition Rel-15 HST PUSCH

| Parameter | Value |
| --- | --- |
|  | Scenario 3 |
|  | 300 m |
|  | 2 m |
|  | 300 km/h |
|  | For 15kHz SCS: - 1150Hz For 30kHz SCS: - Option 1: 2000Hz - Option 2: 1150Hz |

Table 3: Further system parameters for Rel-15 HST PUSCH

- UE speed - 300km/h - Carrier frequency (Band) - 15kHz SCS - Band n1 (2.1GHz) - 30kHz SCS - Band n77 (3.6GHz) - Maximum Doppler shift - 15kHz SCS - 1150Hz - 30kHz SCS - Option 1: 2000Hz - Option 2: 1150Hz - Channel model - AWGN (Single tap HST) - BS-Railway track distance (Dmin) and Initial distance of the train from BS (Ds/2): - Same as LTE HST scenario 3 for tunnel

Summary

Summary of the results

Table 4: Summary of Rel-15 HST PUSCH simulation results

| PUSCH HST 1T2R |  |  |  |  |  | SNR @70% TPUT (Ideal) |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 Mapping type A | 2.1GHz, 15kHz, 10MHz | MCS2 | AWGN HST3 - 1150 | DMRS 1+1 | NA |
|  |  |  |  |  | (DMRS 1+1+1) | -5,96 |
|  |  | 2,1GHz, 30kHz, 40MHz | MCS2 | AWGN HST3 - 1150 | DMRS 1+1 | -5,87 |
|  |  |  |  |  | DMRS 1+1+1 | -5,89 |
|  |  | 3,6GHz, 30kHz, 40MHz | MCS2 | AWGN HST3 - 2000 | DMRS 1+1 | NA |
|  |  |  |  |  | DMRS 1+1+1 | -5,87 |
| DFT-s-OFDM | FR1 Mapping type A | 2.1GHz, 15kHz, 5MHz (25PRB in middle of test BW) | MCS2 | AWGN HST3 - 1150 | DMRS 1+1 | NA |
|  |  |  |  |  | (DMRS 1+1+1) | -6,32 |
|  |  | 2,1GHz, 30kHz, 10MHz (24PRB in middle of test BW) | MCS2 | AWGN HST3 - 1150 | DMRS 1+1 | -6,33 |
|  |  |  |  |  | DMRS 1+1+1 | -6,37 |
|  |  | 3,6GHz, 30kHz, 10MHz (24PRB in middle of test BW) | MCS2 | AWGN HST3 - 2000 | DMRS 1+1 | NA |
|  |  |  |  |  | DMRS 1+1+1 | -6,36 |

Details

Here we present a selection of BLER vs. SNR curves for PUSCH HST (at 2,1 GHz and 1150Hz doppler) to illustrate common observations. The curves match our expectations for simulations using AWGN with HARQ.

Figure 1: BLER vs. SNR curves for PUSCH HST, 2,1 GHz

Simulation results HST PRACH

Setup

Following the WF by Docomo [1], we ran simulations with the following setup. The general system parameters are shared with PUSCH from section 2.1; the propagation conditions are not shared.

Table 5: System parameters for Rel-15 HST PRACH evaluation

- PRACH performance requirements for HST - Baseline: Restricted set type B - Restricted set type A is welcome to provide evaluation results - Burst format - Burst format 0 - Channel model and frequency offset - Reuse parameters of requirements for burst format 0 from LTE - AWGN - Baseline: AWGN with 1875 Hz frequency offset for restricted set type B - FFS: AWGN with 2000 Hz frequency offset for restricted set type B - AWGN with 1340 Hz frequency offset for restricted set type A is welcome to provide evaluation results - TDLC300-100 - Base line: TDLC300-100 with 400Hz frequency offset for restricted set type B - TDLC300-100 with 400Hz frequency offset for restricted set type A is welcome to provide evaluation results

Summary

Summary of the results

Table 6: Summary of Rel-15 HST PRACH simulation results following the system setup

| Ideal results, 1T2R: |  |  |  |
| --- | --- | --- | --- |
| AWGN | FO=1340Hz, RestrictedTypeA | FO=1875Hz, RestrictedTypeB | FO=2000Hz, RestrictedTypeB |
| SNR@1%MD | -15,53 | -13,96 | -14,14 |
| TDLC300, FO=400Hz | Doppler Freq.=100Hz, RestrictedTypeA |  |  |
| SNR@1%MD | -7,91 |  |  |
| TDLC300, FO=400Hz | Doppler Freq.=100Hz, RestrictedTypeB |  |  |
| SNR@1%MD | -7,42 |  |  |

Remark: “SNR@1%MD” is shorthand for “SNR [dB] with Missed detection probability 1 %”

Details

Here we present a selection of missed detection probability and false alarm rate vs. SNR curves for PRACH HST to illustrate common observations.

Figure 1: MD and FA vs. SNR curves for PRACH HST

Conclusion

In this contribution we have provided our simulation results for NR Rel-15 PUSCH and PRACH HST system configuration evaluations following [1].

- References

- R4-1904720, WF on high speed related requirements for NR BS demodulation in Rel.15, NTT DOCOMO, INC., China Telecom, RAN4#90b.

- R4-1906372, On high speed train related PUSCH requirements for NR BS demodulation in Rel-15, Nokia, Nokia Shanghai Bell, RAN4#91.

- R4-1906373, On high speed train related PRACH requirements for NR BS demodulation in Rel-15, Nokia, Nokia Shanghai Bell, RAN4#91.