3GPP TSG-RAN WG4 Meeting #91                 	R4-1906371

Reno, Nevada, US, 13th – 17th May 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	On NR multi-slot PUCCH demodulation requirements

Agenda item:	6.12.2.3.1

Document for:	Discussion

Introduction

Following RAN4#90b, only one major issue remains open for NR multi-slot PUCCH demodulation requirements, which is the choice of frequency hopping configuration. The corresponding way forward [1], captures this as follows:

Frequency hopping - Option 1: Inter-slot frequency hopping enabled and intra-slot frequency hopping disabled - Option 2: both intra and inter slot frequency hopping are disabled - Option 3: Inter-slot frequency hopping disabled, and intra-slot frequency hopping enabled Agreement: Option 1 and 3 for further simulation

In this contribution we provide our views and proposals on the choice of frequency hopping configuration.

Simulation results

Following the WF of multi-slot PUCCH by ZTE [1], we ran simulations with the following setup:

Table 1: Simulation setup multi-slot PUCCH

| Parameters |  | PUCCH Format 1 |
| --- | --- | --- |
| BandWidth/SCS |  | 40 MHz/ 30kHz |
| Antenna |  | 1T8R |
| Propagation condition |  | TDLC300-100; |
| Carrier frequency (GHz) |  | 4GHz; |
| (#Bits, #Symbols, #PRBs) |  | (2, 14,1) |
| initialCyclicShift |  | 0 |
| StartingSymbolIndex |  | 0 |
| Index of orthogonal sequence (time-domain-OCC) |  | 0 |
| Inter/Intra-slot frequency hopping |  | Option 1: Enabled/Disabled |
|  |  | Option 2: Disabled/Enabled |
| number of slot |  | 2 |
| Test metric | DTX to Ack probability <1% | √ |
|  | Missed Ack probability < 1% | √ |
|  | NACK2ACK < 0.1% | √ |

Which led to the following results:

Table 2: Simulation results multi-slot PUCCH

| PUCCH FR1 1T8R, TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Number of slot | Inter/intra-slot frequency hopping | Test metric | SNR (dB) Ideal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format 1 | 30kHz, 40MHz | 2 | 14 | 1 | 2 | Enabled/Disabled | ACK miss (1%) | -17,7 |
|  |  |  |  |  |  |  | NACK to ACK (0.1%) | -16,7 |
|  |  |  |  |  |  | Disabled/Enabled | ACK miss (1%) | -16,7 |
|  |  |  |  |  |  |  | NACK to ACK (0.1%) | -15,6 |

Figure 1: Detailed simulation results

In our simulations, DTX to Ack probability is always ensured to be below 1%, hence we omitted the DTX to Ack test metric; similar to how this metric is not part the “standard” PUCCH simulation collection template [2].

Discussion

During the discussion of this topic at RAN4#90b [3], two main camps for Option 1 and Option 3 emerged. With the main arguments being:

- Option 1 should be chosen to since this configuration has better performance than the other options, with more range being the main point of using multi-slot in the first place.

- Option 3 should be chosen to match the configuration of single-slot PUCCH configuration, which makes sure that the multi-slot aspect is tested and not a mixture of repetition of slots and new frequency hopping.

Our simulation results from the previous section, show about a 1dB coverage gain for Option 1 over Option 3. It can be discussed, if this constitutes a substantial coverage gain or not.

We think that both options can be used to define valid minimum requirements, each with specific strengths and weaknesses. Though, our preference still lies with Option 3, but a majority decision on this topic is acceptable.

RAN4 to consider choosing option 3. If no unanimous decision can be reached, majority decision between option 1 and option 3 is acceptable.

Conclusion

In this contribution we have provided our views on the remaining NR multi-slot PUCCH demodulation issues. We have made the following proposal:

- RAN4 to consider choosing option 3. If no unanimous decision can be reached, majority decision between option 1 and option 3 is acceptable.

- References

- R4-1904719, WF on Multi-slot PUCCH, ZTE, RAN4#90b.

- R4-1902830, Summary of ideal and impairment results for NR BS demodulation requirements, China Telecom, RAN4#90b.

- R4-1904712, Ad Hoc minutes for NR BS demodulation, Huawei, HiSilicon, RAN4#90b.