3GPP TSG-RAN WG4 Meeting #91	R4-1906387

Reno, US, 13 - 17 May, 2019

Agenda item:	8.4.1

Source:	Dish Network

Title:	Considerations on 5G V2X with NR SL

Document for:	Approval

# 1	Introduction

A WF on NR V2X scope and scenarios was agreed in RAN4#90bis [1]. This contribution discusses some aspects related to both Uu and SL bands.

# 2	Discussion

This contribution discusses three topics; NR Uu bands, NR Uu in-device co-existence with LTE and NR SL’s and lastly definition of 5G V2X configurations.

NR Uu bands:

It can be envisioned that similar to LTE V2X, and maybe even more, operators are very interested in having their licensed bands defined as Uu bands.

Dish network requests n66, n70, and n70 included as NR Uu bands

Proposal1: n66, n70, and n71 are included as NR Uu bands

In-device Co-existence:

Assuming both NR and LTE SL (sidelink) operates at 4.7GHz unlicensed ITS spectrum, and depending on further 5GAA feedback and 3GPP discussions even on licensed spectrum, In-device co-existence with NR Uu bands has to be considered.

Dish network requests n66+SL spectrum, n70+SL spectrum, and n71+SL spectrum in-device co-existence to be included in the work. Basically, the content of the work in RAN4 is to identify and specify possible MSD due to harmonics, RX mixing, IMD, and cross-band isolation. At the moment we call sidelink spectrum simply as SL spectrum as there are no official SL band numbers etc.

Proposal2: n66+SL spectrum, n70+SL spectrum, and n71+SL spectrum are included in-device co-existence work

Definition of 5G V2X configurations:

In LTE V2X there was only LTE Uu and LTE SL involved. In that sense defining V2X configurations was pretty straightforward as seen in the excerpt from TS36.101 v16.0 below.

------------------------------ Excerpt from TS36.101 V16.0------------------------------------------------------

## 5.5G	Operating bands for V2X Communication

E-UTRA V2X Communication is designed to operate in the the operating bands defined in Table 5.5G-1.

Table 5.5G-1 V2X operating band

| E-UTRA Operating Band | E-UTRA V2X Operating Band | V2X UE transmit |  |  | V2X UE receive |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| 47 | 47 | 5855 MHz |  | 5925 MHz | 5855 MHz |  | 5925 MHz | HD | PC5 |

E-UTRA V2X communication is designed to operate concurrent with E-UTRA uplink/downlink on the operating bands combinations listed in Table 5.5G-2.

Table 5.5G-2 Inter-band con-current V2X operating bands

| V2X con-current band configuration | E-UTRA or V2X Operating Band | Interface | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_3-47 | 3 | Uu | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_5-47 | 5 | Uu | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | HD |
| V2X_7-47 | 7 | Uu | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_8-47 | 8 | Uu | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_20-47 | 20 | Uu | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_28-47 | 28 | Uu | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_34-47 | 34 | Uu | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
|  | 47 | PC5 | 5855 MHz |  | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_39-47 | 39 | Uu | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_41-47 | 41 | Uu | 2496 MHz | – | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_71-47 | 71 | Uu | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |

------------------------------ Excerpt from TS36.101 V16.0------------------------------------------------------

In 5G V2X there can be 2 sidelinks, LTE and NR along with LTE Uu or NR Uu. Hence, in our understanding the at least to following permutations could exist in RAN4 from band combination perspective.

- NR Uu + NR SL + LTE SL

- LTE Uu + NR SL + LTE SL

- LTE Uu + NR SL

- NR Uu + NR SL

RAN4 should discuss how to define these permutations, if applicable, to avoid confusion and misinterpretations.

For instance, in which specification are these combinations captured? Question sounds admittedly strange, but for instance in permutations b and c the Uu is LTE and only the SL is NR. Hence defining them in 38.101-1 would be something new. On the other hand, none of the perumations are Dual connectivity so defining them in 38.101-3 should not be straightforward either. Furthermore, It should be noted that in permutations a and d the NR Uu is Standalone (NR architecture option 2).

One exemplary configuration is described below. In this example band 71 (B71 or n71) is used as Uu and band 47 (B47 or n47) is assumed as PC5.

Table X.Y.Z-w Inter-band con-current V2X operating bands

| Permutation option | V2X con-current band configuration |
| --- | --- |
| a | V2X_n71-n47_B47 |
| b | V2X_B71-n47_B47 |
| c | V2X_B71-n47 |
| d | V2X_n71-n47 |

Proposal 3: RAN4 should discuss in which specification and in which configuration format should 5G V2X configurations be specified.

# 3	Conclusions

5G V2X with NR SL bands and specification perspective. The following proposals are made.

Proposal1: n66, n70, and n71 are included as NR Uu bands

Proposal2: n66+SL spectrum, n70+SL spectrum, and n71+SL spectrum are included in-device co-existence work

Proposal 3: RAN4 should discuss in which specification and in which configuration format should 5G V2X configurations be specified.

# 3	References

[1] R4-1905098 “WF on coexistence scenarios for NR V2X”, RAN4#90bis, Qualcomm, LG Electronics