3GPP TSG-RAN WG4 Meeting #91	R4-1906409

# Reno, NV, 13 - 17 May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.141 | CR | 1211 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PUSCH demodulation requirements for eFeMTC (36.141) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Perf |  |  |  |  |  | Date: |  |  | 2019-05-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PUSCH demodulation requirements for eFeMTC (High velocity scenario and subPRB transmission) are in square brackets |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of square brackets from high speed scenario Update subPRB results according to R4-1907312 Correction of FRC name |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PUSCH demodulation requirements are not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7, 8.2.10, A.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

### 8.2.7	Performance requirements of PUSCH in multipath fading propagation conditions transmission on single antenna port for coverage enhancment

#### 8.2.7.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

The tests for CEModeA defined in Section 8.2.7 are applicable only to the base stations supporting coverage enhancement configured with CEModeA. The tests for CEModeB defined in Section 8.2.7 are applicable only to the base stations supporting coverage enhancement configured with CEModeB.

A test for a specific channel bandwidth is only applicable if the BS supports it. For a BS supporting FDD multiple channel bandwidths, only the tests for the lowest and the highest FDD channel bandwidths supported by the BS are applicable. For a BS supporting TDD multiple channel bandwidths, only the tests for the lowest and the highest TDD channel bandwidths supported by the BS are applicable.

#### 8.2.7.2	Minimum Requirement

The minimum requirement is in TS 36.104 [2] subclause 8.2.7.

#### 8.2.7.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

#### 8.2.7.4	Method of test

##### 8.2.7.4.1	Initial Conditions

Test environment:	Normal, see subclause D.2.

RF channels to be tested:	M; see subclause 4.7.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex I.3.2.

##### 8.2.7.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth, defined in Table 8.2.7.4.2-1.

Table 8.2.7.4.2-1: AWGN power level at the BS input

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 1.4 | -92.7dBm / 1.08MHz |
| 3 | -88.7dBm / 2.7MHz |
| 5 | -86.5dBm / 4.5MHz |
| 10 | -83.5dBm / 9MHz |
| 15 | -81.7dBm / 13.5MHz |
| 20 | -80.4dBm / 18MHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in Table 8.2.7.4.2-2. The index of the narrowband is set to 0. For reference channels using resource blocks less than 6, the resource blocks shall be allocated from the lowest number within the indicated narrowband.

Table 8.2.7.4.2-2: Test parameters for testing PUSCH

| Parameter | unit | CEMode A | CEMode B |
| --- | --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 | 2 |
| RV sequences |  | 0, 2, 3, 1, 0, 2, 3, 1 | FDD: 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 TDD: 0, 0, 0, 0, 0, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 1, 1, 1, 1,1 |
| Number of PUSCH repetitions |  | 8 | 256 |
| Frequency hopping |  | ON | ON |
| Frequency hopping interval | subframes | 4: FDD 5: TDD | 4: FDD 5: TDD |
| Frequency hopping offset |  | (Note 2) | (Note 2) |
| Note 1: Guard period shall be created according to TS36.211, 5.2.5 [12] Note 2: is the total number of uplink narrowbands specified in TS36.211, 5.2.4 [12] |  |  |  |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex B.

4)	Adjust the equipment so that required SNR specified in Table 8.2.7.5-1 to 8.2.7.5-2 is achieved at the BS input.

5)	For each of the reference channels in Table 8.2.7.5-1 to 8.2.7.5-2 applicable for the base station, measure the throughput, according to annex E.

#### 8.2.7.5	Test Requirement

The throughput measured according to subclause 8.2.7.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.7.5-1 for CEMode A tests and not be below the limits for the SNR levels specified in Table 8.2.7.5-2 for CEMode B tests.

Table 8.2.7.5-1 Minimum requirements for PUSCH for CEMode A

| Number of TX antennas | Number of RX antennas | Channel Bandwidth (MHz) | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | EPA 5Hz Low | A3-2 | 70% | -5.6 |
| 1 | 2 | 5 | EPA 5Hz Low | A3-2 | 70% | -6.0 |
| 1 | 2 | 10 | EPA 5Hz Low | A3-2 | 70% | -6.3 |
| 1 | 2 | 15 | EPA 5Hz Low | A3-2 | 70% | -6.3 |
| 1 | 2 | 20 | EPA 5Hz Low | A3-2 | 70% | -6.4 |
| 1 | 2 | 3 | ETU 200Hz Low | A3-2 | 70% | -6.5 |
| 1 | 2 | 5 | ETU 200Hz Low | A3-2 | 70% | -6.5 |
| 1 | 2 | 10 | ETU 200Hz Low | A3-2 | 70% | -6.5 |
| 1 | 2 | 15 | ETU 200Hz Low | A3-2 | 70% | -6.5 |
| 1 | 2 | 20 | ETU 200Hz Low | A3-2 | 70% | -6.5 |

Table 8.2.7.5-2 Minimum requirements for PUSCH for CEMode B

| Number of TX antennas | Number of RX antennas | Channel Bandwidth (MHz) | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | ETU 1Hz Low | A3-1 | 70% | -14.4 |
| 1 | 2 | 5 | ETU 1Hz Low | A3-1 | 70% | -14.6 |
| 1 | 2 | 10 | ETU 1Hz Low | A3-1 | 70% | -14.7 |
| 1 | 2 | 15 | ETU 1Hz Low | A3-1 | 70% | -14.5 |
| 1 | 2 | 20 | ETU 1Hz Low | A3-1 | 70% | -14.6 |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

### 8.2.10	Performance requirements of PUSCH in multipath fading propagation conditions transmission on single antenna port for subPRB transmission

#### 8.2.10.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

The tests for CEModeA defined in clause 8.2.10 are applicable only to the base stations supporting subPRB transmission under coverage enhancement configured with CEModeA. The tests for CEModeB defined in clause 8.2.10 are applicable only to the base stations supporting subPRB transmission under coverage enhancement configured with CEModeB.

A test for a specific channel bandwidth is only applicable if the BS supports it. For a BS supporting FDD multiple channel bandwidths, only the tests for the lowest and the highest FDD channel bandwidths supported by the BS are applicable. For a BS supporting TDD multiple channel bandwidths, only the tests for the lowest and the highest TDD channel bandwidths supported by the BS are applicable.

#### 8.2.10.2	Minimum Requirement

The minimum requirement is in TS 36.104 [2] subclause 8.2.10.

#### 8.2.10.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

#### 8.2.10.4	Method of test

##### 8.2.10.4.1	Initial Conditions

Test environment:	Normal, see subclause D.2.

RF channels to be tested:	M; see subclause 4.7.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex I.3.2.

##### 8.2.10.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth, defined in Table 8.2.10.4.2-1.

Table 8.2.10.4.2-1: AWGN power level at the BS input

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 3 | -88.7 dBm / 2.7 MHz |
| 5 | -86.5 dBm / 4.5 MHz |
| 10 | -83.5 dBm / 9 MHz |
| 15 | -81.7 dBm / 13.5 MHz |
| 20 | -80.4 dBm / 18 MHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in Table 8.2.10.4.2-2. The index of the narrowband is set to 0. For reference channels using resource blocks less than 6, the resource blocks shall be allocated from the lowest number within the indicated narrowband.

Table 8.2.10.4.2-2: Test parameters for testing PUSCH

| Parameter | unit | CEMode A | CEMode B |
| --- | --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 | 2 |
| RV sequences (Note 3) |  | 0, 2, 3, 1, 0, 2, 3, 1 | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions |  | 1 | 16 |
| Frequency hopping |  | ON | ON |
| Frequency hopping interval | subframes | 4: FDD 5: TDD | 4: FDD 5: TDD |
| Frequency hopping offset |  | (Note 2) | (Note 2) |
| Note 1: Guard period shall be created according to TS 36.211, 5.2.5 [12] Note 2: is the total number of uplink narrowbands specified in TS 36.211, 5.2.4 [12] Note 3: RV changes per repetition every (RU size x number of RUs) transmission subframes. |  |  |  |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex B.

4)	Adjust the equipment so that required SNR specified in Table 8.2.10.5-1 to 8.2.10.5-2 is achieved at the BS input.

5)	For each of the reference channels in Table 8.2.10.5-1 to 8.2.10.5-2 applicable for the base station, measure the throughput, according to annex E.

#### 8.2.10.5	Test Requirement

The throughput measured according to subclause 8.2.10.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.10.5-1 for CEMode A tests and not be below the limits for the SNR levels specified in Table 8.2.10.5-2 for CEMode B tests.

Table 8.2.10.5-1: Minimum requirements for PUSCH for CEMode A

| Number of TX antennas | Number of RX antennas | Channel Bandwidth (MHz) | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | EPA 5Hz Low | A24-1 | 70% | [0.0] |
| 1 | 2 | 5 | EPA 5Hz Low | A24-1 | 70% | [-0.2] |
| 1 | 2 | 10 | EPA 5Hz Low | A24-1 | 70% | [-0.2] |
| 1 | 2 | 15 | EPA 5Hz Low | A24-1 | 70% | [-0.4] |
| 1 | 2 | 20 | EPA 5Hz Low | A24-1 | 70% | [-0.4] |

Table 8.2.10.5-2: Minimum requirements for PUSCH for CEMode B

| Number of TX antennas | Number of RX antennas | Channel Bandwidth (MHz) | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | EPA 5Hz Low | A24-2 | 70% | [-8.7] |
| 1 | 2 | 5 | EPA 5Hz Low | A24-2 | 70% | [-8.9] |
| 1 | 2 | 10 | EPA 5Hz Low | A24-2 | 70% | [-8.9] |
| 1 | 2 | 15 | EPA 5Hz Low | A24-2 | 70% | [-8.9] |
| 1 | 2 | 20 | EPA 5Hz Low | A24-2 | 70% | [-8.9] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

# A.24	Fixed Reference Channels for PUSCH with SubPRB transmission

Table A.24-1: FRC parameters for performance requirements (subPRB transmission)

| Reference channel | A24-1 | A24-2 |
| --- | --- | --- |
| Diversity | No | No |
| Allocated resource blocks | 1 | 1 |
| Number of subcarriers | 2 out of 3 | 2 out of 3 |
| DFT-OFDM Symbols per subframe | 12 | 12 |
| Modulation | π/2 BPSK | π/2 BPSK |
| Code rate | 1/3 | 1/3 |
| MCS index | 0 | 0 |
| Payload size (bits) | 32 | 88 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96*2 | 96*2 |
| Total symbols per resource unit | 96*2 | 96*2 |
| Frequency offset | 0 | 0 |
| Number of RUs | 1 | 2 |
| Tx time (ms) | 8 | 16 |

------------------------------------------------------------- End of change ------------------------------------------------------------