3GPP TSG-RAN WG4 Meeting #91	R4-1906418

# Reno, NV, 13 - 17 May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.133 (A.3.2) OCNG for 2AoA setup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | OCNG for 2AoA setup is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add OCNG for FR2 SCS=120kHz assuming 2AoA setup - Active probe 1 transmits OCNG on even slots. - Active probe 2 transmits OCNG on odd slots. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN5 cannot configure OCNG for RRM tests with 2AoA setup. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

## A.3.2	OFDMA channel noise generator (OCNG)

### A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)

The OCNG pattern is used in a test for modelling allocations of unused resources in the channel bandwidth to virtual UEs (which are not under test). The OCNG pattern comprises PDCCH and PDSCH transmissions to the virtual UEs.

#### A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs

Table A.3.2.1.1-1: OP.1: Generic OCNG pattern for all unused REs

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. |  |  |

#### A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup

Table A.3.2.1.1-2: OP.2: Generic OCNG pattern for all unused REs for 2AoA setup

| OCNG Parameters |  |  | Control Region |  | Data Region |  |  | Control Region |  | Data Region |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Active probe number | 1 |  |  |  |  |  | 2 |  |  |
| Resource allocation |  |  |  |  |  |  |  |  |  |  |
| For Slot i, if mod(i, 2) = 0 for i from {0,…,159} | Unused REs (Note 1) | Unused REs (Note 2) | N/A | N/A |
| For Slot i, if mod(i, 2) = 1 for i from {0,…,159} | N/A | N/A | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel |  |  | PDCCH |  | PDSCH |  |  | PDCCH |  | PDSCH |
| Contents |  |  | Virtual UE IDs |  | Uncorrelated pseudo random QPSK modulated data |  |  | Virtual UE IDs |  | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme |  |  | Same as used in PDCCH RMC |  | Same as used in PDSCH RMC |  |  | Same as used in PDCCH RMC |  | Same as used in PDSCH RMC |
| Subcarrier spacing |  |  | Same as used in PDCCH RMC |  | Same as used in PDSCH RMC |  |  | Same as used in PDCCH RMC |  | Same as used in PDSCH RMC |
| Aggregation level |  |  | Same as used in PDCCH RMC |  | N/A |  |  | Same as used in PDCCH RMC |  | N/A |
| Code rate |  |  | Same as used in PDCCH RMC |  | Same as used in PDSCH RMC |  |  | Same as used in PDCCH RMC |  | Same as used in PDSCH RMC |
| Transmit Power |  |  | Same as used in PDCCH RMC |  | Same as used in PDSCH RMC |  |  | Same as used in PDCCH RMC |  | Same as used in PDSCH RMC |
| CP length |  |  | Same as used in PDCCH RMC |  | Same as used in PDSCH RMC |  |  | Same as used in PDCCH RMC |  | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------