3GPP TSG-WG RAN4 Meeting #91	R4-1906426

Reno, USA, 13 – 17 May, 2019

Source: 	Nokia,  MediaTek Inc.

Title:	TP for TR 37.716-21-11 DC_2-66_n5, DC_2A-2A-66A_n5A, DC_2-66-66_n5, DC_2A-2A-66A-66A_n5A and DC_2-66-66-66_n5

Agenda Item: 			9.4.2 [DC_R16_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# Introduction

In RAN4#90bis meeting held in Xi’an the EN-DC configurations of DC_2-66_n5 where approved in both R4-1903077 (Nokia) and R4-1904083 (MediaTek). However, the ∆TIB and MSD values presented in the two TPs did not mach why R4-1903077 was implemented with MSD in brackets. As a result, this joint TP adds the agreed MSD value.

**************************** Start of changes ********************************************

## 5.1.X	DC_2-66_n5, DC_2-2-66_n5, DC_2-66-66_n5 DC_2-2-66-66_n5, and DC_2-66-66-66_n5

### 5.1.X.1	Operating bands for DC

Table 5.1.X.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n5 | CA_2-66 | n5 | No |

### 5.1.X.2	Configuration for DC

Table 6.1.X.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-66A | n5A |
| DC_2A-2A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-2A-66A | n5A |
| DC_2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-66A-66A | n5A |
| DC_2A-2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-2A-66A-66A | n5A |
| DC_2A-66A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-66A-66A-66A | n5A |

### 5.1.X.3	Coexistence studies

Based on co-existence studies of DC_66A_n5A in TR 37.863-01-01, the 2nd and 5th IMD of 66+n5 may fall into the band n5 downlink and Pcell REFSENS excpetions can be represented by the REFSENS expcetion already specified for DC_66A-n5A.

Co-existence studies for DC_2A_n5A is also available in TR 37.863-01-01. For this 4th IMD product of 2+n5 may fall into band 66 downlink.

### 5.1.X.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are derived from CA_2-5-66 in TS 36.101.

Table 5.X.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n5, DC_2A-2A-66A_n5A, DC_2-66-66_n5, DC_2A-2A-66A-66A_n5A, DC_2-66-66-66_n5 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |

Table 5.X.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n5, DC_2A-2A-66A_n5A, DC_2-66-66_n5, DC_2A-2A-66A-66A_n5A, DC_2-66-66-66_n5 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0 |

### 5.1.X.5	Refsens requirements

Pcell REFSENS exceptions for band n5 due to 2 uplink of band 66 and band n5 can be represented by the REFSENS expcetion already specified for DC_66A-n5A in TS 38.101-3.

Scell REFSENS exceptions for band 66 due to 2 uplink of band 2 and band n5, MSD is specified in Table 5.1.X.5-1

Table 5.1.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n5A | 2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 7.2 | FDD | IMD4 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |

The MSD value is based on predefined value for LTE CA_2A-2A-5A-66A-66A / CA_2A-5A_66A.

**************************** End of changes ********************************************