3GPP TSG-RAN WG4 Meeting # 91	R4-1906437

# Reno, United States of America,  13 May, 2019 - 17 May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | <Updates to FR2 EN-DC intra-frequency RSRP accuracy test in section A.5.7.1.1> |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2019/3/5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | OTA parameters in intrafrequency RSRP accuracy tests are incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify PCI to be used in the test to avoid potential cross correlation issues Change subtests to time phases as there is an agreement to compare results between different time phases Remove subtest (time phase) 2 since testing at Io=-70dBm may already violate minimum SS_RSRP side condition for some bands/power classes regardless of Zn value and is hence not feasible Add a new table Table A.5.7.1.1.2-4: SS-RSRP Intra frequency OTA power class and band dependent SS-RSRP settings for lowest level testing phase based on minimim side conditions for measurement Add OTA settings for subtest (time phase) for highest level testing phase targetting Io approximately -50dBm at reference point. Specify that in sub test (time phase) 2 there is no AWGN added in the test equipment |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OTA parameters in intrafrequency RSRP accuracy tests remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | OTA setup 2b is currently assumed in the CR, but the CR may be modified for setup 1 by changing table A.5.7.1.1.2-4 to match the values for peak bem direction. |  |  |  |  |  |  |  |  |

#### A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

##### A.5.7.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 10.1.2.1.1and 10.1.2.1.1 for intra-frequency measurements.

##### A.5.7.1.1.2	Test parameters

In this est case all NR cells are on the same carrier frequency. Supported test configurations are shown in table A.5.7.1.1.2-1. Both absolute and relative accuracy of SS-RSRP intra frequency measurements are tested by using the parameters in Table A.5.7.1.1.2-2 and A.5.7.1.1.2-3. The E-UTRA PCell is configured as specified in section A.3.7.2.2. In all test cases, Cell 1 is the PCell, cell 2 is the PSCell and Cell 3 the target cell. The test consists of two time phases T1 and T2. The angle of arrival of cell 2 and cell 3 must not change between phase T1 and T2.

Table A.5.7.1.1.2-1: SS-RSRP Intra frequency SS-RSRP supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD LTE PCell, Cell 2&3 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | TDD LTE PCell, Cell 2&3 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

Table A.5.7.1.1.2-2: SS-RSRP Intra frequency general test parameters

| ParameterNote 5 | Unit | T1NOTE 6 |  | T2 NOTE 6 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |  |
| Physical Cell ID (PCI) |  | 550 | 0 | 550 | 0 |  |
| SSB ARFCN |  | freq1 |  | freq1 |  |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - |  |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |  |
| Dedicated CORESET Reference Channel |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |  |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 |  |
| SSB configuration |  | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 |  |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 | SMTC.1 |  |
| Time offset with Cell 2 | s | - | 3 | - | 3 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |  |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
|  | dB | 0 | -6 | NOTE 7 | NOTE 7 |  |
| Propagation conditions |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: All parameters apply for configuration 1 and 2 Note 6: Time durations T1 and T2 may be chosen by RAN5 to obtain a sufficient number of samples for statistical confidence during each time phase of the test Note 7: No additional noise is added in time phase T2 |  |  |  |  |  |  |

Table A.5.7.1.1.2-3: SS-RSRP  Intra frequency OTA related test parameters

| ParameterNote 6 | Unit | T1NOTE 7 |  |  |  | T2NOTE 7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 3 |  |  | Cell 2 | Cell 3 |
| Angle of arrival configuration |  | According to section A.3.15.2.2 |  |  |  | According to section A.3.15.2.2 |  |
| Note1 | dBm/15kHzNote4 | -92 |  |  |  | -∞NOTE 8 |  |
| Note1 | dBm/SCSNote3 | -82.97 |  |  |  | -∞NOTE 8 |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -82.97 | -88.97 |  |  | See table A.5.7.1.1.2-4 | See table A.5.7.1.1.2-4 |
| Note9 | dB | 0 | -6 |  |  | Note 8 | Note 8 |
| IoNote2,9 | dBm/95.04 MHz Note4 | -50.97 | -53.01 |  |  | SS-RSRP+28.98 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels in phase T1 and T2 have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: All parameters apply for configuration 1 and 2 Note 7: Time durations T1 and T2 may be chosen by RAN5 to obtain a sufficient number of samples for statistical confidence during each time phase of the test Note 8: No additional noise is added by the test system during time phase T2 Note 9: TDM transmissiom of cell 1 and cell 2 is performed |  |  |  |  |  |  |  |

Table A.5.7.1.1.2-4: SS-RSRP  Intra frequency OTA power class and band dependent SS-RSRP settings for time phase T2

| SS-RSRP (dBm/120kHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Band | Power class 1 | Power class 2 | Power class 3 | Power class 4 |
| n257 | -120.3+Z1 | -114.3+Z2 | -108.2+Z3 | -118.8+Z4 |
| n258 | -120.3+Z1 | -114.3+Z2 | -108.2+Z3 | -118.8+Z4 |
| n260 | -117.3+Z1 |  | -103.9+Z3 | -113.8+Z4 |
| n261 | -120.3+Z1 | -114.3+Z2 | -108.2+Z3 | -118.8+Z4 |

Editor’s Note : Z1, Z2, Z3 and Z4 will be replaced with values once final values are settled

##### A.5.7.1.1.3	Test Requirements

The SS-RSRP measurement accuracy for cell 2 and cell 3 shall fulfil the absolute accuracy requirements in sections 10.1.3.1.1 and the SS-RSRP difference between cell 2 and cell 3 shall fulfil the relative accuracy requirements in section 10.1.3.1.2 during each time phase of the test.The SS-RSRP difference between cell 2 in time phase T1 and cell 2 in time phase T2 shall fulful the relative accuracy requirements in section 10.1.3.1.2. The SS-RSRP difference between cell 3 in time phase T1 and cell 3 in time phase T2 shall fulful the relative accuracy requirements in section 10.1.3.1.2.