3GPP TSG-RAN WG4 Meeting #91		R4-1906443

Reno, United States of America, 13 - 17 May, 2019

Title: 					Requirements analysis for RRM idle mode / PCell requirements supporting enhanced high speed operation

Source: 	Ericsson

Agenda item:	7.15.2.2

Document for:	Discussion

# Introduction

In this contribution we develop an analysis framework based on free space path loss considerations, and the rate of change of RSRP at the cell edge for different sizes of cell (ISD). Based on this we examine legacy requirements for PCell, and also consider the case that legacy requirements would be extended to active SCells in release 16. We examine the suitability of detection and measurement periods for:

- PCell in RRC connected state at speeds up to 500km/h

- SCells in RRC connected state at speeds up to 350km/h and 500 km/h

- Idle mode requirements at speeds up to 500 km/h

# Discussion

## System level analysis framework

In this section we present a system model which we will use to evaluate the necessity of improvements to RRM requirements to support UE speeds up to 500 km/h. The approach is less thorough than system level simulation but more formal than the “moving time” analysis that has been previously presented in RAN4.

We start by defining UE speed vue  in metres per second, with the convention that positive velocity is velocity away from the basestation, and we define intersite distance as ISD. For an SFN deployment, ISD should be interpreted as the distance between the last radio node in one SFN area and the first radio node in the adjacent SFN area. In the analysis we assume constant speed UEs, so if a UE passes a basestation transmitter at time t=0, its subsequent distance from that basestation is simply

x(t)=vue ∙ t

For freespace propagation and penetration into a vehicle such as train, the pathloss may be modelled as

PLFS (d,f) = Lpenetration(f)+ 20 log10(d)+20*log10(f)+32.54 where d is in units of m, and f is frequency in GHz.

Lpenetration is the loss due to the UEs being inside the train carriage (frequency dependent).

The RSRP experienced by a UE at a distance d may thus be expressed as

RSRP(d,f)= PCRS – PLFS(d,f) = PCRS - Lpenetration(f)– 20 log10(d)-20*log10(f)-32.54

Where PCRS is the CRS transmission power in dBm/15Khz Strictly for an SFN network, we should sum the RSRP contribution from all SFN nodes along with their respective distances d1…dN however for simplicity in the analysis we assume the RSRP to be dominated by a single SFN node at the edge of the SFN area, since all other SFN nodes are at a distance of at least 3xISD from the UE.

We can now substitute x(t) to derive RSRP as a function of time as the UE moves away from a cell at constant speed

RSRP(t,f) = PCRS - Lpenetration(f)– 20 log10(vue t)-20*log10(f)-32.54 																		(1)

Since the rate of change of RSRP is also of interest, we differentiate with respect to time, noting that most of the terms in (1) except the 20log10(…) term are constant w.r.t time

From this we derive RSRP’(t)=-20/ln(10) ∙(1/t)																											(2)

Specifically, it can be noted that the derivative (rate of change) of RSRP is independent of frequency, although naturally the pathloss is higher on higher frequency bands, and that it has a 1/t (or equivalently 1/d due to constant speed assumption) dependency on time (and distance) from the basestation.

Without hysteresis, L3 filtering or time to trigger configuration, the handover will ideally be triggered at time t= 0.5xISD / vue since the cell radius is ISDx0.5. Hence the ideal RSRP and rate of change of RSRP at the ideal handover point is

RSRP (at ideal handover) = PCRS - Lpenetration – 20 log10( 0.5xISD)  -20*log10(f)-32.54					(3)

RSRP’(at ideal handover)=-20/ln(10).( vue /0.5xISD)															(4)

## Analysis of necessary requirements

From equation (4), it can be seen that the rate of change of RSRP (eg in dB per second) depends on the UE velocity (eg in metres per second) relative to 0.5xISD. 0.5xISD is simply the radius of the source (and target) cells. On different frequency bands, the rate of change of RSRP is the same, based on the assumption of free space path loss.

The most important parameters for RRM requirements are cell detection time, Tidentify and measurement period Tmeasure. For successful cell identification to trigger the handover, side conditions are also relevant. If we assume a simple 2 cell model, and no other noise or interference, cell detection can be successfully performed when RSRPtarget>(RSRPsource-6) dB.  Around the handover area, the source cell will have RSRPsource reducing at rate -20/ln(10).( vue /0.5xISD) and the target cell has RSRPtarget increasing at rate 20/ln(10).( vue /0.5xISD). Allowing for a safety margin (due to the fact that RSRP is not reducing/increasing linearly, and there may be other noise and interference), it seems reasonable to assume that cell detection will be successful if the rate of change of RSRP (for one cell) is less than around 1.5dB during the Tidentify period which is 50% of the theoretical value. Hence, we assume for successful identification of target before handover should be possible if the following condition applies (looking from serving cell perspective, which is falling)

0≥Tidentfy∙RSRP’(at ideal handover) ≥ -1.5dB															(Cond 1)

For already identified cells, a similar approach can be taken. Since RSRP relative accuracy is ±3dB, we follow a similar approach and it seems reasonable to assume that measurement will give a reasonably accurate indication of cell usability and ranking if the rate of change of RSRP (for one cell) is less than around 0.75dB during the Tmeasure period.

0≥Tmeasur∙RSRP’(at ideal handover) ≥ -0.75dB															(Cond 2)

The next step is to substitute RSRP’(at ideal handover) from the previous section. This gives the following conditions

Identify∙-20/ln(10).( vue /0.5xISD)≤ 1.5dB																(Cond1a)

Tmeasure∙-20/ln(10).( vue /0.5xISD)≤ 0.75dB																(Cond2a)

Conditions 1a and 2a are readily evaluated to verify the suitability of requirements for different conditions (eg different UE speed, different ISD, different UE requirements due to DRX etc.)

## Requirements for non-DRX

In table 1, we evaluate the product with Tidentify=800ms and Tmeasure=200ms for different ISD and UE speeds. Entries highlighted in red are for cases where condition 1a/1b is not met.

| Tidentify | 0.8 |  |  |  |  | Tmeasure | 0.2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISD | Speed | 350 | 500 |  |  | ISD | 350 | 500 |
| 500 |  | 2.702277 | 3.860395 |  |  | 500 | 0.675569 | 0.965099 |
| 1000 |  | 1.351138 | 1.930198 |  |  | 1000 | 0.337785 | 0.482549 |
| 1500 |  | 0.900759 | 1.286798 |  |  | 1500 | 0.22519 | 0.3217 |
| 2000 |  | 0.675569 | 0.965099 |  |  | 2000 | 0.168892 | 0.241275 |
| 2500 |  | 0.540455 | 0.772079 |  |  | 2500 | 0.135114 | 0.19302 |
| 3000 |  | 0.450379 | 0.643399 |  |  | 3000 | 0.112595 | 0.16085 |
| 3500 |  | 0.38604 | 0.551485 |  |  | 3500 | 0.09651 | 0.137871 |
| 4000 |  | 0.337785 | 0.482549 |  |  | 4000 | 0.084446 | 0.120637 |
| 4500 |  | 0.300253 | 0.428933 |  |  | 4500 | 0.075063 | 0.107233 |
| 5000 |  | 0.270228 | 0.38604 |  |  | 5000 | 0.067557 | 0.09651 |

Table 1: Requirements analysis for non DRX

From the table we can see that cell identification is likely to be the limiting factor, and:

Observation 1: Legacy non DRX requirements are sufficient for UE velocities up to 350km/h with ISD≥1000m

Observation 2: Legacy non DRX requirements are sufficient for UE velocities up to 500km/h with ISD≥1500m

Since

## Requirements for PCell in DRX

Following a similar approach for DRX requirements, we can evaluate the requirements necessary in DRX. We perform the evaluations for 80ms, 160ms, 320ms, 640ms and 1.28s. Requirements are evaluated using the following minimum requirements

| DRX cycle(s) | Tdetect(s) | Tmeasure(s) |
| --- | --- | --- |
| 0.08 | 1.2 | 0.32 |
| 0.16 | 1.6 | 0.48 |
| 0.32 | 3.2 | 0.96 |
| 0.64 | 6.4 | 1.92 |
| 1.28 | 12.8 | 3.84 |

Table 2: Requirements summary with release 14 high speed enhancements assumed

Next, we can evaluate the smallest ISD where each DRX cycle becomes feasible. The results are shown in table 3.

|  | Speed (km/h) |  |
| --- | --- | --- |
| DRX | 350 | 500 |
| 0.08 | 1500 | 2000 |
| 0.16 | 2000 | 3000 |
| 0.32 | 4000 | 5500 |
| 0.64 | 7500 | >10000 |
| 1.28 | >10000 | >10000 |

Table 3: Supported ISD for different cDRX configurations

|  | Speed (km/h) |  |
| --- | --- | --- |
| DRX | 350 | 500 |
| 0.32 | 4000 | 5500 |
| 0.64 | 7500 | >10000 |
| 1.28 | >10000 | >10000 |
| 2.56 | >10000 | >10000 |

Table 4: Supported ISD for different idle DRX configurations

Since different network deployments will use different ISD (depending on frequency band used etc.), for cDRX it means that certain DRX cycles may be suitable only in certain deployments. This statement holds for both 350km/h and 500km/h operation (for example an operator using 4km ISD can only use 0.32s DRX at 300km/h or 0.16s DRX at 500km/h). However, along with ISD, the cDRX cycle used is a network design parameter and can be set appropriately considering mobility and other considerations.

Of more significance are the idle mode requirements. Looking at table 3, only legacy 0.32s DRX is feasible for 500km/h operation (there is no option for 0.16 or 0.08 DRX cycle in idle mode), and even then, only under the condition that ISD>5500m. In practice this appears to be the most significant limitation in legacy requirements.

Based on these observations we propose

Observation 1: Legacy non DRX requirements are sufficient for UE velocities up to 350km/h with ISD≥1000m

Observation 2: Legacy non DRX requirements are sufficient for UE velocities up to 500km/h with ISD≥1500m

Proposal 1: Enhanced requirements for PCell in RRC connected DRX developed in release 14 for 350km/h may also be used for PCell requirements for speeds up to 500km/h in release 16

Proposal 2: Enhanced requirements for PCell in RRC connected DRX developed in release 14 for 350km/h may also be used for active SCell requirements for speeds up to 500km/h in release 16

Proposal 3: RAN4 considers further enhancement of idle mode requirements in release 16 to facilitate speeds up to 500km/h

Regarding proposal 3, suitable further enhanced requirements for idle mode are

Proposal 4: Further enhanced idle mode requirements are specified as

| DRX cycle length [s] | Tdetect,EUTRAN_Intra [s] (number of DRX cycles) | Tmeasure,EUTRAN_Intra [s] (number of DRX cycles) | Tevaluate,E-UTRAN_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.24 (7) | 0.32(1) | 0.64(2) |
| 0.64 | 4.48 (7) | 0.64 (1) | 1.28(2) |
| 1.28 | 8.96(7) | 1.28 (1) | 2.56 (2) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

The values allow the same ISD (deployment) to be supported with 500km/h UE speed as would be feasible for 350km/h based on the release 14 enhancement. While meeting the proposed Tdetect requirement may imply more UE power consumption for measurements (such as performing multiple intra cell search attempts per idle DRX cycle), this is still better for overall UE power consumption than having to re-acquire a suitable serving cell which involves initial search, MIB and SIB decoding etc.

# Conclusions

In this contribution we developed an analysis framework based on free space path loss considerations, and the rate of change of RSRP at the cell edge for different sizes of cell (ISD). Based on this we examined legacy requirements for PCell, and also considered the case that legacy requirements would be extended to active SCells in release 16. We examined the suitability of detection and measurement periods for:

- PCell in RRC connected state at speeds up to 500km/h

- SCells in RRC connected state at speeds up to 350km/h and 500 km/h

- Idle mode requirements at speeds up to 500 km/h

Based on this we propose

Proposal 1: Enhanced requirements for PCell in RRC connected DRX developed in release 14 for 350km/h may also be used for PCell requirements for speeds up to 500km/h in release 16

Proposal 2: Enhanced requirements for PCell in RRC connected DRX developed in release 14 for 350km/h may also be used for active SCell requirements for speeds up to 500km/h in release 16

Proposal 3: RAN4 considers further enhancement of idle mode requirements in release 16 to facilitate speeds up to 500km/h

Proposal 4: Further enhanced idle mode requirements are specified as

| DRX cycle length [s] | Tdetect,EUTRAN_Intra [s] (number of DRX cycles) | Tmeasure,EUTRAN_Intra [s] (number of DRX cycles) | Tevaluate,E-UTRAN_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.24 (7) | 0.32(1) | 0.64(2) |
| 0.64 | 4.48 (7) | 0.64 (1) | 1.28(2) |
| 1.28 | 8.96(7) | 1.28 (1) | 2.56 (2) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |