3GPP TSG-RAN WG4 Meeting #91                    	   R4-1906464

Reno, US, May 13th – 17th, 2019

Agenda item:            7.15.4.2

Source:	MediaTek Inc.

Title:	LTE PDSCH simulation for HST enhancement

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation results of LTE PDSCH for HST enhancement scenario.

# 2	Discussion

Based on [1], and the HST-SFN channel model and test case in [2], we provide the simulation results of PDSCH. The existed HST-SFN test case is used with the modification of maximum Doppler frequency and the vehicle velocity. Table A.1 is the HST-SFN channel model parameter.

Figure 2.1 is PDSCH performance with Doppler frequency equal to 972Hz, and Figure 2.2 is PDSCH performance with Doppler frequency equal to 1100Hz.

Figure 2.1 PDSCH performance for SFN-HST bi-directional with Doppler = 972Hz

Figure 2.2 PDSCH performance for SFN-HST bi-directional with Doppler = 1100Hz

Observation 1: For Doppler frequency 1100Hz, almost all MCS (MCS12~MCS17) have the error floor.

Observation 2: For 70% Tput SNR, only MCS12 and MCS13 can operate below 10dB SNR.

For HST scenario, considering fast moving property, it is hard to keep high SNR all the time. For more typical case, we may consider SNR lower than 10dB. For Doppler frequency up to 1100Hz, the UE performance is not so robust (error floor) since it approaches the limitation of CRS-based data transmission. Furthermore, such high Doppler will introduce even more challenge to UL and it is not a stable system design.

Proposal 1: Use Doppler 972Hz for enhanced HST PDSCH demodulation requirement.

Proposal 2: Use MCS12 or MCS 13 for enhanced HST PDSCH demodulation requirement.

# 3	Conclusions

In this contribution, we provide our performance evaluation of LTE PDSCH for HST enhancement scenario. We have the following proposal:

Proposal 1: Use Doppler 972Hz for enhanced HST PDSCH demodulation requirement.

Proposal 2: Use MCS12 or MCS 13 for enhanced HST PDSCH demodulation requirement.

# 4	Reference

[1] R4-1904832, “Way forward on UE demodulation enhancement for HST”, Qualcomm, NT DOCOMO.

[2] 3GPP TS 36.101 V16.1.0.

Appendix

Table A.1 HST-SFN channel model parameter

| Parameter | Value |
| --- | --- |
|  | 1000 m |
|  | 50m |
|  | 390/442 km/h |
|  | 972/1100 Hz |