3GPP TSG-RAN4 Meeting #91	R4-1906534

# Reno, US, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Finalization of inter-RAT RSTD requriements for TDD E-UTRA (section 9.4.4.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For inter-RAT RSTD measurement, UE may need to use autonomous gaps to read MIB and/or ECGI. The corresponding requirements for ACK/NACK number are mostly TBD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the TBD to exact values for TDD E-UTRA case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inter-RAT RSTD measurement requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 9.4.4.2	SA: NR − E-UTRAN TDD RSTD measurements

##### 9.4.4.2.1	Introduction

The requirements are applicable for NR−E-UTRAN TDD RSTD measurements requested via LPP [22, 26].

The requirements in section 9.4.4.1 apply when:

-	the UE is provided with the LTE timing information via LPP [27], including both nr-LTE-SFN-Offset and nr-LTE-fineTiming-Offset, or

-	the UE is not provided with nr-LTE-SFN-Offset or nr-LTE-fineTiming-Offset, or

-	the UE is provided with nr-LTE-SFN-Offset but not with nr-LTE-fineTiming-Offset.

When the UE is not aware of the SFN of at least one LTE cell in the OTDOA assistance data, the UE may be using autonomous gaps in FR1 to acquire SFN of the OTDOA assistance data reference cell prior to requesting measurement gaps for performing the requested E-UTRA RSTD measurements before the  time period starts while meeting all the requirements in Section 9.4.4.2.2, provided that the OTDOA assistance data is provided to allow sufficient time for the UE to acquire the SFN before the  starts.When the UE is not aware of and cannot derive the subframe timing difference between the NR serving cell and the OTDOA assistance data reference cell, the UE may need to request measurement gaps to perform cell detection for the OTDOA assistance data reference cell prior to requesting measurement gaps for performing the requested E-UTRA RSTD measurements before the  time period starts while meeting all the requirements in Section 9.4.4.2.2, provided that the OTDOA assistance data is provided to allow sufficient time for the UE to detect the cell before the  starts.

##### 9.4.4.2.2	Requirements

When the physical layer cell identities of neighbour cells together with the OTDOA assistance data are provided, the UE shall be able to detect and measure inter-RAT -UTRAN TDD RSTD, specified in TS 38.215 [4], for at least n=16 cells, including the reference cell, within  ms as given below:

,

where

is the total time for detecting and measuring at least n cells,

is the largest value of the cell-specific positioning subframe configuration period, defined in TS 36.211 [23], among the measured n cells including the reference cell,

is the number of PRS positioning occasions as defined in Table 9.4.4.2.2-1, where a PRS positioning occasion is as defined in clause 9.4.4.1.2,

CSSFinterRAT=CSSFwithin_gap_i is the scaling factor determined by the gap sharing scheme for the RSTD measurements on the carrier frequency i as defined in Section 9.1.5.2,

=  ms is the measurement time for a single PRS positioning occasion which includes the sampling time and the processing time, and

the n cells are distributed on up to two E-UTRAN TDD carrier frequencies.

Table 9.4.4.2.2-1: Number of PRS positioning occasions within

| Positioning subframe configuration period | Number of PRS positioning occasions |  |
| --- | --- | --- |
|  | f2 Note1 | f1 and f2 Note2 |
| 160 ms | 16 × CSSFinterRAT | 32 × CSSFinterRAT |
| >160 ms | 8 × CSSFinterRAT | 16 × CSSFinterRAT |
| NOTE 1: When inter-RAT E-UTRAN TDD RSTD measurements are performed over the reference cell and neighbour cells, which belong to the E-UTRAN TDD carrier frequency f2. NOTE 2: When inter-RAT E-UTRAN TDD RSTD measurements are performed over the reference cell and the neighbour cells, which belong to the E-UTRAN TDD carrier frequency f1 and the E-UTRAN TDD carrier frequency f2 respectively. |  |  |

The requirements in this section shall apply for all TDD special subframe configurations specified in TS 36.211 [23] and for the TDD uplink-downlink configurations as specified in Table 9.4.4.2.2-2 for UE requiring measurement gaps for these measurements. For UEs capable of performing inter-RAT RSTD measurements without measurement gaps, TDD uplink-downlink subframe configurations as specified in Table 9.4.4.2.2-3 shall apply.

Table 9.4.4.2.2-2: TDD uplink-downlink subframe configurations applicable for inter-RAT RSTD requirements

| PRS Transmission Bandwidth [RB] | Applicable TDD uplink-downlink configurations |
| --- | --- |
| 6, 15 | 3, 4 and 5 |
| 25 | 1, 2, 3, 4, 5 and 6 |
| 50, 75, 100 | 0, 1, 2, 3, 4, 5 and 6 |
| NOTE 1: Uplink-downlink configurations are specified in Table 4.2-2 in TS 36.211 [23]. |  |

Table 9.4.4.2.2-3: TDD uplink-downlink subframe configurations applicable for inter-RAT RSTD requirements without gaps

| PRS Transmission Bandwidth [RB] | Applicable TDD uplink-downlink configurations |
| --- | --- |
| 6, 15 | 1, 2, 3, 4 and 5 |
| 25, 50, 75, 100 | 0, 1, 2, 3, 4, 5 and 6 |
| NOTE: Uplink-downlink configurations are specified in Table 4.2-2 in TS 36.211 [23]. |  |

The UE physical layer shall be capable of reporting RSTD for the reference cell and all the neighbor cells i out of at least (n-1) neighbor cells within  provided:

-6 dB for all Frequency Bands for the reference cell,

-13 dB for all Frequency Bands for neighbour cell i,

and   conditions apply for all subframes of at least  PRS positioning occasions,

PRP 1,2|dBm according to TS 36.133 [15, Annex B.2.6] for a corresponding Band,

is as defined in Section 9.4.4.1.2.

The time  starts from the first subframe of the PRS positioning occasion closest in time after both the OTDOA-RequestLocationInformation message and the OTDOA assistance data in the OTDOA-ProvideAssistanceData message via LPP as specified in TS 38.305 [22], are delivered to the physical layer of the UE.

The RSTD measurement accuracy for all measured neighbor cells i shall be fulfilled according to the accuracy as specified in Section 10.2.4.

###### 9.4.4.2.2.1	RSTD Measurement Reporting Delay

This requirement assumes that that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

###### 9.4.4.2.2.2	Requirements for acquiring the timing of the E-UTRA OTDOA reference cell

When the UE is not aware of the SFN of at least one LTE cell in the OTDOA assistance data, the UE supporting per-FR gaps may make autonomous gaps in downlink reception and uplink transmission of the PCell and each of the SCells in FR1 for acquiring SFN of the reference cell in the E-UTRA OTDOA assistance data. No autonomous gaps in downlink reception or uplink transmission are allowed in any of the UE serving cells in FR2. The UE, which are only supporting per-UE gaps, may make autonomous gaps in downlink reception and uplink transmission of the PCell and each of the SCells for acquiring the SFN of the reference cell in the E-UTRA OTDOA assistance data.

When the UE is not aware of and cannot derive the subframe timing difference between the NR serving cell and the OTDOA assistance data reference cell, the UE may need to request measurement gaps while indicating eutra-FineTimingDetection according to TS 38.331 [2] for detecting the reference cell in the E-UTRA OTDOA assistance data.

When the UE is performning one or both of SFN acquisition or cell detection as specified above, the UE shall be able to determine the timing of the E-UTRA OTDOA assistance data reference cell during the time period

TRefCell,E-UTRAN = TDetect, E-UTRAN TDD + TMIB + TECGI ,

where

TDetect, E-UTRAN TDD =  TIdentify, E-UTRAN TDD  - Tmeasure, E-UTRAN TDD is according to Section 9.4.3 assuming CSSFinterRAT=1  and it is the time needed to detect the E-UTRA OTDOA assistance data reference cell when the UE needs to acquire the subframe and slot timing of the cell, provided the UE is configured with measurement gaps (TDetect, E-UTRAN TDD=0 when both nr-LTE-SFN-Offset and nr-LTE-fineTiming-Offset are provided in the E-UTRA OTDOA assistance data or the E-UTRA OTDOA assistance data reference cell is known to the UE), and

TMIB = 50 ms is the time required to acquire SFN of the E-UTRA OTDOA assistance data reference cell provided the OTDOA assistance data reference cell is decodable and at least all E-UTRA subframes #0 during TMIB are available at the UE receiver (TMIB=0 when nr-LTE-SFN-Offset is provided in the E-UTRA OTDOA assistance data) , and

TECGI = 100 ms is the time required to acquire ECGI of the E-UTRA OTDOA assistance data reference cell when cellGlobalId is included in OTDOA-ReferenceCellInfo and the UE is not aware of the ECGI of this cell (TECGI = 0 when cellGlobalId is not included in OTDOA-ReferenceCellInfo or the UE is aware of the ECGI of the E-UTRA OTDOA assistance data reference cell).

When detecting the E-UTRAN OTDOA reference cell, the requirements in this section shall be met, provided the conditions for the detectable cell are fulfilled according to Section 9.4.3.1. In addition, the MIB of an the E-UTRA OTDOA reference cell whose SFN is acquired shall be considered decodable by the UE provided the PBCH demodulation requirements are met according to TS 36.101 [25].

The requirement for acquiring the timing of the E-UTRA OTDOA reference cell within TRefCell,E-UTRAN is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [2] is used.

When TMIB>0 and UE is using autonomous gaps during TMIB, the UE shall transmit at least NACK/NACK, MIB, TDD, FR1 ACK/NACKs on PCell or each of activated SCell(s), specified in Table 9.4.4.2.2.2-1. When TECGI>0 and UE is using autonomous gaps during TECGI, the UE shall transmit at least NACK/NACK, ECGI, TDD, FR1 ACK/NACKs on PCell or each of activated SCell(s), specified in Table 9.4.4.2.2.2-2. When both TMIB>0 and TECGI>0 and UE is using autonomous gaps during TMIB+TECGI, the UE shall transmit on PCell or each of activated SCell(s) at least NACK/NACK, MIB+ECGI, FDD, FR1 ACK/NACKs specified in Table 9.4.4.2.2.2-3, provided the OTDOA reference cell bandwidth is configured in the OTDOA assistance data [22, 26]. The requirements in Tables 9.4.4.2.2.2-1, 9.4.4.2.2.2-2 and 9.4.4.2.2.2-3 apply, provided that:

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	scheduling restriction due to intra-frequency measurement is not considered.

Table 9.4.4.2.2.2-1: Minimum number of ACK/NACKs transmitted by the UE during TMIB

| NACK/NACK, MIB, TDD, FR1 | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| 15 | FDD | 15 kHz |
| 39 | FDD | 30 kHz |
| 85 | FDD | 60 kHz |
| [0] | TDD Note 1 | 15 kHz |
| [4] | TDD Note 1 | 30 kHz |
| [12] | TDD Note 1 | 60 kHz |
| [48] | TDD Note 2 | 60 kHz |
| [106] | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

Table 9.4.4.2.2.2-2: Number of ACK/NACKs transmitted by the UE during TECGI

| NACK/NACK, ECGI, TDD, FR1 | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| [66] | FDD | 15 kHz |
| [145] | FDD | 30 kHz |
| [298] | FDD | 60 kHz |
| [30] | TDD Note 1 | 15 kHz |
| [65] | TDD Note 1 | 30 kHz |
| [135] | TDD Note 1 | 60 kHz |
| [180] | TDD Note 2 | 60 kHz |
| [370] | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

Table 9.4.4.2.2.2-3: Number of ACK/NACKs transmitted by the UE during TMIB+TECGI

| NACK/NACK, MIB+ECGI, TDD, FR1 | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| TBD[81] | FDD | 15 kHz |
| TBD[184] | FDD | 30 kHz |
| TBD[383] | FDD | 60 kHz |
| TBD[30] | TDD Note 1 | 15 kHz |
| TBD[69] | TDD Note 1 | 30 kHz |
| TBD[147] | TDD Note 1 | 60 kHz |
| TBD[228] | TDD Note 2 | 60 kHz |
| TBD[476] | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

<End of Change 1>