3GPP TSG-RAN4 Meeting #91	R4-1906572

# Reno, US, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introduction of interruption requirements for inter-frequency SFTD measurement (section 8.2.4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Interruption requirements for inter-freqeuncy SFTD should be added for interruption based measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Interruption requirements for inter-freqeucny SFTD are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No interruption requirements for interruption based SFTD measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 8.2.4.1	Introduction

This section contains the requirements related to the interruptions on PCell, PSCell and activated SCell if configured, when up to TBD SCells are configured, deconfigured, activated or deactivated.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command. How to specify this is FFS.

The requirements shall apply for NR-NR DC with a NR PCell, PSCell or SCell.

This section also contains the requirements related to the interruptions on other active serving cells in the same frequency range wherein the UE is performing BWP switching.

For a UE which does not support per-FR measurement gaps, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For UE which support per-FR gaps, interruptions to PCell, PSCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

This section also contains the requirements related to the interrutpoion on the PCell and activated SCell if configured, when UE is requested to perform SFTD measurement on inter-frequency cells.

<End of Change 1>

<Start of Change 2>

##### 8.2.4.2.6	Interruption due to SFTD measurement on inter-freqeuncy cell

This section contains the requirements related to the interruptions on PCell and activated SCells if configured, when performing inter-frequency SFTD measurements on NR cells without measurement gaps.

When the inter-frequency SFTD measurement is configured for CA or NR-DC band combinations that are supported by the UE, the UE is allowed an interruption of up to 10 subframes on PCell and activated SCells if configured during Tmeasure_SFTD1 as specified in 9.3.8 due to intra-band SFTD measurements on NR cells. This interruption is for both uplink and downlink of PCell and activated SCells if configured.

When the inter-frequency SFTD measurement is configured for CA or NR-DC band combinations that are supported by the UE, the UE is allowed an interruption on PCell and activated SCells if configured during Tmeasure_SFTD1 as specified in 9.3.8 due to inter-band SFTD measurements on NR cells with probability of missed ACK/NACK up to as specified in Table 8.2.4.2.6-1. Each interruption shall not exceed 1 subframe.

Table 8.2.4.2.6-1: Requirements on probability of ACK/NACKs

| SMTC periodicity |  | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Probability of missed A/N | With RSRP report | 8.4% | 6.3% | 8.4% | 6.3% | 5.3% | 4.7% |
|  | Without RSRP report | 11.4% | 8.6% | 7.8% | 6.8% | 6.3% | 6.0% |

When the inter-frequency SFTD measurement is not configured for CA or NR-DC band combinations that are supported by the UE, the UE is allowed an interruption on PCell and activated SCells if configured during Tmeasure_SFTD1 as specified in 9.3.8 as TBD.

<End of Change 2>