3GPP TSG-RAN WG4 Meeting#91                	R4-1906588

Reno, USA, May 13 – 17, 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Demodulation performance for NPRACH FDD preamble format 2

Agenda item:	5.3.7

Document for:	Agreement

Introduction

Preliminary missed detection requirements for NPRACH FDD preamble format 2 were agreed at RAN4 #90 in [1].

At RAN4 #90-Bis the sourcing companies contributed proposed performance figures for NPRACH FDD preamble format 2 in [2]. Discussion was carried out with other companies to check their performance figures until RAN4 #91, which was captured in the RRM Chairman report:

“Nokia reported the status of the offline discussion on FDD/TDD requirements. The further discussion is needed for the next meeting.”

This contribution lists proposals for performance requirements from the sourcing companies based on simulation results and reuses the methodology applied for Rel-13 to specify final BS demodulation performance requirements for NPRACH FDD preamble format 2.

Simulation results

Simulations were carried out for NPRACH FDD preamble format 2 as specified in 3GPP TS 36.211. The results including impairments are shown in Table 1. Ideal simulation results are shown in Table 2.

Table 1: Simulation results with impairments for missed detection for NPRACH FDD preamble format 2

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | FDD preamble format 2 |
| 1 | 2 | 8 | AWGN | 0 | -0.6 |
|  |  |  | EPA1 Low | 200 Hz | 9.4 |
|  |  | 32 | AWGN | 0 | -4.7 |
|  |  |  | EPA1 Low | 200 Hz | 3.0 |

Table 2: Simulation results for ideal performance for missed detection for NPRACH FDD preamble format 2

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | FDD preamble format 2 |
| 1 | 2 | 8 | AWGN | 0 | -2.6 |
|  |  |  | EPA1 Low | 200 Hz | 7.4 |
|  |  | 32 | AWGN | 0 | -6.7 |
|  |  |  | EPA1 Low | 200 Hz | 1.0 |

Updated performance summary and final performance requirements

The simulation results reported in section 2 are added to the performance summary, version 3 in [3] being an update of version 2 in [4].

For deriving final performance requirements, the methodology applied for Rel-13 being described in [5] is reused:

- Add the STD from ideal results to companies’ averaged values with impairments as the final NB-IoT BS demodulation performance requirements.

  - The STD should be 2dB if larger than 2dB

Reusing this methodology, the averages for simulation results with impairments, the standard deviations for ideal simulation results and the resulting margins added to the averages are updated in [3] to determine the final SNR requirements for FDD preamble format 2. The summary is given below in Table 3.

Table 3: Determination of final performance requirements for NPRACH FDD preamble format 2

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | FDD preamble format 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | AVE [dB] results w/ impairments | STD [dB] ideal results | SNR [dB] final req. (36.104) | SNR [dB] prelim. req. (36.104) |
| 1 | 2 | 8 | AWGN | 0 | -2.8 | 1.8 | -1.0 | [-3,9] |
|  |  |  | EPA1 Low | 200 Hz | 7.4 | 1.6 | 9.0 | [6,4] |
|  |  | 32 | AWGN | 0 | -7.1 | 1.9 | -5.1* | [-8,2] |
|  |  |  | EPA1 Low | 200 Hz | 0.2 | 2.6 -> 2.0 | 2.2 | [-0,3] |

* impact due to rounding operation

According to the methodology, the standard deviation for ideal performance for EPA1 Low, 32 repetitions is truncated to 2.0 dB.

Agree the final performance requirements for FDD preamble format 2 in Table 3 based on the updated performance summary in [3].

SNR definition for NPRACH

Moreover, the SNR definition in TS 36.104 states in subclause 8.1, General, the following:

Extract of TS 36.104, subclause 8.1:

Performance requirements for the BS are specified for the fixed reference channels defined in Annex A and the propagation conditions in Annex B. The requirements only apply to those FRCs that are supported by the base station.

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for a BS supporting carrier aggregation are defined in terms of single carrier requirements. For FDD operation the requirements in clause 8 shall be met with the transmitter(s) on.

NOTE:	In normal operating conditions the BS in FDD operation is configured to transmit and receive at the same time. The transmitter may be off for some of the tests as specifed in 36.141 [4].

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the subframe on a single antenna port.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe.

For enhanced performance requirements type A and type B, the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy of one intra-cell UE in the subframe on a single antenna port.

is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single antenna port. The respective energy of each inter-cell interferer relative to  is defined by its associated DIP value.

End of extract

As NPRACH transmission is not aligned to subframes, e.g. symbol groups have a duration of 1.6 ms for NPRACH FDD preamble format 1 and 1.4 ms for NPRACH FDD preamble format 0, the SNR definition needs to be adjusted for the NPRACH.

It is proposed to change the highlighted part of the extract above to:

S	is the total signal energy in the subframe, or, in case of NPRACH, in the random access symbol group, on a 	single antenna port.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe, or, in case of NPRACH, of a random access symbol group.

Furthermore, the SNR requirement is derived based on noise energy in the transmission bandwidth, which according to the definition in subclause 3.1, Definitions, is:

Extract of TS 36.104, subclause 3.1:

Transmission bandwidth: RF Bandwidth of an instantaneous transmission from a UE or BS, measured in resource block units.

End of extract

This means, since the instantaneous transmission is used for the definition, any hopping gain of the signal in the 45 kHz hopping range, or the lower subcarrier spacing for NPRACH FDD preamble 2 will not impact on the SNR definition.

The current SNR definition does not cover the NPRACH case and thus should be adjusted.

Agree the changes to SNR definition in TS 36.104 as depicted in section 4.

Conclusion

This contribution has listed our proposals for performance requirements based on simulation results included in the updated performance summary [3] for NPRACH FDD preamble format 2 and has updated final performance requirements following the methodology applied for Rel-13 [5] to specify final BS demodulation performance requirements for NPRACH. Following proposals are made:

- Agree the final performance requirements for FDD preamble format 2 in Table 3 based on the updated performance summary in [3].

Agree the changes to SNR definition in TS 36.104 as depicted in section 4.

The companion CRs are submitted in [6] and [7].

References

- R4-1902061, CR 36.104-4834 Addition of demodulation performance for new NPRACH formats, source: Huawei, HiSilicon

- R4-1904101, Demodulation performance for NPRACH FDD preamble format 2, source: Nokia, Nokia Shanghai Bell

- R4-1906589, Updated Summary of simulation results for Rel-15 FeNB-IoT NPRACH demodulation requirements, source: Nokia, Nokia Shanghai Bell

- R4-1901054, Summary of simulation results for Rel-15 FeNB-IoT NPRACH demodulation requirement, source: Huawei, HiSilicon

- R4-1610559, Way forward on NB-IoT BS demodulation performance, source: Huawei, Nokia, Ericsson, ZTE, Samsung

- R4-1906590, CR 36.104-4867 Corrections to demodulation performance for NPRACH FDD preamble format 2 (Rel-15), source: Nokia, Nokia Shanghai Bell

- R4-1906593, CR 36.141-1224 Corrections to demodulation performance for NPRACH FDD preamble format 2 (Rel-15), source: Nokia, Nokia Shanghai Bell