3GPP TSG-RAN WG4 Meeting #91	R4-1906660

Reno, Nevada, USA, 13th – 17th May, 2019

Agenda item:		8.7.2

Source:	MediaTek Inc.

Title:	Discussion on UE dynamic adaptation to the maximum number of MIMO layers

Document for:		Discussion

# 1	Introduction

In the agreed WID for UE power saving WI [1], RAN4 is in charge of evaluating the required switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers:

- Specify power saving techniques with UE adaptation with focus in RRC_CONNECTED mode [RAN1, RAN4] - Specify the power saving techniques with power saving signal/channel - Specify the PDCCH-based power saving signal/channel triggering UE adaptation in RRC_CONNECTED - Note: this objective shall not duplicate DRX operation and impact to DRX is studied at RAN2 - Note: Any change of PDCCH channel coding and payload interleaver is not in the scope - Specify the procedure of cross-slot scheduling power saving techniques - Note: The procedure is in addition to Rel-15 cross-slot scheduling procedure - Evaluate the required switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers [RAN4] - Note: Switching on/off the RF is part of the evaluation Note: - These objectives are RAN1/RAN4 focus and do not consider RAN2 impact. - The objectives are subject to further update in RAN#84. The update will be based on recommendations from the completion of RAN2 study and remaining RAN1 recommendations based on the conclusion of RAN1 study.

In this paper, the switch delay in the switching cell is discussed in Section 2, and the interruption time to the other active cells is discussed in Section 3. Finally, summary are provided in Section 4.

# 2	Required Switch Delay for UE Dynamic Adaptation to the Maximum Number of MIMO Layers

For NR Rel-15, the maximum number of MIMO layers is indicated via RRC signaling on a per-cell parameter ‘maxMIMO-Layers’ under PDSCH-ServingCellConfig [2]. The per-cell upper limit allows UE to prepare limited number of possible antenna configurations for a serving cell/carrier. For the case of CA, the number of possible antenna configurations is combinatorial if the limit is not specified for each carrier. When there requires any change in the cell-specific limit, RRC re-configuration also provide sufficient time for UE to recalculate new antenna configurations.

Observation 1: Introduction of Rel-15 per-cell parameter on maximum number of MIMO layers reduces the possible antenna configurations to be calculated for a serving cell. When there is change in this per-cell upper limit, RRC reconfiguration can provide sufficient time for UE to recalculate new antenna configurations.

Proposal 1: For Rel-16, additional dynamic adaptation to the maximum number of MIMO layers should comply with Rel-15 per-cell limit configured via RRC signalling.

In Rel-15, implicit DL MIMO layer adaptation via DCI indication on the DMRS antenna port setting is supported. The indication applies to the scheduled PDSCH, and there is no switch delay for UE to adapt antenna setting whenever same slot scheduling is applied. The consequence is that UE always keep active at least the same RX antenna number as the cell-specific maximum number of MIMO layers.

Observation 2: For Rel-15 implicit DCI indication on PDSCH MIMO layers, the absence of switch delay forces UE to always activate maximal RX antenna number in order not to abandon any PDSCH with the maximum number of MIMO layers.

For Rel-16 UE power saving, dynamic adaptation on physical layer parameters related to UE power consumption is targeted. When gNodeB has no data for a UE, the UE is allowed/indicated to apply a setting for power saving. When there is data intended for the UE, gNodeB can indicate UE back to a setting for spectral efficiency. To avoid the long switch delay in Observation1 and the power saving obstacle in Observation 2, a new MIMO layer switch mechanism with proper switch delay is necessary. In Table 1, three MIMO Layers adaptation mechanisms are compared. To maximize the power saving gain at the UE side, the switch delay of this new mechanism need to be larger than zero but much shorter 10ms.

Table 1: Comparison between MIMO Layers adaptation mechanisms

|  | Delay | interruption | valid time | Power saving gain |
| --- | --- | --- | --- | --- |
| RRC-based MIMO layer switch | 10ms | 10ms | Till next RRC adaptation | y |
| NEW MIMO layer switch Note | TBD | TBD | Till next adaptation | y |
| DCI -based MIMO layer switch | N.A | N.A | The PDSCH scheduled by this DCI | n |
| Note: New maximum number of MIMO layers should not be larger than the per-cell parameter maxMIMO-Layers under PDSCH-ServingCellConfig. |  |  |  |  |

The switch delay for BWP switching can be referenced for the discussion on the adaption to the maximum number of MIMO layers [3]. In Figure 1, there illustrated the time components to be considered, and in Table 2, there summarize our suggested requirement for each of the components.

Figure 1: Reference time components for a physical layer setting change

Table 2: Requirements for the time components in adapting the maximum number of MIMO layers

| Time component | Suggested requirement | Comment |
| --- | --- | --- |
| Control information monitoring (from PDCCH symbol reception to DCI parsing) | min(500 us, slot duration) | For 15k SCS, total reception and processing time can be finished within 6 to 7 symbols [3]. When slot duration is short, UE should at least finish control information monitoring within a slot duration. |
| RF/BB parameter calculating and loading | 20 us | Following proposal 1, there is no need to re-calculate the parameters. Up to parameter selection and loading, 20 us will be sufficient. |
| Applying the new parameters | 250 us | If there is no carrier frequency, bandwidth or SCS change involved in adapting the maximum number of MIMO layers, the required time can be half to the proposal in [3]. |

By the above analysis, the following proposal is suggested:

Proposal 2: For Rel-16 dynamic adaptation to maximum number of MIMO layers in an active serving cell, the following switch delay is supported: For DCI-triggered switch, when UE receive the request at slot n, UE shall be ready to receive PDSCH with new settings at slot n+ TMIMOswitchDelay.

| μ (SCS = 15 kHz⋅2μ) | NR Slot length (ms) | MIMO switch delay TMIMOswitchDelay (slots)* |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 4 |
| *Rel-15 cell-specific maximum number of MIMO layers is not changed by the dynamic adaptation |  |  |

# 3	Required Interruption Time for UE Dynamic Adaptation to the Maximum Number of MIMO Layers

Interruption time is applied to the active serving cells other than the switching cell. Whether there is impact to the other serving cells depends on UE implementation in RX antenna switching as well as UE intelligence in deciding RX antenna switching.

Actually, allowing UE to adapt its number of Rx antennas is not new to RAN4. It was firstly discussed in Rel-13 4Rx WI in LTE. The test cases were designed to accommodate the fact that UE is able to fall back to 2Rx if it does not expect continuous DL data to be decoded. At that time, no interruption requirements were introduced. Therefore, we think that we can follow this logic here in Rel-16 power saving WI. Consequently, the following observation and proposal are suggested:

Observation 3: It is possible to realize zero interruption time to non-switching CC(s) since there is no change in RX antenna setting for the non-switching CC(s).

Proposal 4: For Rel-16 dynamic adaptation to maximum number of MIMO layers, zero interruption time is supported.

# 4	Summary

In this contribution, the switch delay and interruption time for dynamic adaptation to the maximum number of MIMO layers are discussed. In particular, we have the following observations:

Observation 1: Introduction of Rel-15 per-cell parameter on maximum number of MIMO layers reduces the possible antenna configurations to be calculated for a serving cell. When there is change in this per-cell upper limit, RRC reconfiguration can provide sufficient time for UE to recalculate new antenna configurations.

Observation 2: For Rel-15 implicit DCI indication on PDSCH MIMO layers, the absence of switch delay forces UE to always activate maximal RX antenna number in order not to abandon any PDSCH with the maximum number of MIMO layers.

Observation 3: It is possible to realize zero interruption time to non-switching CC(s) since there is no change in RX antenna setting for the non-switching CC(s).

And we propose

Proposal 1: For Rel-16, additional dynamic adaptation to the maximum number of MIMO layers should comply with Rel-15 per-cell limit configured via RRC signalling.

Proposal 2: For Rel-16 dynamic adaptation to maximum number of MIMO layers in an active serving cell, the following switch delay is supported: For DCI-triggered switch, when UE receive the request at slot n, UE shall be ready to receive PDSCH with new settings at slot n+ TMIMOswitchDelay.

| μ (SCS = 15 kHz⋅2μ) | NR Slot length (ms) | MIMO switch delay TMIMOswitchDelay (slots)* |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 4 |
| *Rel-15 cell-specific maximum number of MIMO layers is not changed by the dynamic adaptation |  |  |

Proposal 3: For Rel-16 dynamic adaptation to maximum number of MIMO layers, zero interruption time is supported.

# Reference

- RP-190727, New WID: UE Power Saving in NR, March, 2019

- TS38.331, Radio Resource Control (RRC) Protocol Specification, April, 2019

- R4-1810021, BWP Switching Delay, Aug., 2019.