3GPP TSG-RAN WG4 Meeting #91	R4-1906664

# Reno, US, May 13 - 17 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | <CR#> | rev | <Rev#> | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR38.817-02 on MU budget table for FR2 Tx OFF power (12.3.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN4#90bis meeting, the FR2 TX OFF and transient test method agreed, the MU budget table captured in the TR with MU value with square bracket also agreed to use same value for TT. This draft CR is to enter updated MU value and remove square bracket, also update TT value and remove square bracket. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test equipment MU value updated and square bracket removed. This change updated calculated total system MU value and square braket removed. Also updated TT value and square braket removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MU and TT value for FR2 TX OFF measurement remain as tentative with square braket. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGE, changes entered by this document is highlighted with yellow marker>

### 12.3.6	Transmitter OFF power and transmitter transient period

#### 12.3.6.1	FR1

The FR1 Transmitter OFF power and transmitter transient period is a co-location requirement and is described in sub-clause 12.8.2.

#### 12.3.6.2	FR2

##### 12.3.6.2.1	General

The MU assessment was carried out using a CATR chamber only. However other chamber types are not precluded if suitable MU assessment is done.

##### 12.3.6.2.2	MU assessment

###### 12.6.3.2.2.1	CATR

A CATR MU budget was assessed in order to determine acceptable MU for the EIRP accuracy measurement in FR2. The CATR test setup and calibration and measurement procedures for FR2 are expected to be similar to those of FR1, although the test chamber dimensions and associated MU values will scale due to the shorter wavelengths and larger relative array apertures. However, it is noted that in order to achieve the test instrument uncertainties that were assumed, calibration of the spectrum analyzer may be needed.

Table 12.3.6.2.2.1-1: Compact antenna test range uncertainty assessment for EIRP measurements for transmitter OFF power and transmitter transient period.

| UID | Uncertainty Source | Uncertainty value | Uncertainty value | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) | Standard uncertainty ui (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (Note 1) |  | 24.25 <f< 29.5 GHz | 37 <f< 40 GHz |  |  |  | 24.25 <f< 29.5 GHz | 37 <f< 40 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| E2-1 | Misalignment DUT & pointing error | 0.3 | 0.3 | Exp. normal | 2 | 1 | 0.15 | 0.15 |
| Note 2 | RF power measurement equipment | 1.25 | 1.45 | Gaussian | 1 | 1 | 1.25 | 1.45 |
| E2-3 | Standing wave between DUT and test range antenna | 0.03 | 0.03 | U-shaped | 1.414 | 1 | 0.02 | 0.02 |
| E2-4 | RF leakage, test range antenna cable connector terminated. | 0.01 | 0.01 | Normal | 1 | 1 | 0.01 | 0.01 |
| E2-5 | QZ ripple with DUT | 0.4 | 0.4 | Normal | 1 | 1 | 0.4 | 0.4 |
| E2-16 | Frequency flatness | 0.25 | 0.25 | Normal | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| E2-6 | Network Analyzer | 0.2 | 0.2 | Normal | 1 | 1 | 0.2 | 0.2 |
| E2-7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.72 | 0.72 | U-shaped | 1.414 | 1 | 0.51 | 0.51 |
| E2-8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | 1.732 | 1 | 0.1 | 0.1 |
| E2-4 | RF leakage, test range antenna cable connector terminated. | 0.01 | 0.01 | Normal | 1 | 1 | 0.01 | 0.01 |
| E2-9 | Influence of the calibration antenna feed cable | 0 | 0 | U-shaped | 1.414 | 1 | 0 | 0 |
| E2-10 | SGH Calibration uncertainty | 0.5 | 0.5 | Rectangular | 1.732 | 1 | 0.3 | 0.3 |
| E2-11 | Misalignment positioning system | 0.2 | 0.2 | Exp. normal | 2 | 1 | 0.1 | 0.1 |
| E2-1 | Misalignment SGH and pointing error | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| E2-12 | Rotary joints | 0 | 0 | U-shaped | 1.414 | 1 | 0 | 0 |
| E2-3 | Standing wave between SGH and test range antenna | 0.03 | 0.03 | U-shaped | 1.414 | 1 | 0.02 | 0.02 |
| E2-5 | QZ ripple with SGH | 0.07 | 0.07 | Normal | 1 | 1 | 0.07 | 0.07 |
| E2-15 | Switching uncertainty | 0.1 | 0.1 | Rectangular | 1.732 | 1 | 0.06 | 0.06 |
| Combined standard uncertainty (1σ) (dB)(dB) |  |  |  |  |  |  | 1.49 | 1.66 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB)(dB) |  |  |  |  |  |  | 2.91 | 3.25 |
| Note 1: UID are referenced to TR 37.843 [9]. Note 2: RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2 is given as 1.25 dB for the frequency range 24.25<f<29.5 GHz, 1.45 dB for the frequency range 37<f<40 GHz. |  |  |  |  |  |  |  |  |

###### 12.3.6.2.2.2	Summary

Based on the above evaluation and root sum square combining of the dB values for the MU, the MU was decided to be 2.9 dB for the frequency range 24.25<f<29.5GHz and 3.3 dB for the frequency range 37<f<40GHz.

##### 12.3.6.2.3	TT value

The TT value was agreed to be the same as the MU (2.9 dB for the frequency range 24.25<f<29.5GHz and 3.3 dB for the frequency range 37<f<40GHz).

<END OF CHANGE>