3GPP TSG-RAN4#91                                                                                     		 R4-1906710

Reno, USA, 13th– 17th May, 2019

Agenda Item:	6.12.1.2.1

Source: 	Ericsson

Title: 	Simulation results for NR UE PDSCH demodulation 2nd priority TDD patterns

Document for:	Discussion

# Introduction

In this contribution we provide the simulation results for alignment purposes for TDD FR1, based on the agreed simulation assumption from last meeting as listed in the following tables.

Table 1 List of simulation cases for FR1 TDD

| Case Number | CHBW/ SCS | MIMO | PDSCH type and MCS | Number of layer | Number of PDSCH PRBs | Channel Model | TDD Configuration | Max number of HARQ process |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 106 | TDL-B 100ns, 400 Hz | 4 | [10] |
| 17 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 106 | TDL-B 100ns, 400 Hz | 5 | [8] |

# Simulation results

Figure 1 Simulation results for FR1 TDD tests

Table 1 Results for FR1 TDD tests

| SNR (dB) | Test case: 16 2Tx2Rx (bps) | Test case: 16 2Tx4Rx (bps) | Test case: 17 2Tx2Rx (bps) | Test case: 17 2Tx4Rx (bps) |
| --- | --- | --- | --- | --- |
| -14 | 0 | 0 | 0 | 0 |
| -13 | 0 | 23436,2 | 0 | 10066 |
| -12 | 0 | 734052,7 | 0 | 715226,9 |
| -10 | 4917,8 | 2445587 | 1306,3 | 1906708 |
| -9 | 176501,5 | 3296053 | 182648,7 | 2401404 |
| -8 | 1157518 | 4159735 | 1109877 | 3079287 |
| -7 | 2620552 | 4936203 | 1994767 | 3526957 |
| -6 | 3403706 | 5752705 | 2483162 | 4111633 |
| -5 | 4181096 | 7844751 | 3050933 | 5580122 |
| -4 | 4943887 | 9207970 | 3542863 | 6733645 |
| -3 | 5746174 | 9389466 | 4060457 | 6888938 |
| -2 | 7218813 | 9393693 | 5190082 | 6896392 |
| -1 | 8650573 | 9393693 | 6221583 | 6896392 |
| 0 | 9285809 | 9393693 | 6767301 | 6896392 |
| 1 | 9388698 | 9393693 | 6881178 | 6896392 |
| 2 | 9392694 | 9393693 | 6896392 | 6896392 |

# Conclusions

In this contribution, we provide results based on the agreements made in previous meeting. We propose to include our results for further alignment to adjust the 2nd priority FR1 TDD patterns for PDSCH UE demodulation tests requirements.

# References

[1] R4-19xxxxx, “Simulation assumptions for NR PDSCH demodulation performance requirements”, RAN4#90bis, Intel Corporation