3GPP TSG-RAN WG4 Meeting #91	R4-1906748

Reno, Nevada, US, 13 May – 17 May 2019

Source:	Ericsson, Telstra

Title:	TP for TR 37.716-21-11: MSD values for 7A-28A_n5A and 7C-28A_n5A

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 2 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-21-11 v0.5.0 to add the missing MSD values for 7A-28A_n5A and 7C-28A_n5A

# 2. Text Proposal

# ---Start of changes---

## 5.1.47	DC_7A-28A_n5A
DC_7C-28A_n5A

### 5.1.47.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-28_n5 | CA_7-28 | n5 | DC_7_n5 |

### 5.1.47.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-28A_n5A DC_7C-28A_n5A | DC_7A_n5A DC_7C_n5A DC_28A_n5A | CA_7A-28A CA_7C-28A | n5 |

#### 5.1.47.3	Co-existence Studies

Co-existence analysis from DC_7_n5 shows that there is IMD3 and IMD5 impact from DC_7_n5 UL to Band 28 DL.

Co-existence analysis from DC_28_n5 shows that there is IMD5 impact from DC_28_n5 UL to Band 7 DL.

### 5.1.47.4		∆TIB and ∆RIB values

For DC_7-28_n5, the TIB,c and RIB,c values are reused from the LTE combination CA_5-7-28, and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n5 | 7 | 0.3 |
|  | 28 | 0.5 |
|  | n5 | 0.5 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n5 | 7 | 0 |
|  | 28 | 0 |
|  | n5 | 0 |

### 5.1.47.5	REFSENS requirements

Co-existence analysis from shows that there is IMD3 and IMD5 impact from DC_7_n5 UL to Band 28 DL, as well as IMD5 impact from DC_28_n5 UL to Band 7 DL. Therefore, MSD values are needed in 38.101-3.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2500 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | 28 | 747 | 5 | 25 | 802 | 18.5 | FDD | IMD3 |
|  | n5 | 849 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 9.4 | FDD | IMD5 |
|  | 28 | 730 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n5 | 840 | 5 | 25 | 874 | N/A | FDD | N/A |

---End of changes---

# Reference

[1]	RP-190379, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei