3GPP TSG-RAN WG4 Meeting #91	R4-1906749

Reno, Nevada, US, 13 May – 17 May 2019

Source:	Ericsson, Swisscom

Title:	TP to TR 37.716.21-21: Addition of CA configuration for DC_7A_n3A-n78A

Agenda item:	9.8.2

Document for:	Approval

# Background

This contribution provides text proposal on the EN-DC configuration DC_7A_n3A-n78A as defined in Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) RP-190151 [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_7_n3-n78

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n3-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7_n3-n78 | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  |  |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.x.3	Co-existence studies

Co-existence studies of Band 7 + Band n3 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

### 6.x.4	∆TIB and ∆RIB values

For DC_7_n3-n78, the TIB,c and RIB values are same as for DC_3-7_n78 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n3-n78 | 7 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n3-n78 | 7 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 6.x.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_7_n3-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n3A-n78A | 7 | 2560 | 5 | 25 | 2680 | 5 | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | 5 | FDD | N/A |
|  | n78 | 3390 | 10 | 52 | 3390 | 10 | TDD | 16.1 |
|  | 7 | 2565 | 5 | 25 | 2685 | 5 | FDD | N/A |
|  | n78 | 3310 | 10 | 52 | 3310 | 10 | TDD | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 5 | FDD | 15.6 |
|  | 7 | 2565 | 5 | 25 | 2685 | 5 | FDD | N/A |
|  | n78 | 3480 | 10 | 52 | 3480 | 10 | TDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 5 | FDD | 9.2 |

##### ---End of changes---

# Reference

[1]	RP-190151, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics

[2]	R4-1903007, “TR 37.716-21-21 v0.4.0 update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16”, LG Electronics