3GPP TSG-RAN WG4 Meeting #91	R4-1906754

# Reno, Nevada, US, 13 May – 17 May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to simplify Table 5.5A.2-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R16_Intra |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Table 5.5A.2-2 is becoming uncontrollably big. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Note 3, 4 and 5 that summarize all that was previously written in each and every cell. This reduce the current size of Table 5.5A.2-2 from 40 pages down to 5 pages. This CR is written towards Draft_38101-2_REL16_postRAN4_90. This CR is planned to be implemented in the big CR for NR Intra-band FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Table 5.5A.2-2 will be possible to read and to maintain. And since the size of Table 5.5A.2-2 is about to increase considerably in the near time, this kind of improvement is necessary. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

Table 5.5A.2-2: NR CA configurations and bandwidth combination sets for intra-band non-contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |
| CA_n260(A-D) | - | n260 | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260D |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-D) | - | CA_n260(2A) |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260D |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2D) | - | n260 | CA_n260(2D) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(2D) |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2D) | - | CA_n260(2A) |  | CA_n260(2D) |  |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(2D) |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-D-O) | - | n260 | CA_n260D |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260O |  | CA_n260D |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-D-O) | - | CA_n260(2A) |  | CA_n260D |  | CA_n260O |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260O |  | CA_n260D |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-D-2O) | - | n260 | CA_n260D |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260D |  | n260 |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-D-2O) | - | CA_n260(2A) |  | CA_n260D |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260D |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-G) | - | CA_n260(2A) |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260G |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2G) | - | n260 | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(2G) |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2G) | - | CA_n260(2A) |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(2G) |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2G-O) | - | CA_n260(2A) |  | CA_n260(2G) |  |  |  | CA_n260O |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260O |  | CA_n260(2A) |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2G-2O) | - | CA_n260(2A) |  | CA_n260(2G) |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260(2A) |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n260(2G) |  |  |  | CA_n260(2O) |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |
| CA_n260(3A-2G) | - | CA_n260(3A) |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(2G) |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-G) | - | CA_n260(4A) |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  | 1800 | 0 |
|  |  | CA_n260G |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-2G) | - | CA_n260(4A) |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  | 2000 | 0 |
|  |  | CA_n260(2G) |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-G) | - | n260 | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260G |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3A-G) | - | CA_n260(3A) |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260G |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2H) | - | n260 | CA_n260(2H) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(2H) |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-H) | - | CA_n260(2A) |  | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  | 1100 | 0 |
|  |  | CA_n260H |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2H) | - | CA_n260(2A) |  | CA_n260(2H) |  |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260(2H) |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-H) | - | n260 | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n260H |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-O) | - | n260 | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260O |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-O) | - | CA_n260(2A) |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260O |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2O) | - | n260 | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(2O) |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2O) | - | CA_n260(2A) |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2O-2P) | - | CA_n260(2A) |  | CA_n260(2O) |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  | 1800 | 0 |
|  |  | CA_n260(2P) |  |  |  |  |  | CA_n260(2A) |  | CA_n260(2O) |  |  |  |  |  |  |  |  |
|  |  | CA_n260(2O) |  |  |  | CA_n260(2P) |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |
| CA_n260(2A-2O-2Q) | - | CA_n260(2A) |  | CA_n260(2O) |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  | 2000 | 0 |
|  |  | CA_n260(2Q) |  |  |  |  |  |  |  | CA_n260(2A) |  | CA_n260(2O) |  |  |  |  |  |  |
|  |  | CA_n260(2O) |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |
| CA_n260(2A-3O) | - | CA_n260(2A) |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3A-2O) | - | CA_n260(3A) |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-O) | - | CA_n260(4A) |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  | 1800 | 0 |
|  |  | CA_n260O |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-2O) | - | CA_n260(4A) |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  | 2000 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-4O) | - | CA_n260(4A) |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  | 2400 | 0 |
|  |  | CA_n260(4O) |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |
| CA_n260(4A-2Q) | - | CA_n260(4A) |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  | 2400 | 0 |
|  |  | CA_n260(2Q) |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |
| CA_n260(3A-3O) | - | CA_n260(3A) |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  | 1800 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-G-O) | - | n260 | CA_n260G |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260G |  | CA_n260O |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-G-O) | - | CA_n260(2A) |  | CA_n260G |  | CA_n260O |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260G |  | CA_n260O |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2G-O) | - | n260 | CA_n260(2G) |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(2G) |  |  |  | CA_n260O |  | n260 |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-3O) | - | n260 | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3A-O) | - | CA_n260(3A) |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260O |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3A-O-P) | CA_n260O, CA_n260P | CA_n260(3A) |  |  | CA_n260O |  | CA_n260P |  |  |  |  |  |  |  |  |  | 1700 | 0 |
|  |  | CA_n260P |  |  | CA_n260(3A) |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n260O |  | CA_n260P |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-4O) | - | n260 | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(4O) |  |  |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |
| CA_n260(2A-4O) | - | CA_n260(2A) |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(4O) |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |
| CA_n260(A-P) | - | n260 | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n260P |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-P-Q) | CA_n260P, CA_n260Q | n260 | CA_n260P |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  | 1100 | 0 |
|  |  | CA_n260Q |  |  |  | n260 | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n260P |  |  | CA_n260Q |  |  |  | n260 |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-P) | - | CA_n260(2A) |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  | 1100 | 0 |
|  |  | CA_n260P |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2P) | - | n260 | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(2P) |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2P) | - | CA_n260(2A) |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260(2P) |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-4P) | - | CA_n260(2A) |  | CA_n260(4P) |  |  |  |  |  |  |  |  |  |  |  |  | 2000 | 0 |
|  |  | CA_n260(4P) |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |
| CA_n260(6A-2O) | - | CA_n260(6A) |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  | 24502 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(6A-3O) | - | CA_n260(6A) |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  | 26002 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |
| CA_n260(6A-2P) | - | CA_n260(6A) |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  | 26502 | 0 |
|  |  | CA_n260(2P) |  |  |  |  |  | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |
| CA_n260(8A-2O) | - | CA_n260(8A) |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  | 25502 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260(8A) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-2G) | - | CA_n260D |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(2G) |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2D-O) | - | CA_n260(2D) |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260O |  | CA_n260(2D) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-2O) | - | CA_n260D |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-I) | CA_n260I | n260 | CA_n260I |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260I |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-G) | CA_n260D CA_n260G | CA_n260D |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260G |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-H) | CA_n260D CA_n260H | CA_n260D |  | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n260H |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-I) | CA_n260D CA_n260I | CA_n260D |  | CA_n260I |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260I |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-O) | CA_n260D CA_n260O | CA_n260D |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260O |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-P) | CA_n260D CA_n260P | CA_n260D |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n260P |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-Q) | CA_n260D CA_n260Q | CA_n260D |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260Q |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(E-O) | CA_n260E CA_n260O | CA_n260O |  | CA_n260E |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260E |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(E-P) | CA_n260E CA_n260P | CA_n260E |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  | 8001 | 0 |
|  |  | CA_n260P |  |  | CA_n260E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(E-Q) | CA_n260E CA_n260Q | CA_n260E |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260Q |  |  |  | CA_n260E |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-I) | CA_n260G CA_n260I | CA_n260G |  | CA_n260I |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260I |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-O) | - | CA_n260G |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
|  |  | CA_n260O |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-2O) | - | CA_n260G |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2G-O) | - | CA_n260(2G) |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260O |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2G-2O) | - | CA_n260(2G) |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-3O) | - | CA_n260G |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3G-O) | - | CA_n260(3G) |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260O |  | CA_n260(3G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2G-3O) | - | CA_n260(2G) |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-4O) | - | CA_n260G |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(4O) |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |
| CA_n260(2G-4O) | - | CA_n260(2G) |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(4O) |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |
| CA_n260(4G-O) | - | CA_n260(4G) |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260O |  | CA_n260(4G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(H-O) | - | CA_n260H |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  | 500 | 0 |
|  |  | CA_n260O |  | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2H-O) | - | CA_n260(2H) |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260O |  | CA_n260(2H) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2O-2P) | - | CA_n260(2P) |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(2O) |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-D) | - | n261 | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n261D |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-2D) | - | n261 | CA_n261(2D) |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261(2D) |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-D-H) | - | n261 | CA_n261D |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261H |  |  | CA_n261D |  | n261 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-D-2O) | - | n261 | CA_n261D |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
|  |  | CA_n261(2O) |  |  |  | n261 | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n261D |  | CA_n261(2O) |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-G) | - | n261 | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n261G |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-G-H) | - | n261 | CA_n261G |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
|  |  | CA_n261H |  |  | CA_n261G |  | n261 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-G-I) | - | n261 | CA_n261G |  | n261 |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
|  |  | n261 |  |  |  | CA_n261G |  | n261 |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-2G-2O) | - | n261 | CA_n261(2G) |  |  |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  | 6501 | 0 |
|  |  | CA_n261(2O) |  |  |  | n261 | CA_n261(2G) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n261(2G) |  |  |  | CA_n261(2O) |  |  |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(A-3G-O) | - | n261 | CA_n261(3G) |  |  |  |  |  | CA_n261O |  |  |  |  |  |  |  | 6501 | 0 |
|  |  | CA_n261O |  | n261 | CA_n261(3G) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n261(3G) |  |  |  |  |  | CA_n261O |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(A-4G) | - | n261 | CA_n261(4G) |  |  |  |  |  |  |  |  |  |  |  |  |  | 6501 | 0 |
|  |  | CA_n261(4G) |  |  |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(A-H) | - | n261 | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n261H |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-2H) | - | n261 | CA_n261(2H) |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
|  |  | CA_n261(2H) |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-H-I) | - | n261 | CA_n261H |  |  | CA_n261I |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261I |  |  |  | CA_n261H |  |  | n261 |  |  |  |  |  |  |  |  |  |
| CA_n261(A-I) | - | n261 | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n261I |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-2I) | - | n261 | CA_n261(2I) |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261(2I) |  |  |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(A-4O) | - | n261 | CA_n261(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 6501 | 0 |
|  |  | CA_n261(4O) |  |  |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(A-7O) | - | n261 | CA_n261(7O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 5001 | 0 |
|  |  | CA_n261(7O) |  |  |  |  |  |  |  |  |  |  |  |  |  | n261 |  |  |
| CA_n261(A-2P) | - | n261 | CA_n261(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261(2P) |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-2Q) | - | n261 | CA_n261(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261(2Q) |  |  |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(D-G) | CA_n261D CA_n261G | CA_n261D |  | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n261G |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(D-H) | CA_n261D CA_n261H | CA_n261D |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n261H |  |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(D-I) | CA_n261D CA_n261I | CA_n261D |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n261I |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(D-O) | CA_n261D CA_n261O | CA_n261D |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n261O |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(D-P) | CA_n261D CA_n261P | CA_n261D |  | CA_n261P |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n261P |  |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(D-Q) | CA_n261D CA_n261Q | CA_n261D |  | CA_n261Q |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n261Q |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(E-O) | CA_n261E CA_n261O | CA_n261E |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n261O |  | CA_n261E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(E-P) | CA_n261E CA_n261P | CA_n261E |  |  | CA_n261P |  |  |  |  |  |  |  |  |  |  |  | 900 | 0 |
|  |  | CA_n261P |  |  | CA_n261E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(E-Q) | CA_n261E CA_n261Q | CA_n261E |  |  | CA_n261Q |  |  |  |  |  |  |  |  |  |  |  | 8001 | 0 |
|  |  | CA_n261Q |  |  |  | CA_n261E |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(G-I) | - | CA_n261G |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n261I |  |  |  | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(G-H) | - | CA_n261G |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  | 500 | 0 |
|  |  | CA_n261H |  |  | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(H-I) | - | CA_n261H |  |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n261I |  |  |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The maximum bandwidth of band n261 is 850MHz and a non-contiguous gap is in between NR component carriers NOTE 2: The maximum bandwidth of band n260 is 3000MHz and a non-contiguous gap is in between NR component carriers NOTE 3: Channel bandwidth per operating band defined in Table 5.3.5-1 NOTE 4: Configurations for intra-band contiguous CA defined in Table 5.5A.1-1 NOTE 5: Configurations for intra-band non-contiguous CA defined in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

---End of changes---