3GPP TSG RAN WG4 Meeting #91	R4-1906762

Reno, USA, May 13-17, 2019

Agenda Item:	6.10.7.5.1

Source: 	Ericsson

Title:	Analysis of RRC based BWP switching delay

Document for:	Discussion

- Introduction

In the last RAN4 meeting the RRC based BWP switching delay requirements were agreed and incorporated in section 8.6.3 of TS 38.133. However, the BWP switching delay (TBWPswitchDelayRRC) is still within 5 ms and 8 ms as follows:

TBWPswitchDelayRRC=[5 ~ 8]ms is the time used by the UE to perform BWP switch.

RAN4 needs to finalize the core requirements in this meeting (RAN4#91). In his paper we further analyse the RRC based BWP delay requirements.

# Analysis of BWP Switching Delay Requirements

The total RRC based BWP switching delay including the RRC procedure delay is defined as the duration comprising:

TRRCprocessingDelay + TBWPswitchDelayRRC

Where TRRCprocessingDelay is the RRC procedure delay in slots defined in clause 12 in TS 38.331.

RAN4 has to finalize the delay, TBWPswitchDelayRRC, which has been agreed between: 5 ms and 8 ms.

It has been pointed out that the BWP switching is identical to handover from the UE processing perspective. Based on this argument the TBWPswitchDelayRRC has been proposed to be 8 ms. However in our opinion there are differences between the BWP switching procedure and the handover which require relatively less processing in the UE when performing BWP switching. The BWP switching is done within the existing cell. Secondly, in handover case all cell related parameters are changed. Therefore, BWP switching could be performed in a relatively shorter time. We therefore propose that lower limit of 5 ms is used as the time to perform the RRC based BWP switching (TBWPswitchDelayRRC).

- Proposal # 1: Time to perform the RRC based BWP switching (TBWPswitchDelayRRC) = 5 ms.

- Summary

In this paper we have further analysed the BWP switching delay requirements for RRC based BWP switching. The following are the main proposals:

- Proposal # 1: Time to perform the RRC based BWP switching (TBWPswitchDelayRRC) = 5 ms.

A CR to correct the requirements based on the above proposal is provided in [1].

# References

- R4-1906763, RRC based BWP switching delay requirement (8.6.3), Ericsson