3GPP TSG RAN WG4 Meeting #90bis	　　　　　　　　　　　　　　　　R4-1906784

Reno, Nevada, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for correction of out-of-band blocking exceptions for SUL in TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone, BT plc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Out-of-band blocking exceptions approved in R4-1810235 are against the previous RAN4 agreement on the band definition of n77 and n78 filters. The justification given for the exceptions was that n77 and n78 bands may share a common RF filter and that the second order intermodulation product generated from the SUL carrier and a blocker (which would be sufficiently attenuated by an n78 filter as shown in R4-1906785) may fall onto the n78 DL carrier. RAN4 agreement on the n77 and n78 band definition (as captured in R4-1706893) states, “A UE supporting band X (n78) is not required to support band Z (n77)”. Therefore, RAN4 requirements for the n78 band cannot be derived based on the assumption that UE is also required to support the n77 band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete out-of-band blocking exceptions in subcause 7.6C.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification not aligned with the RAN4 agreement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-------------------- Start of changes #1 --------------------

## 7.6C	Blocking characteristics for SUL

### 7.6C.1	General

### 7.6C.2	In-band blocking for SUL

For SUL operation, the in-band blocking requirement for downlink bands specified in subclause 7.6.2 shall be met.

### 7.6C.3	Out-of-band blocking for SUL

For SUL operation, the out-of-band blocking requirement for downlink bands specified in subclause 7.6.3 shall be met.

| ¦fInterferer- fSUL- fDL¦≤(BWSUL+ BWDL)/2BWSUL BWDL |  |  |
| --- | --- | --- |

### 7.6C.4	Narrow band blocking for SUL

Narrow band blocking is not specified for SUL band combination.

-------------------- End of changes #1 --------------------