3GPP TSG-RAN WG4 Meeting #91                                  R4-1906801

# Reno, USA, 13th – 17th May 2019

Agenda item:  10.3.1

Source:       China Unicom

Title:          TP for TR 37.815: Conclusion of SI for FDD+TDD EN-DC HPUE

Document for:	 Approval

- Introduction

The study item of high power UE (power class 2) for EN-DC (1 LTE FDD band + 1 NR TDD band) is coming to an end according to the initial work plan. The main target of the SI is to study the feasible solution to comply with the SAR requirement for two cases in the SI scope, i.e. case 1 for 23dBm LTE + 23dBm NR and case 2 for 23dBm LTE + 26dBm NR.

Several solutions are studied and discussed in RAN4, which include:

- Reporting combined maximum duty cycle capability between LTE and NR

- Solving SAR issue by only reporting one overall maximum duty cycle capability

- UE centric solution can be further studied, including combining with network centric solution

- TDM patterns based on NR TDD configuration requested by operators as well as existing LTE TDD configurations

After further study, the group prefers to define a maximum duty cycle capability in Rel-16. With this capability, network can help to guarantee the UE not exceeding the SAR limits at the UE maximum output power. The specific signalling design can be further considered during the WI stage, which can also take the study outcome of TDD + TDD EN-DC WI into consideration. Meanwhile, the UE based solution should also be considered as the reported maximum duty cycle capability cannot always be guaranteed by the network scheduling. Furthermore, the proximity sensing mechanism does not rely on the maximum duty cycle capability which can even be used for PC3 UE. A combined UE based, and network based solution to better improve the UL performance is suggested in the SI study, which should be reflected in the specification afterwards once the work item is established.

Required changes in TS 38.101-3 were also analysed during the SI study, the identified requirements shall be specified for specific FDD+TDD EN-DC band combinations in the following WI.

In the SI study, only one band combination, i.e. DC_3_n78 is proposed as an example combination to facilitate the study. Once the SI is finished, a WI will be proposed to standardize the SAR solution as well the specifying requirements for proposed band combinations. It is expected that more band combinations will be proposed in the WI stage, thus it is proposed to treat the FDD+TDD EN-DC HPUE as a basket WI.

- References

[1] R4-1902452, WF on PC2 FDD TDD HPUE, CHTTL

[2] R4-1905004, WF for EN-DC FDD-TDD High Power UE, CHTTL, China Unicom, SGS Wireless

[3] R4-1903145, TP for TR37.815: Required changes in TS 38.101-3, ZTE

- Text Proposal

<Start of TP>

- 9	Summary of the SI

The main target of the SI is to study the feasible solution to comply with the SAR requirement for two cases in the SI scope, i.e. case 1 for 23dBm LTE + 23dBm NR and case 2 for 23dBm LTE + 26dBm NR.

Several solutions are studied and discussed in RAN4, which include:

- Reporting combined maximum duty cycle capability between LTE and NR

- Solving SAR issue by only reporting one overall maximum duty cycle capability

- UE centric solution can be further studied, including combining with network centric solution

- TDM patterns based on NR TDD configuration requested by operators as well as existing LTE TDD configurations

After further study, the group prefers to define a maximum duty cycle capability in Rel-16. With this capability, network can help to guarantee the UE not exceeding the SAR limits at the UE maximum output power. The specific signaling design can be further considered during the WI stage, which can also take the study outcome of TDD + TDD EN-DC WI into consideration. Meanwhile, the UE based solution should also be considered as the reported maximum duty cycle capability cannot always be guaranteed by the network scheduling. Furthermore, the proximity sensing mechanism does not rely on the maximum duty cycle capability which can even be used for PC3 UE. A combined UE based and network-based solution to better improve the UL performance is suggested in the SI study, which should be reflected in the specification afterwards once the work item is established.

Required changes in TS 38.101-3 were also analyzed during the SI study, the identified requirements shall be specified for specific FDD+TDD EN-DC band combinations in the following WI.

In the SI study, only one band combination, i.e. DC_3_n78 is proposed as an example combination to facilitate the study. Once the SI is finished, a WI will be proposed to standardize the SAR solution as well specifying requirements for proposed band combinations. It is expected that more band combinations will be proposed in the WI stage, thus it is proposed to treat the FDD+TDD EN-DC HPUE as a basket WI.

<End of TP>