3GPP TSG-RAN4  #91	R4-1906838

# Reno, US, 13-17 May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.810 | CR | - | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 38.810 on the feasibility of simultaneous transmission approach |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Antenna gain difference and SINR calculator for simultaneous transmission apporach in 2AoA RRM testing is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The antenna gain difference and SINR calclulater table for simultaneous transmission apporach in 2AoA RRM are proposed to include in TR38.810: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The simultaneous transmission apporach for 2AoA case are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.4.2, Annex H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

=================================Start of change=================================

##### 6.2.1.4.2	Radiated requirements Reference point and Testing directions

For RRM baseline measurement setup based on DFF and IFF, the reference point is located at the centre of the QZ. From the UE perspective the reference point is the input of UE antenna array.

Calibration of power level and relative power level test parameters and RRM metrics (at the required AoAs) is required and shall be provided by the test system.

Appropriate timing and relative timing test parameters and RRM metrics (at the required AoAs) shall be provided by the test system within declared uncertainties.

The following Modes for useful signals (S) and noise signals (N) configuration have been identified and can be supported by the NR RRM Test Methods:

-	Mode 1 (SNR emulation): Test system transmits useful signals (S) and noise signals (N) to emulate target SNR condition.

-	Mode 2 (noise-free transmission): Test system transmits only useful signals (S).

The test cases in core specification TS 38.133 [17] will be specified at the reference point, according to the following principles:

-	Mode 1

-	Specify absolute Noc level at the Reference point per angle of arrival (AoA)

-	Noc level may have different value according to operating band and UE power class

-	Specify SNR at the Reference point per angle of arrival (AoA)

-	SNR is a test-specific value

-	The angle(s) of arrival (AoA(s)) will be specified in each test case.

Figure 6.2.1.4.2-1: DL SNR reference point for RRM testing methodology

The following Scenarios for RRM requirements have been identified and can be supported by the NR RRM test methods:

-	Scenario 1: RRM requirement with single Angle of Arrival (1 AoA) with signal coming from RX beam peak direction.

-	Scenario 2: RRM requirement with single Angle of Arrival (1 AoA) with signal coming from RX non-beam peak direction.

-	The test can be performed in any single direction which is covered by Nth percentile EIS spherical coverage of the DUT

-	Value of N is according to FR2 UE power class, as defined in TS 38.101-2 [16] clause 7.3.4.

-	Scenario 3: RRM requirement with two Angle of Arrivals (2 AoAs).

-	Test directions:

-	Both signals come from the directions covered by the N% percentile EIS spherical coverage of the DUT

-	The angle between two probes should match the relative probe spacing of 30, 60, 90, 120, 150 deg and UE is in the directions in which the UE RRM test cases can be performed.

-	Value of N is according to FR2 UE power class, as defined in TS 38.101-2 [16] clause 7.3.4.

-	The following sub-cases are considered

Case 1: TDM transmissions from 2 probes (i.e. each probe transmits both signal and/or artificial noise in TDM manner).

-	Case 2: Simultaneous transmission of signals from 2 probes

-	The details on how to control the SINR for Case 1 and Case 2 are provided in Annex H.

-    An example of SINR calculation for Case 2 is provided in “Spreadsheet 3 - RRM SINR range calculator for 2AoA.xls” file attached to the TR.

Note: No conclusions on the feasibility of generating the testable side conditions made for Case 2 with Mode 1. The feasibility of generating the testable side conditions for Case 2 with Mode 1 can be further studied in the future releases.

The following Types of RRM requirements have been identified and can be supported by the NR RRM test methods:

-	Type 1:

-	RRM requirements defined under the assumption that the UE is using “Fine” UE RX beams.

-	“Fine” UE RX beams are the beams used by the UE to perform PDSCH reception and used to define UE RF requirements (e.g. EIS, EIS spherical coverage)

-	Type 2:

-	Requirements defined under the assumption that the UE is using “Rough” UE RX beams.

-	“Rough” UE RX beams are the beams which the UE is using for RRM measurements (e.g. for SSB measurements)

For selecting the testing direction (AoA to test the requirement) fulfilling certain preconditions, two methods are feasible from the perspective of RRM baseline measurement setup, as follows:

-	Method 1: Run a pretest in the RRM baseline measurement system to identify all the directions (with a given spatial granularity) at which the UE fulfils a given precondition (e.g. spherical coverage EIS). The testing directions are then chosen out of the valid directions, following a given rule. The precondition to be fulfilled, and the rule how to select the testing direction out of the valid directions, are specified in the test description.

-	Method 2: For each given potential direction, test first a given precondition (e.g. minimum TP for a given power), which validates the direction as valid for testing or not. If the direction is valid, test the requirement, if not, jump to the next potential direction following a given rule. The rule how to select the potential directions and the precondition to validate them as testing direction, are specified in the test description.

=================================End of change=================================

=================================Start of change=================================

Annex H:
SINR control for Scenario 3 RRM Test with two Angles of Arrival (2 AoAs)

# H.1	Case 1: TDM transmissions from 2 probes

For Scenario 3 with 2AoA for both Mode 1 and Mode 2, the methods are same as the Scenario 1 and Scenario 2 with 1AoA.

# H.2	Case 2: Simultaneous transmission of signals from 2 probes

## H.2.1	Mode 1

For Mode 1, the SINR equation for signal from probe 1 at baseband is given as follows:

SINR1BB=S1*G1*LossS2+NG2*Loss+N*G1*Loss+Noise_floor

Where S1 and S2 are signal level for probe 1 and probe 2 respectively. G1 and G2 are the antenna gain for probe 1 and probe 2 respectively. Loss is the pathloss between probe 1&2 and DUT. Noise_floor is the total noise at DUT baseband receiver. N is the artificial noise at the reference point for probe 1 and 2 (artificial noise level is identical for probe 1 and probe 2).

Since the wanted noise is set 6dB above UE thermal noise, therefore the total noise of baseband Noise_floor can be ignored when calculating the SINR. And the SINR can be rewritten as:

SINR1BB≈S1*G1*LossS2+NG2*Loss+N*G1*Loss=1S2S1+1SNR1*G2G1+1SNR1

Where SNR1 is the SNR from probe 1, and the key parameter affecting the SINR at baseband is G2/G1.

However, it is difficult to obtain the exact G2/G1 value since it depends on UE implementation, relative probe spacing, and relative probe and DUT orientation. The lower bound of SINR can be derived under assumption on maximum antenna gains difference G2/G1 is determined. For fine beam, the lower bound of G2/G1 is X. And for rough beam, the lower bound of G2/G1 is X+Z-Y.

## H.2.1	Mode 2

For Mode 2, the SINR for signal from probe 1 at baseband is given as follows:

SINR1BB=S1*G1*LossS2*G2*Loss+Noise_floor

If we set the signal level from interfering probe at reference point is 6dB higher than UE thermal noise, then the approximate SINR can be rewritten as follows:

SINR1≈S1S2*G2/G1

Same as for Mode 1, for fine beam the lower bound of G2/G1 is X. And for rough beam, the lower bound of G2/G1 is X+Z-Y.

=================================End of change=================================