3GPP TSG-RAN WG4 Meeting #91												  	         			R4-1906879

Reno, Nevada, USA, 13 – 17 May, 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					TP to TR 38.828, ACI simulation results

Agenda item:			8.2.2

Document for:			Approval

# 1	Introduction

This contribution is a Text Proposal to include into TR 38.828 [1] the simulation results of RF adjacent channel coexistence in Dynamic TDD. The results presented follow our previous contribution [2].

# 2	References

[1]	R4-1905090, TR skeleton for Co-existence study of Cross-Link Interference (CLI), LG Electronics Finland

[2]	R4-1904314, Simulation results for Dynamic TDD coexistence, Nokia, Nokia Shanghai Bell

<<< Start of TP >>>

Annex A: Detailed simulation results for non-zero grid shift

## A.1	FR1 (Simulation results from Nokia)

The FR1 simulations cover scenarios 3-8. Any parameters that differ from section 5.2, or are otherwise clarified further, are listed in Table A.1-1.

Table A.1-1: Simulation parameters for FR1

| Simulation type | Fully dynamic system simulation including CSI measurements and reporting delay |
| --- | --- |
| Layout for nodes, indoor | Indoor layout: 6 BSs per 120m X 50m Distance b = 20m, d = 40m 5 UEs per BS (total 15 UEs per operator) |
| Layout for nodes, macro/indoor | Single Wide Area cell (operator 1) One building with 3 indoor Local Area cells (operator 2) 15 UEs per operator (50% indoor for operator 1, 100% indoor for operator 2) |
| Beamforming and MIMO | No beamforming, MIMO max rank 2 Wide Area base station uses 120 degree sector antenna with 18 dBi gain |
| Traffic model | Full buffer and FTP3 with 10% loading |
| BS scheduler | Single UE scheduled at a time per each BS |
| TDD configurations | DDDDDDDDDU (DL) DUUUUUUUUU (UL) DDDDDUUUUU (mixed DL/UL) |

The simulation results include the baseline synchronized case (i.e. aggressor is 90% DL), mixed case (i.e. aggressor 50% DL and 50% UL), and conflicting case (i.e. aggressor is 90% UL). Statistics are shown first for full buffer case, and second for the 10% load case.

### A.1.1	Scenario 3 (Macro  Indoor DL victim)

#### A.1.1.1	Full Buffer

Figure A.1.1.1-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Indoor DL victim, full buffer traffic

Table A.1.1.1-1: SINR and throughput degradation for Macro aggressor Indoor DL victim, full buffer traffic

| Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | UL | 50DL/50UL | UL |
| 5% | -0.1 | -0.1 | 9.0 | 7.1 |
| 50% | -0.1 | 0.1 | 0.9 | 1.2 |
| 95% | 0.2 | 1.5 | 1.3 | 1.7 |

#### A.1.1.2	FTP3 with 10% load

Figure A.1.1.2-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Indoor DL victim, 10% traffic

Table A.1.1.2-1: SINR and throughput degradation for Macro aggressor Indoor DL victim, 10% traffic

| Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | UL | 50DL/50UL | UL |
| 5% | 0.3 | 1.4 | 4.4 | 7.7 |
| 50% | -0.1 | 2.4 | 1.0 | 6.9 |
| 95% | 0.2 | 0.9 | 0.0 | 0.0 |

#### A.1.1.3	Results evaluation

The results indicate that indoor DL performance is on the average largely unaffected by the choice of macro cell TDD configuration. However, the indoor 5th percentile users may experience some degradation for conflicting TDD configurations. The 95th percentile users, which already have good channel conditions, can tolerate the interference. The 10% load scenario, which results in bursty traffic, seems to have more impact than the more stable full buffer case.

For the baseline scenario (synchronized TDD configs), the ACI interferer is the macro cell DL transmissions, which are relatively low level due to the wall penetration loss. With conflicting TDD configurations, the ACI interferer is instead the macro cell UL transmissions, and if the UE-to-UE distance between the interferer and the victim is short, the ACI could become dominant over co-channel interference from the indoor network. However in the chosen geometry, co-channel interference dominates. With a different indoor geometry, or enhanced inter-cell interference coordination or cancellation, the ACI could become more visible.

### A.1.2	Scenario 4 (Macro  Indoor UL victim)

#### A.1.2.1	Full Buffer

Figure A.1.2.1-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Indoor UL victim, full buffer traffic

Table A.1.2.1-1: SINR and throughput degradation for Macro aggressor Indoor UL victim, full buffer traffic

| Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | DL | 50DL/50UL | DL |
| 5% | -0.6 | -2.2 | -0.9 | -19.0 |
| 50% | -1.0 | -2.0 | -4.2 | -9.5 |
| 95% | -0.6 | -2.1 | -0.1 | -11.1 |

#### A.1.2.2	FTP3 with 10% load

Figure A.1.2.2-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Indoor UL victim, 10% traffic

Table A.1.2.2-1: SINR and throughput degradation for Macro aggressor Indoor UL victim, 10% traffic

| Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | DL | 50DL/50UL | DL |
| 5% | -1.1 | -7.0 | -0.9 | -4.8 |
| 50% | -1.8 | -2.4 | -0.1 | -0.2 |
| 95% | -0.1 | -0.1 | -0.1 | -1.0 |

#### A.1.2.3	Results evaluation

The results indicate, that the indoor cell throughput improves, when the TDD configuration differs between the indoor network and the interfering macro, except for the 95th percentile users, which already have strong UL performance.

The reasoning is that for synchronized TDD, the users connected to the outdoor macro transmit at close to maximum power in UL, causing significant UL ACI. In conflicting TDD, the ACI is result from the macro cell DL transmissions, which are significantly attenuated by the path and wall penetration losses.

### A.1.3	Scenario 5 (Indoor  Macro DL victim)

#### A.1.3.1	Full Buffer

Figure A.1.3.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Macro DL victim, full buffer traffic

Table A.1.3.1-1: SINR and throughput degradation for Indoor aggressor Macro DL victim, full buffer traffic

| Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | UL | 50DL/50UL | UL |
| 5% | 5.5 | -0.1 | 38.2 | 31.8 |
| 50% | 1.2 | -0.4 | 7.4 | -5.9 |
| 95% | 0.7 | -0.6 | 4.0 | 4.9 |

#### A.1.3.2	FTP3 with 10% load

Figure A.1.3.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Macro DL victim, 10% traffic

Table A.1.3.2-1: SINR and throughput degradation for Indoor aggressor Macro DL victim, 10% traffic

| Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | UL | 50DL/50UL | UL |
| 5% | -1.9 | -2.4 | -51.4 | -57.0 |
| 50% | -0.8 | -1.1 | -57.5 | -77.2 |
| 95% | -0.8 | -1.0 | -25.0 | -28.6 |

#### A.1.3.3	Results evaluation

The macro DL SINR curves do not show very large differences with different TDD configurations. First, it could be assumed, that the outdoor users are not significantly affected by the indoor network, due to the wall penetration losses. The results therefore show the impact mostly to the macro UEs that are located indoors.

In the synchronized scenario, the victim UEs can locate very close to the indoor BS transmitters, and get quite significant ACI from the aggressor DL. In the conflicting TDD cases, the aggressor is instead a lower power UE transmitter, which may reduce the overall interference. This is at least visible in the 10% traffic scenario, in which the macro DL performance seems to improve with conflicting TDD.

In the full buffer scenario, the 50% UL/DL split of the aggressor network causes the worst victim DL performance, which is likely a result of signalling delay from DL CSI measurement, to the setting of the MCS in a subsequent TTI. However, the simulation results are relatively noisy at the 5th and 95th percentile observation points.

### A.1.4	Scenario 6 (Indoor  Macro UL victim)

#### A.1.4.1	Full Buffer

Figure A.1.4.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Macro UL victim, full buffer traffic

Table A.1.4.1-1: SINR and throughput degradation for Indoor aggressor Macro UL victim, full buffer traffic

| Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | DL | 50DL/50UL | DL |
| 5% | 0.0 | 0.0 | 7.2 | 20.9 |
| 50% | 0.1 | 0.0 | 17.1 | 15.0 |
| 95% | 0.1 | 0.1 | -0.8 | 0.0 |

#### A.1.4.2	FTP3 with 10% load

Figure A.1.4.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Macro UL victim, 10% traffic

Table A.1.4.2-1: SINR and throughput degradation for Indoor aggressor Macro UL victim, 10% traffic

| Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | DL | 50DL/50UL | DL |
| 5% | 0.0 | 0.0 | 2.8 | -13.6 |
| 50% | 0.2 | 0.0 | 0.6 | -0.2 |
| 95% | 0.0 | 0.0 | 0.8 | -21.6 |

#### A.1.4.3	Results evaluation

There is no real difference in the macro UL SINR curves, when the indoor network TDD configuration is changed. There is some variation in the throughput, but this is likely result of simulation noise.

### A.1.5	Scenario 7 (Indoor  Indoor DL victim)

#### A.1.5.1	Full Buffer

Figure A.1.5.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor DL victim, full buffer traffic

Table A.1.5.1-1: SINR and throughput degradation for Indoor aggressor Indoor DL victim, full buffer traffic

| Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | UL | 50DL/50UL | UL |
| 5% | 0.2 | 0.0 | 0.6 | -0.5 |
| 50% | -0.2 | -0.5 | -0.8 | -0.8 |
| 95% | -0.2 | -2.8 | -1.8 | -1.2 |

#### A.1.5.2	FTP3 with 10% load

Figure A.1.5.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor DL victim, 10% traffic

Table A.1.5.2-1: SINR and throughput degradation for Indoor aggressor Indoor DL victim, 10% traffic

| Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | UL | 50DL/50UL | UL |
| 5% | -0.7 | -0.7 | -6.7 | -6.3 |
| 50% | -5.0 | -6.1 | -5.4 | -5.4 |
| 95% | -3.0 | -3.3 | 0.0 | 0.0 |

#### A.1.5.3	Results evaluation

The results indicate that in the dense indoor deployment case, the conflicting TDD configurations improve DL performance.

The reasoning is that in the synchronized TDD baseline, the high power adjacent channel DL transmissions are significantly higher source of ACI, than in the conflicting TDD case where the source of ACI are the lower power UL transmissions. Also the co-channel interference from own network base stations is rather large in the baseline case, making it difficult to see any potential degradation in the conflicting TDD.

### A.1.6	Scenario 8 (Indoor  Indoor UL victim)

#### A.1.6.1	Full Buffer

Figure A.1.6.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor UL victim, full buffer traffic

Table A.1.6.1-1: SINR and throughput degradation for Indoor aggressor Indoor UL victim, full buffer traffic

| Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | DL | 50DL/50UL | DL |
| 5% | -0.2 | 0.3 | -8.3 | -3.2 |
| 50% | 0.5 | 0.8 | -1.9 | -2.8 |
| 95% | 0.3 | 0.3 | 3.5 | 1.5 |

#### A.1.6.2	FTP3 with 10% load

Figure A.1.6.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor UL victim, 10% traffic

Table A.1.6.2-1: SINR and throughput degradation for Indoor aggressor Indoor UL victim, 10% traffic

| Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | DL | 50DL/50UL | DL |
| 5% | 0.2 | 3.0 | 0.0 | -5.0 |
| 50% | -0.2 | 0.7 | 0.0 | 0.0 |
| 95% | 0.3 | 0.3 | 0.0 | 0.0 |

#### A.1.6.3	Results evaluation

The results indicate that for UL performance, the dense indoor deployment may be resilient to the interfering network’s TDD configuration. In the synchronized baseline scenario, the interfering UEs can be very close to the victim base station, whereas in the conflicting TDD configurations, the interferers are the more distant adjacent channel base stations. Depending on the geometry, either of these mechanisms may dominate, however, there should be relatively good UL link margins available.

## A.2	FR2 (Simulation results from Nokia)

The FR2 simulations cover scenarios 15-16. Any parameters that differ from section 5.2, or are otherwise clarified further, are listed in Table A.2-1.

Table A.2-1: Simulation parameters for FR2

| Simulation type | Fully dynamic system simulation including CSI measurements and reporting delay |
| --- | --- |
| Layout for nodes, indoor | 50m x 120m, 6 BSs per operator Distance between BSs = 20m 3 UEs per BS (18 UEs per operator) |
| Layout for nodes, macro | 3 macro sites with 3 sectors (9 BSs) per operator 5 UEs per BS 100% grid shift |
| Beamforming and MIMO | No beamforming, MIMO max rank 2 Wide Area base station uses 120 degree sector antenna with 18 dBi gain |
| Traffic model | Full buffer and FTP3 with 10% loading |
| BS scheduler | Single UE scheduled at a time per each BS |
| Beamforming | 17 equally distributed beams per polarization Antenna element gain = 3.5 dBi |
| TDD configurations | DDDDDDDDDU (DL) DUUUUUUUUU (UL) DDDDDUUUUU (mixed DL/UL) |

The simulation results include the baseline synchronized case (i.e. aggressor is 90% DL), mixed case (i.e. aggressor 50% DL and 50% UL), and conflicting case (i.e. aggressor is 90% UL). Statistics are shown first for full buffer case, and second for the 10% load case.

### A.2.1	Scenario 9 (Macro  Macro DL victim)

#### A.2.1.1	Full Buffer

Figure A.2.1.1-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Macro DL victim, full buffer traffic

Table A.2.1.1-1: SINR and throughput degradation for Macro aggressor Macro DL victim, full buffer traffic

| Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | UL | 50DL/50UL | UL |
| 5% | -0.7 | -1.5 | -7.9 | -31.1 |
| 50% | -0.3 | -0.7 | -3.0 | -9.0 |
| 95% | -0.1 | -0.4 | 1.7 | -1.1 |

#### A.2.1.2	FTP3 with 10% load

Figure A.2.1.2-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Macro DL victim, 10% traffic

Table A.2.1.2-1: SINR and throughput degradation for Macro aggressor Macro DL victim, 10% traffic

| Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | UL | 50DL/50UL | UL |
| 5% | 1.1 | 1.4 | -3.3 | -2.6 |
| 50% | 0.5 | 0.8 | 0.0 | 0.0 |
| 95% | 0.5 | 0.8 | 0.0 | 1.1 |

#### A.2.1.3	Results evaluation

The SINR curves seem to be well aligned regardless of TDD synchronicity or the conflicting configurations. Any throughput variation is likely due to simulation noise.

### A.2.2	Scenario 10 (Macro  Macro UL victim)

#### A.2.2.1	Full Buffer

Figure A.2.2.1-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Macro UL victim, full buffer traffic

Table A.2.2.1-1: SINR and throughput degradation for Macro aggressor Macro UL victim, full buffer traffic

| Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | DL | 50DL/50UL | DL |
| 5% | 0.1 | 0.8 | -3.1 | 2.0 |
| 50% | 0.1 | 0.4 | 2.2 | 2.8 |
| 95% | 0.1 | 0.3 | 0.4 | 0.1 |

#### A.2.2.2	FTP3 with 10% load

Figure A.2.2.2-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Macro UL victim, 10% traffic

Table A.2.2.2-1: SINR and throughput degradation for Macro aggressor Macro UL victim, 10% traffic

| Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | DL | 50DL/50UL | DL |
| 5% | 0.2 | 0.5 | 1.0 | 3.7 |
| 50% | 0.2 | 0.3 | 1.8 | 1.2 |
| 95% | 0.1 | 0.2 | 3.1 | 2.8 |

#### A.2.2.3	Results evaluation

For the UL scenario, the SINR and throughput seem to degrade by a small amount with conflicting TDD configurations. This is possibly due to simulation noise, as in some cases even a slight throughput boost is observed. The variations are so small that any TDD configuration seems to be functional for coexistence.

### A.2.3	Scenario 15 (Indoor  Indoor DL victim)

#### A.2.3.1	Full Buffer

Figure A.2.3.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor DL victim, full buffer traffic

Table A.2.3.1-1: SINR and throughput degradation for Indoor aggressor Indoor DL victim, full buffer traffic

| Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | UL | 50DL/50UL | UL |
| 5% | 0.1 | 0.3 | 3.7 | 26.7 |
| 50% | -0.1 | 0.4 | 6.3 | 9.2 |
| 95% | -0.1 | -0.1 | -7.5 | 0.4 |

#### A.2.3.2	FTP3 with 10% load

Figure A.2.3.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor DL victim, 10% traffic

Table A.2.3.2-1: SINR and throughput degradation for Indoor aggressor Indoor DL victim, 10% traffic

| Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | UL | 50DL/50UL | UL |
| 5% | 0.1 | 0.2 | 5.4 | 5.4 |
| 50% | 0.1 | 0.1 | 1.8 | 3.6 |
| 95% | 0.0 | 0.0 | 0.0 | 2.1 |

#### A.2.3.3	Results evaluation

The SINR curves seem to be well aligned regardless of TDD synchronicity or the conflicting configurations. Any throughput variation is likely due to simulation noise.

### A.2.4	Scenario 16 (Indoor  Indoor UL victim)

#### A.2.4.1	Full Buffer

Figure A.2.4.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor UL victim, full buffer traffic

Table A.2.4.1-1: SINR and throughput degradation for Indoor aggressor Indoor UL victim, full buffer traffic

| Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | DL | 50DL/50UL | DL |
| 5% | 1.7 | 3.0 | 8.8 | 49.4 |
| 50% | -0.1 | 0.8 | -0.7 | 9.9 |
| 95% | 0.0 | -0.5 | 1.5 | -1.1 |

#### A.2.4.2	FTP3 with 10% load

Figure A.2.4.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor UL victim, 10% traffic

Table A.2.4.2-1: SINR and throughput degradation for Indoor aggressor Indoor UL victim, 10% traffic

| Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- |
|  | SINR degradation (dB) |  | Throughput degradation (%) |  |
|  | 50DL/50UL | DL | 50DL/50UL | DL |
| 5% | 1.1 | 2.5 | 8.1 | 23.3 |
| 50% | 0.2 | 0.5 | 1.8 | 8.7 |
| 95% | 0.0 | 0.3 | 0.0 | 0.0 |

#### A.2.4.3	Results evaluation

For the UL scenario, the SINR and throughput for cell edge users (5th percentile) seems to degrade somewhat with conflicting TDD configurations. This is likely due to the slightly higher ACI to the victim UL receiver from the adjacent channel base station DL transmissions, than what would be the case from adjacent channel UL transmissions which are lower power. The results are highly dependent on the geometry.

Annex B: Detailed analysis for zero grid shift

<<< End of TP >>>