3GPP TSG-RAN WG4 Meeting #91	R4-1906891

# Reno, Nevada, US, May 13 – 17, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.133 (9.3.8) Introduction of inter-frequency SFTD core requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 has in LS R2-1905453 asked RAN4 to introduce inter-frequency SFTD between NR PCell and NR neighbour cell in scope of Rel-15 late drop. Support of such feature is currently missing in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing section 9.3.8 with core requirements for inter-fequency SFTD measurements. The requirements are based on corresponding requirements in EN-DC: - For neighbor cell in FR1: - For SFTD measurements without additional SS-RSRP reporting: Tmeasure_SFTD = [14] SMTC periods - For SFTD measurements with additional SS-RSRP reporting: Tmeasure_SFTD = [19] SMTC periods - For neighbor cell in FR2: - For SFTD measurements without additional SS-RSRP reporting: Tmeasure_SFTD = [112] SMTC periods - For SFTD measurements with additional SS-RSRP reporting: Tmeasure_SFTD = [152] SMTC periods |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-frequency SFTD measurements for supporting configuration of RRM measurements in NR SA networks will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Unchanged Sections Omitted

First Modification

##### 9.3.8	NR inter-frequency SFTD Measurements

9.3.8.1	Introduction

This clause contains requirements on UE that are applicable in RRC_CONNECTED state and conditioned on that no PSCell is configured, and further on that PCell is in FR1, and that PCell and neighbor cell(s) are in different bands. The UE shall, depending on network request, perform inter-frequency SFTD measurement and report SFTD result with or without SS-RSRP. The overall delay includes RRC procedure delay defined in [2], SFTD measurement delay in clause 9.3.8.2, and SFTD measurement reporting delay in clause 9.3.8.3.

9.3.8.2	SFTD Measurement delay

The requirements on SFTD measurement delay defined in this section are applicable under the side condition SCH Ês/Iot ≥ -3 dB for the cells.

The UE shall be able to detect, identify and measure SFTD of up to [3] of the strongest neighbor cells on the carrier frequency provided in the SFTD measurement configuration. Further depending on the SFTD measurement configuration, the UE shall additionally report SS-RSRP for the one or more neighbor cells.

The UE shall be capable of finding the neighbor cell regardless of its SSB position in the SMTC period. The SFTD measurement shall be conducted with sustained connection to the PCell and activated SCell(s) in MCG. Depending on capability, the UE may be allowed to cause interruptions as specified in sub-clause 8.2.2.2.6.

When no MCG DRX is used, the UE shall be capable of determining SFTD within a physical layer measurement period of Tmeasure_SFTD as follows, where SMTC period refers to the one used in the inter-frequency neighbour cell:

- For neighbor cell in FR1:

- For SFTD measurements without additional SS-RSRP reporting: Tmeasure_SFTD = [14] SMTC periods

- For SFTD measurements with additional SS-RSRP reporting: Tmeasure_SFTD = [19] SMTC periods

- For neighbor cell in FR2:

- For SFTD measurements without additional SS-RSRP reporting: Tmeasure_SFTD = [112] SMTC periods

- For SFTD measurements with additional SS-RSRP reporting: Tmeasure_SFTD = [152] SMTC periods

When MCG DRX is used, the same Tmeasure_SFTD1 as for non-DRX applies, but the reporting delay depends on the DRX cycle length in use.

In case an NR PSCell is added, or in case PCell is changed due to handover, the UE shall terminate the inter-frequency SFTD measurement.

The measurement accuracy for the SFTD measurement shall fulfill the requirement in sub-clause 10.1.21. The measurement accuracy for additionally reported SS-RSRP shall fulfill the requirement in sub-clause 10.1.5.1.

9.3.8.3	SFTD Measurement reporting delay

This requirement assumes that the measurement report is not delayed by other RRC signalling on DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

End of First Modification

Unchanged Sections Omitted