3GPP TSG-RAN WG4 Meeting #91	R4-1906895

# Reno, Nevada, US, May 13 – 17, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6529 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 36.133 Side conditions for SFTD in FR2 (Rel-15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-perf |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Side condition on Minimum Io level is missing for SFTD measurement accuracy requirements in FR2. Meanwhile, corresponding side conditions have been agreed for intra- and inter-frequency SS-RSRP measurement accuracy at RAN4#90bis (R4-1904769 Anritsu). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modifying Table 9.1.27.1-3: “NR PSCell, or NR cell Io range conditions for SFTD measurement accuracy in FR2” to reflect the agreements in R4-1904769. Diffrerent side conditions are introduced depending on scenario: - For NR PSCell: Minimum Io level for intra-frequency SS-RSRP accuracy is reused - For NR neighbour cell: Minimum Io level for inter-frequency SS-RSRP accuracy is reused. (Please note that structural changes of the table are not captured by tracked changes). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Side conditions on Minimum Io level for SFTD measurement accuracy requirements remain incomplete for FR2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.27, 9.1.27.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<UNCHANGED SECTIONS OMITTED>

<FIRST CHANGE>

### 9.1.27	SFN and frame Timing Difference (SFTD)

#### 9.1.27.1	SFTD Accuracy Requirement

The SFN and frame timing difference (SFTD) is measured between PCell and NR PSCell under EN-DC, or between PCell and NR cell for inter-RAT SFTD. The inter-RAT SFTD measurement can only be configured for E-UTRA - NR band combinations that are supported by the UE.

The accuracy requirements in Table 9.1.27.1-4 are appilicable under the following conditions:

For PCell SFN and frame timing measurement:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in TS 36.101 [5] Clause 7.3 for reference sensitivity are fulfilled.

No changes to the uplink transmission timing are applied during the measurement period.

RSRP|dBm according to Annex B.3.5 for a corresponding Band.

Io range deifined in Table 9.1.27.1-1.

Table 9.1.27.1-1: PCell Io range conditions for SFTD measurement accuracy

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | E-UTRA operating band groups Note 4, 5 | Minimum Io | Maximum Io |
| Conditions |  | dBm/15kHz Note 2, 3 | dBm/BWChannel |
|  | FDD_A, TDD_A | -121 | -50 |
|  | FDD_C, TDD_C | -120 | -50 |
|  | FDD_D | -119.5 | -50 |
|  | FDD_E, TDD_E | -119 | -50 |
|  | FDD_F | -118.5 | -50 |
|  | FDD_G | -118 | -50 |
|  | FDD_H | -117.5 | -50 |
|  | FDD_N | -114.5 | -50 |
| NOTE 1: When in dBm/15kHz, the minimum Io condition is expressed as the average Io per RE over all REs in that symbol. Io may be different in different symbols within a subframe. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: The condition level is increased by MSD as defined in Sections 7.3B in TS 38.101-3 [54], if applicable depending on E-UTRA – NR band combination. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. NOTE 5: Only E-UTRA bands within EN-DC band combinations as specified in section 5.5B in TS 38.101-3 [54] are applicable. |  |  |  |

For NR PSCell, or NR cell SFN and frame timing measurement:

Conditions defined in TS 38.101-1 [18] Clause 7.3 for reference sensitivity are fulfilled for FR1.

Conditions defined in TS 38.101-2 [19] Clause 7.3 for reference sensitivity are fulfilled for FR2.

Io range deifined in Table 9.1.27.1-2 for FR1.

Io range deifined in Table 9.1.27.1-3 for FR2.

Table 9.1.27.1-2: NR PSCell, or NR cell Io range conditions for SFTD measurement accuracy in FR1

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 4, 5 | Minimum Io Note 2, 3 |  | Maximum Io |
| Conditions |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | -50 |
|  | NR_FDD_FR1_B | -120.5 | -117.5 | -50 |
|  | NR_TDD_FR1_C | -120 | -117 | -50 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -50 |
|  | NR_FDD_FR1_G | -118 | -115 | -50 |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ΔRIB,c as defined in Sections 7.3B in TS 38.101-3 [54], depending on E-UTRA – NR band combination. NOTE 3: The condition level is increased by MSD as defined in Sections 7.3B in TS 38.101-3 [54], if applicable depending on E-UTRA – NR band combination. NOTE 4: NR operating band groups are as defined in Section 3.5. NOTE 5: Only NR bands within EN-DC band combinations as specified in section 5.5B in TS 38.101-3 [54] are applicable. |  |  |  |  |

Table 9.1.27.1-3: NR PSCell, or NR cell Io range conditions for SFTD measurement accuracy in FR2

| Scenario |  | Io Note 1 range |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Minimum Io Note 2, 6, 7 |  |  | Maximum Io |  |
|  |  | dBm/ SCSSSB |  |  | dBm/BWChannel |  |
|  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |  |
| NR PSCell | Same value as SSB_RP in TS 38.133 [50] Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  |  | -50 |  |
| NR neighbour cell | Same value as SSB_RP in TS 38.133 [50] Table B.2.3-2, according to UE Power class, operating band and angle of arrival |  |  | -50 |  |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ΔRIB,c as defined in Sections 7.3B in TS 38.101-3 [54], if applicable depending on E-UTRA – NR band combination. NOTE 3: (Void) NOTE 4: (Void) NOTE 5: Only NR bands within EN-DC band combinations as specified in section 5.5B in TS 38.101-3 [54] are applicable. NOTE 6: Values based on Refsens and EIS spherical coverage as defined in TS 38.101-2 [53] clauses 7.3.2 and 7.3.4. Applicable side condition selected depending on angle of arrival. NOTE 7: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |

Table 9.1.27.1-4: SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot | Frequency range |
| Ts Note 1 | dB |  |
| 40 | -3 dB | FR1 |
| 40 |  | FR2 |
| NOTE 1: Ts is the basic timing unit defined in TS 36.211 [16]. |  |  |

<END OF FIRST CHANGE>

<UNCHANGED SECTIONS OMITTED>