3GPP TSG-RAN WG4 Meeting # 91	R4-1906905

# Reno, USA,  13th May – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-01 | CR | <CR#> | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-01: Test model (section 4.9.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Based on the companion discussion paper R4-1906812, TAE test model is not considered yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the descriptions in the section 4.9.2.3 to distinguish signals from different ports. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - There will be no test model designed for TAE requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Start of CR>

4.9.2	Test models

4.9.2.1	General

The following subclauses will describe the NR FR1 test models needed for BS type 1-C and BS type 1-H. Note that the NR FR1 test models are also applicable to BS type 1-O conformance testing in TS 38.141-2 [3].

4.9.2.2	FR1 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR FR1 test models (NR-FR1-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-1 in TS 38.104 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For FR1-TDD, test models are derived based on the uplink/downlink configuration as shown in the table 4.9.2.2-1 using information element TDD-UL-DL-ConfigCommon as defined in TS 38.331 [19].

Table 4.9.2.2-1: Configurations of TDD for BS type 1-C and BS type 1-H test models

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

Common physical channel parameters for all NR FR1 test models are specified in the following tables: table 4.9.2.2-2 for PDCCH, table 4.9.2.2-3 and table 4.9.2.2-4 for PDSCH. Specific physical channel parameters for NR FR1 test models are described in subclauses 4.9.2.2.1 to 4.9.2.2.8.

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-C and BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control region | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for PDSCH for BS type 1-C and BS type 1-H test models

| Parameter | Value |
| --- | --- |
| Mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | ‘pos2’ |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 1 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-C and BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |

4.9.2.2.1	FR1 test model 1.1 (NR-FR1-TM1.1)

This model shall be used for tests on:

-	BS output power

-	Transmit ON/OFF power

-	TAE

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

-	Transmitter intermodulation

-	Receiver spurious emissions

Common physical channel parameters are defined in subclause 4.9.2.2. Specific physical channel parameters for NR-FR1-TM1.1 are defined in table 4.9.2.2.1-1.

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of QPSK PDSCH PRBs with nRNTI=0 | NRB - 3 |
| Starting RB location of QPSK PDSCH PRBs | 3 |
| Modulation of PDSCH PRBs with nRNTI=2 | Same as PDSCH PRBs |

4.9.2.2.2	FR1 test model 1.2 (NR-FR1-TM1.2)

This model shall be used for tests on:

-	Unwanted emissions

-	ACLR

-	Operating band unwanted emissions

Common physical channel parameters are defined in subclause 4.9.2.2. Specific physical channel parameters for NR-FR1-TM1.2 are defined in table 4.9.2.2.2-1.

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR1-TM1.2

| Parameter | Value |
| --- | --- |
| Percent of QPSK PDSCH PRBs boosted (target) | x=40% |
| # of QPSK PDSCH RBGs which are boosted | NRBG=minxNRB-3xNRBP,12NRB-3P-NRB-3Pmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NRB-3 as the size of the bandwidth part |
| Level of boosting (dB) | 3 |
| NRBGLocations of PDSCH PRBs which are boosted | Pk+3, Pk+4, …, P(k+1)+2 where NRBG-1k=0,2,4…,2NRBG-2 and NRB-3P-1 |
| # of QPSK PDSCH PRBs which are deboosted | NRB-3-PNRBG |
| Level of deboosting (dB) | 10log10NRB-3-103/10PNRBGNRB-3-PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 | QPSK |

4.9.2.2.3	FR1 test model 2 (NR-FR1-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power)

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Common physical channel parameters are defined in subclause 4.9.2.2. Specific physical channel parameters for NR-FR1-TM2 are defined in table 4.9.2.2.3-1.

Table 4.9.2.2.3-1: Specific physical channel parameters of NR-FR1-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
| # of PDSCH PRBs which are not allocated | NRB-1 |

4.9.2.2.4	FR1 test model 2a (NR-FR1-TM2a)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power)

-	EVM of single 256QAM PRB allocation (at min power)

-	Frequency error (at min power)

Physical channel parameters and numbers of the allocated PRB are defined in table 4.9.2.2.3-1 with all 64QAM PDSCH PRBs replaced by 256QAM PDSCH PRBs.

4.9.2.2.5	FR1 test model 3.1 (NR-FR1-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation (at max power)

Physical channel parameters are defined in table 4.9.2.2.1-1 with all QPSK PDSCH PRBs replaced by 64QAM PDSCH PRBs.

4.9.2.2.6	FR1 test model 3.1a (NR-FR1-TM3.1a)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 256QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 256QAM modulation (at max power)

Physical channel parameters are defined in table 4.9.2.2.1-1 with all QPSK PDSCH PRBs replaced by 256QAM PDSCH PRBs.

4.9.2.2.7	FR1 test model 3.2 (NR-FR1-TM3.2)

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for 16QAM modulation

Common physical channel parameters are defined in subclause 4.9.2.2. Specific physical channel parameters for NR-FR1-TM3.2 are defined in table 4.9.2.2.7-1.

Table 4.9.2.2.7-1: Specific physical channel parameters of NR-FR1-TM3.2

| Parameter | Value |
| --- | --- |
| Percent of 16QAM PDSCH PRBs deboosted (target) | x = 60% |
| # of 16QAM PDSCH RBGs within a slot for which EVM is measured | NRBG=minxNRB-3xNRBP,12NRB-3P-NRB-3Pmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NRB-3 as the size of the bandwidth part |
| Level of deboosting (dB) | -3 |
| NRBG Locations of 16QAM PRBs which are deboosted | Pk+3, Pk+4, …, P(k+1)+2 where NRBG-1k=0,2,4…,2NRBG-2 and NRB-3P-1 |
| # of QPSK PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB-3-PNRBG |
| Level of boosting (dB) | 10log10NRB-3-10-3/10PNRBGNRB-3-PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 | QPSK |

4.9.2.2.8	FR1 test model 3.3 (NR-FR1-TM3.3)

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for QPSK modulation

Common physical channel parameters are defined in subclause 4.9.2.2. Specific physical channel parameters for NR-FR1-TM3.3 are defined in table 4.9.2.2.8-1.

Table 4.9.2.2.8-1: Specific physical channel parameters of NR-FR1-TM3.3

| Parameter | Value |
| --- | --- |
| Percent of QPSK PDSCH PRBs deboosted (target) | x = 50 % |
| # of QPSK PDSCH RBGs within a slot for which EVM is measured | NRBG=minxNRB-3xNRBP,12NRB-3P-NRB-3Pmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NRB-3 as the size of the bandwidth part |
| Level of deboosting (dB) | -6 |
| NRBG Locations of QPSK PRBs which are deboosted | Pk+3, Pk+4, …, P(k+1)+2 where k=NRBG-10,2,4…,2NRBG-2 and NRB-3P-1 |
| # of 16QAM PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB-3-PNRBG |
| Level of boosting (dB) | 10log10NRB-3-10-610PNRBGNRB-3-PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 | QPSK |

4.9.2.3	Data content of Physical channels and Signals for NR-FR1-TM

Randomisation of the data content is obtained by utilizing the length-31 Gold sequence scrambling of TS 38.211 [17], subclause 5.2.1 which is invoked by all physical channels prior to modulation and mapping to the RE grid. An appropriate number of ‘0’ bits shall be generated prior to the scrambling.

Initialization of the scrambler and RE-mappers as defined in TS 38.211 [17] use the following additional parameters:

-	 = 1 for the lowest configured carrier,  = 2 for the 2nd lowest configured carrier,…,  = n for the nth configured carrier

-	Antenna ports starting with 1000 for PDSCH

-	Antenna ports starting with 2000 for PDCCH

-	q = 0 (single code word)

-	Rank 1 (single layer)

Note: NR MIMO TAE requirement testing, antenna ports for PDSCH DMRS should be 1000 and 1001 with CDM

4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS 38.211 [17], subclause 5.1.3

-	For each slot the required amount of bits for all PDCCHs is as follows: 1(# of PDCCH) * 1(# of CCE per PDCCH) * 6(REG per CCE) * 9(data RE per REG) * 2(bits per RE) with these parameters according to the NR-FR1-TM definitions in subclause 4.9.2.2

-	Generate this amount of bits according to ‘all 0’ data

-	1 CCE shall be according to TS 38.211 [17], subclause 7.3.2 using non-interleaved CCE-to-REG mapping. PDCCH occupies the first 2 symbols for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [17], subclause 7.3.2.3

-	NID=NIDcell in DM-RS sequence generation in TS 38.211 [17], subclause 7.4.1.3

-	nRNTI=0 in scrambling sequence generation in TS 38.211 [17], subclause 7.3.2.3

-	Perform mapping to REs according to TS 38.211 [17], subclause 7.3.2.5.

4.9.2.3.2	PDSCH

-	For each slot generate the required amount of bits for all PRBs according to ‘all 0’ data

-	NR-FR1-TMs utilize 1, 2 or 3 user PDSCH transmissions distinguished by nRNTI. For each NR-FR1-TM, PRBs are mapped to user (nRNTI) as follows:

Table 4.9.2.3.2-1: Mapping of PRBs to  for NR-FR1-TM

| Test model |  | Number of users |
| --- | --- | --- |
| NR-FR1-TM1.1 | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |
| NR-FR1-TM1.2 | 0 for boosted PRBs 1 for de-boosted PRBs 2 for PRBs located in PRB#0-2 | 3 |
| NR-FR1-TM2 | 2 for all PRBs | 1 |
| NR-FR1-TM2a | 2 for all PRBs | 1 |
| NR-FR1-TM3.1 | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |
| NR-FR1-TM3.1a | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |
| NR-FR1-TM3.2 | 0 for QPSK PRBs 1 for 16QAM PRBs 2 for PRBs located in PRB#0-2 | 3 |
| NR-FR1-TM3.3 | 0 for 16QAM PRBs 1 for QPSK PRBs 2 for PRBs located in PRB#0-2 | 3 |

-	Perform user specific scrambling according to TS 38.211 [17], subclause 7.3.1.1.

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS 38.211 [17], subclause 7.3.1.1

-	nID=NIDcell

-	Perform mapping of the complex-valued symbols to layer according to TS 38.211 [17], subclause 7.3.1.3.   Complex-valued modulation symbols  for codeword q shall be mapped onto the layers ,  where υ is equal to number of layers.

-	Perform PDSCH mapping according to TS 38.211 [17] using parameters listed in table 4.9.2.2-3.

-	PDSCH resource allocation type 1 according to TS 38.214 [18].

-	DM-RS sequence generation according to TS 38.211 [17], subclause 7.4.1.1.1 where l is the OFDM symbol number within the slot with the symbols indicated by table 4.9.2.2-3.

-	NIDnSCID=NIDcell

-	nSCID=0

-	DM-RS mapping according to TS 38.211 [17], subclause 7.4.1.1.2 using parameters listed in table 4.9.2.2-3.

< End of CR>