3GPP TSG-RAN WG4 Meeting #91	R4-1906995

Reno, US, 12th – 17th May 2019

Agenda item:	6.12.2.2	PUSCH	[NR_newRAT-Perf]

Source: 		NTT DOCOMO, INC.

Title: 	NR PUSCH for high speed in Rel.15

Document for:	Approval

- Introduction

In the past meetings, the following agreements were reached for BS performance under HST scenarios [1, 2]. In this contribution, we provide our views on PUSCH for HST.

RAN#81 [1] The UE and BS demodulation performance requirements under HST scenarios are treated in TEI15 after December 2018. RAN4#90bis [2] - Overview - Release to define NR BS HST requirements - Option 1: Release 15 - Option 2: Release 16 with starting point of 350km/h on the same Band 1 as LTE - One of the above options will be decided based on the outcome of the evaluation. - The performance requirements for HST are optional, and the applicability depends on the declaration. - Assumptions for the following HST evaluations are for NR Rel-15 HST for RAN4#91 meeting. Bases on the evaluation results, companies can share their views on the HST requirements definition. - Way forward - Companies are encourage to evaluate PUSCH and PRACH performance under HST for RAN4#91. - If no technical issues are identified and no concern is raised on the evaluation assumptions, Draft CRs implementing framework of PUSCH for HST and PRACH for restricted set type A or/and B, for TS 38.104, TS 38.141-1 and TS 38.141-2, might be provided in RAN4 #91.

2.	Discussion

The current situation of UE capability/RRC signalling and UE/BS demodulation for HST are summarized in Table 1. In E-UTRA, single tap HST tests are specified in Table 8.2.1.1.1-2, 8.2.1.2.1-2, 8.2.2.1.1-2 and 8.2.2.2.1-2, TS 36.101 [3] as mandatory tests. There is no UE capability and RRC signalling. In NR, according to UE feature list [4, 5], there is no UE capability and RRC signalling for single tap HST scenario. Thus, single tap HST tests in UE demodulation will be introduced from Rel.15 as mandatory tests without UE capability signalling. If single tap HST test of BS demodulation is not introduced in Rel.15, it means that NR Rel.15 does not support HST scenario even though single tap HST is a mandatory feature in UE.

Observation 1: If single tap HST tests for BS demodulation are not introduced in Rel.15, it means that NR Rel.15 does not support HST scenario even though a single tap HST is a mandatory feature without UE capability signalling.

Table 1: Summary of UE capability/RRC signalling and UE/BS demodulation for HST

|  | E-UTRA |  | NR |  |
| --- | --- | --- | --- | --- |
|  | Single tap HST scenario | HST-SFN scenario | Single tap HST scenario | HST-SFN scenario |
| UE capability/RRC signalling | None | Required | None | Future study |
| UE demodulation tests (PDSCH) | Introduced from Rel.8 | Introduced from Rel.14 | Will be introduced from Rel.15 | Future study |
| BS demodulation tests (PUSCH) | Introduced from Rel.8 | Covered by single tap HST scenario (Background is captured in sub-clause 6.5.2.1.1, TR 36.878[6]) | Whether introduced from Rel.15 or not is under discussion | Future study |

Regarding the applicability of HST tests, in E-UTRA, single tap HST tests for BS demodulation are explicitly specified as optional tests in Section 8.2.3, TS 36.104 [7].

8.2.3	Requirements for high speed train

The performance requirement of PUSCH for high speed train is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ retransmissions. The performance requirements for high speed train are optional.

This requirement shall not be applied to Local Area BS and Home BS.

In NR, the same approach can be used and single tap HST tests can be specified as optional tests. If BS does not support HST scenario, the BS does not have to test HST scenario. Therefore, there is no impact to existing BSs even if single tap HST is continued discussion in TEI15.

Observation 2: Even if continue discussion on HST in TEI15, there is no impact to existing BSs since the single tap HST tests are optional in BS demodulation.

Proposal 1: Introduce NR HST requirements including PUSCH in Rel.15.

Regarding the assumptions for HST, we propose the following parameters based on evaluation assumptions.

Proposal 2: Define parameters for NR HST as follows:

Table 2: Proposed parameters for NR high speed train scenario

| Parameter | Value |
| --- | --- |
|  | Scenario 3 |
|  | 300 m |
|  | 2 m |
|  | 300 km/h |
|  | 15kHz SCS: 1150Hz 30kHz SCS: 2000Hz |

Table 3: Proposed parameters for NR high speed train test

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR1 |
| Transform precoding | Disabled | Enabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | Option 1: DMRS 1+1, the position of FL DMRS l0= 3 for both 15kHz and 30kHz SCS Option 2: DMRS 1+1+1 for 15kHz and 30kHz SCS | Option 1: DMRS 1+1, the position of FL DMRS l0= 3 for both 15kHz and 30kHz SCS Option 2: DMRS 1+1+1 for 15kHz and 30kHz SCS |
| symbols length | 14 | 14 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type A |
| Frequency domain resource | Full applicable test bandwidth | 15kHz: 25 PRB; 30kHz: 24 PRB (middle of test BW) |
| MCS index | 2 | 2 |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |
| Propagation condition | Scenario 3 with AWGN | Scenario 3 with AWGN |
| SCS and BW | 15kHz: 5MHz, 10MHz, 20MHz; 30kHz: 10MHz, 20MHz, 40MHz, 100MHz |  |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @30% of maximum throughput SNR @70% of maximum throughput | SNR @30% of maximum throughput SNR @70% of maximum throughput |

For introduction of NR HST, we provide draft CRs to capture in TS 38.104, TS 38.141-1 and TS 38.141-2.

Proposal 3: Agree on draft CRs to introduce NR HST to BS demodulation.

3.	Conclusion

In this contribution, we provide parameters for PUSCH under high speed scenarios. The following observations and proposals are obtained.

Observation 1: If single tap HST tests for BS demodulation are not introduced in Rel.15, it means that NR Rel.15 does not support HST scenario even though a single tap HST is a mandatory feature without UE capability signalling.

Observation 2: Even if continue discussion on HST in TEI15, there is no impact to existing BSs since the single tap HST tests are optional in BS demodulation.

Proposal 1: Introduce NR HST requirements including PUSCH and PRACH in Rel.15.

Proposal 2: Define parameters for NR HST as follows:

Table 2: Proposed parameters for NR high speed train scenario

| Parameter | Value |
| --- | --- |
|  | Scenario 3 |
|  | 300 m |
|  | 2 m |
|  | 300 km/h |
|  | 15kHz SCS: 1150Hz 30kHz SCS: 2000Hz |

Table 3: Proposed parameters for NR high speed train test

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR1 |
| Transform precoding | Disabled | Enabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | Option 1: DMRS 1+1, the position of FL DMRS l0= 3 for both 15kHz and 30kHz SCS Option 2: DMRS 1+1+1 for 15kHz and 30kHz SCS | Option 1: DMRS 1+1, the position of FL DMRS l0= 3 for both 15kHz and 30kHz SCS Option 2: DMRS 1+1+1 for 15kHz and 30kHz SCS |
| symbols length | 14 | 14 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type A |
| Frequency domain resource | Full applicable test bandwidth | 15kHz: 25 PRB; 30kHz: 24 PRB (middle of test BW) |
| MCS index | 2 | 2 |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |
| Propagation condition | Scenario 3 with AWGN | Scenario 3 with AWGN |
| SCS and BW | 15kHz: 5MHz, 10MHz, 20MHz; 30kHz: 10MHz, 20MHz, 40MHz, 100MHz |  |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @30% of maximum throughput SNR @70% of maximum throughput | SNR @30% of maximum throughput SNR @70% of maximum throughput |

References

- RP-182149, “Way forward on RAN4 work for Core and Perf. part for NR”, RAN4 Chairman, RAN #81, September 2018

- R4-1904720, “WF on high speed related requirements for NR BS demodulation in Rel.15”, NTT DOCOMO, INC., China Telecom, RAN4 #90bis, April 2019

- “3GPP TS 36.101: Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”, 3GPP

- RP-190124, “LS on UE feature list for NR (R4-1902220; to: RAN2, RAN; cc: RAN1, RAN3; contact: NTT DOCOMO)”, RAN4, RAN#83, March 2019

- RP-190704, “Update of RAN4 UE feature list for REL-15 NR”, NTT DOCOMO, INC., RAN#83, March 2019

- “3GPP TS 36.878: Study on performance enhancements for high speed scenario in LTE”, 3GPP

- “3GPP TS 38.104: NR; Base Station (BS) radio transmission and reception”, 3GPP