3GPP TSG-RAN WG4 Meeting #91	R4-1907002

Reno, US, 12th – 17th May 2019

Agenda item:	7.15.5	BS Demodulation 	[LTE_high_speed_enh2-Perf]

Source: 		NTT DOCOMO, INC.

Title: 	BS demodulation for LTE high speed in Rel.16

Document for:	Approval

1.	Introduction

In RAN4 #90bis, WF on BS demodulation for high speed in Rel.16 was approved [1]. In this contribution, we discuss on open issues and provide our views.

RAN4#90bis - WF on PUSCH for HST - The following deployment scenario are considered: - Open space - Bi-directional (1st priority) - Uni-directional - Tunnel for multi-antennas - Bi-directional (1st priority) - Uni-directional - BS-Railway track distance (Dmin) and Initial distance of the train from BS (Ds/2): - Same as existing HST scenario 1 for open space and 3 for tunnel - Other options are not precluded - WF on PUSCH for HST for evaluation (not for requirement) - Deployment scenarios and related parameters (e.g., Ds, Dmin, fd) will be decided based on evaluation results. The following parameters should be considered in the evaluation. - The requirements will be specified based on the outcome of simulation. - WF on PUSCH for UL timing adjustment - PUSCH requirements for UL timing adjustment considering 500km/h UE velocity - Option 1: Introduce the following moving propagation scenario - Option 2: Not introduce new moving propagation scenario - Companies are encourage to provide the performance impact compared to the existing moving propagation scenario - WF on PUSCH and PUCCH for multipath fading - PUSCH for multipath fading - Option 1: Introduce the requirements with higher Doppler spread above 600Hz - Option 2: Not introduce new requirements - PUCCH for multipath fading - Option 1: Introduce the requirements with higher Doppler spread above 300Hz - Option 2: Not introduce new requirements - Companies are encouraged to provide the practical deployment with multipath fading scenario considering high velocity about 500km/h - WF on PRACH restricted set B - After defining maximum Doppler shift, the necessity of a new test case for PRACH restricted set B would be discussed.

2.	Discussion

2.1	HST scenario

In WF [1], the options for maximum Doppler shift for evaluation were shown as below:

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | Open space |  | Tunnel for multi-antennas |  |
|  | Bi-directional (1st priority) | Uni-directional | Bi-directional (1st priority) | Uni-directional |
| Ds | 1000 m |  | 300 m |  |
| Dmin | 50 m |  | 2 m |  |
| v | 500 km/h |  | 500 km/h (Other options are not precluded) |  |
| fd | 2200Hz 1945Hz (1st priority for initial simulation) 1750Hz 1600Hz Any values in between acceptable are not precluded |  |  |  |

Table 1 shows maximum Doppler shift calculated based on Band 1 (2.1GHz) and Band 7 (2.7GHz). In the exisiting BS HST scenario specified in TS 36.104 [2], Band 1 (2.1GHz) is assumed as carrier frequency. Also in enhanced requirements, at least the same carrier frequency should be assumed. In actual operations, Band 1 (2.1GHz) is one of the main bands for HST coverage and is expected to be used for deployment of enhanced HST covering 500km/h UE speed, also in Japan. However, as shown in Table 2, the maximum Doppler shift of 1750 Hz or 1600 Hz does not seem to guarantee a UE speed of 500 km/h. As the target UE speed is explicitly mentioned in the WID as 500 km/h, 1945 Hz or more should be considered as the maximum Doppler shift

Proposal 1: For BS demodulation requirements for HST, define 1945Hz or more as maximum Doppler shift.

Table 1: UE speed corresponding to Maximum Doppler shift and carrier frequency

| Maximum Doppler Shift | UE speed |  |
| --- | --- | --- |
|  | @Band 1 (2.1GHz) | @Band 7 (2.7GHz) |
| 2200Hz | 565km/h | 440km/h |
| 1945Hz | 500km/h | 388km/h |
| 1750Hz | 450km/h | 350km/h |
| 1600Hz | 411km/h | 320km/h |

2.2	Moving propagation scenario for UL timing adjustment

In TS36.104 [2], moving propagation scenario is defined for PUSCH requirements for UL timing adjustment. There are two scenarios specified as follows:

TS 36.104 Figure B.4-1: Moving propagation conditions (B.4-1) Table B.4-1: Parameters for UL timing adjustment Parameter; Scenario 1; Scenario 2. Channel model; Stationary UE: AWGN Moving UE: ETU200; AWGN. UE speed; 120 km/h; 350 km/h. CP length; Normal; Normal. A; 10 s; 10 s. ; 0.04 s-1; 0.13 s-1. NOTE 1: Multipath fading propagation conditions for Scenario 1 were derived for Band 1 with additional rounding applied to the Doppler frequency calculated for the specified UE speed. NOTE 2: In Scenario 2, Doppler shift is not taken into account.

Existing scenario 2 is defined based on HST scenario (UE speed: 350km/h). Also in the enhanced HST scenario (UE speed: 500km/h), the open space scenario is expected and there is a possibility to multiplex a stationary UE and a moving UE as in the example shown in Figure 1.

Figure 1: Example of deployment scenarios for HST

Based on equation (B.4-1) in TS 36.104, the time difference between the reference timing and the first tap (∆τ) corresponding to ∆ω are shown in Figure 3, where ∆ω=0.04, 0.13 and 0.18 are corresponding to UE speed = 120km/h, 350km/h and 500km/h, respectively. The rationale of  is presented in [3].

Figure X: Time difference between reference timing and the first tap

To guarantee the performance under 500km/h, the requirement for UL timing adjustment should be defined. Our proposed parameters are summarized in Table 2.

Proposal 2: For Rel.16 UL timing adjustment, define a new scenario for UL timing adjustment as follow:

Table 2: Proposed parameters for UL timing adjustment in Rel.16.

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: AWGN |
| UE speed | 500 km/h |
| CP length | Normal |
| A | 10 s |
|  | 0.18 s-1 |
| NOTE: In Scenario X, Doppler shift is not taken into account. |  |

3.	Conclusion

In this contribution, we provide our views on BS demodulation requirements for LTE high speed in Rel.16. The following proposals are obtained.

Proposal 1: For BS demodulation requirements for HST, define 1945Hz or more as maximum Doppler shift.

Proposal 2: For Rel.16 UL timing adjustment, define a new scenario for UL timing adjustment as follow:

Table 2: Proposed parameters for UL timing adjustment in Rel.16.

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: AWGN |
| UE speed | 500 km/h |
| CP length | Normal |
| A | 10 s |
|  | 0.18 s-1 |
| NOTE: In Scenario X, Doppler shift is not taken into account. |  |

References

- R4-1904793, “Way forward on BS demodulation requirement for HST enhancement”, NTT DOCOMO, INC., RAN4 #90bis, April 2019

- “3GPP TS 36.104: Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”, 3GPP

- R4-080223, “Performance requirements on UL Timing Adjustment”, NTT DoCoMo, RAN4 #46, February 2008