3GPP TSG-RAN4 Meeting #91	R4-1907045

Reno, US, May 13th – 17th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.113 | CR | 0079 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.113: Introduction of Band 87 and 88 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE410_Europe_PPDR-Core |  |  |  |  |  | Date: |  |  | 2019-04-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements are missing for Band 87 and 88 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant requirements are introduced for Band 87 and 88 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for Band 87 and 88 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 4.5.2	Receiver exclusion band

The receiver exclusion band for base stations extends from the lower frequency of the Base Station receive band minus 20 MHz to the upper frequency of the Base Station receive band plus 20 MHz. The exclusion bands are as set out below:

- 1900 MHz to 2000 MHz (Band 1) ;

- 1830 MHz to 1930 MHz (Band 2) ;

- 1690 MHz to 1805 MHz (Band 3) ;

- 1690 MHz to 1775 MHz (Band 4) ;

- 804 MHz to 869 MHz (Band 5) ;

- 810 MHz to 860 MHz (Band 6) ;

- 2480 MHz to 2590 MHz (Band 7) ;

- 860 MHz to 935 MHz (Band 8) ;

- 1729.9 MHz to 1804.9 MHz (Band 9) ;

- 1690 MHz to 1790 MHz (Band 10) ;

- 1407.9 MHz to 1467.9 MHz (Band 11) ;

- 679 MHz to 736 MHz (Band 12) ;

- 757 MHz to 807 MHz (Band 13) ;

- 768 MHz to 818 MHz (Band 14) ;

- 684 MHz to 736 MHz (Band 17);

- 795 MHz to 850 MHz (Band 18);

- 810 MHz to 865 MHz (Band 19) ;

- 812 MHz to 882 MHz (Band 20);

- 1427.9 MHz to 1482.9 MHz (Band 21);

- 3390 MHz to 3510 MHz (Band 22);

- 1980 MHz to 2040 MHz (Band 23);

- 1606.5 MHz to 1680.5 MHz (Band 24);

- 1830 MHz to 1935 MHz (Band 25);

- 794 MHz to 869 MHz (Band 26);

- 787 MHz to 844 MHz (Band 27);

- 683 MHz to 768 MHz (Band 28);

- 			N/A 			(Band 29);

- 2285 MHz to 2335 MHz (Band 30);

- 432.5 MHz to 477.5 MHz (Band 31);

- 			N/A 			(Band 32);

- 1880 MHz to 1940 MHz (Band 33);

- 1990 MHz to 2045 MHz (Band 34);

- 1830 MHz to 1930 MHz (Band 35);

- 1910 MHz to 2010 MHz (Band 36);

- 1890 MHz to 1950 MHz (Band 37);

- 2550 MHz to 2640 MHz (Band 38);

- 1860 MHz to 1940 MHz (Band 39);

- 2280 MHz to 2420 MHz (Band 40);

- 2476 MHz to 2710 MHz (Band 41);

- 3380 MHz to 3620 MHz (Band 42);

- 3580 MHz to 3820 MHz (Band 43);

- 683 MHz to 823 MHz (Band 44);

- 1427 MHz to 1487 MHz (Band 45);

- 5130 MHz to 5945 MHz (Band 46)

- 3530 MHz to 3720 MHz (Band 48);

- 3530 MHz to 3720 MHz (Band 49);

- 1412 MHz to 1537 MHz (Band 50);

- 1407 MHz to 1452 MHz (Band 51);

- 3280 MHz to 3420 MHz (Band 52);

- 2463.5 MHz to 2515 MHz (Band 53);

- 1900 MHz to 2030 MHz (Band 65);

- 1690 MHz to 1800 MHz (Band 66);

- 			N/A 			(Band 67);

- 678 MHz to 748 MHz (Band 68);

- 			N/A 			(Band 69);

- 1675 MHz to 1730 MHz (Band 70);

- 643 MHz to 718 MHz (Band 71);

- 431 MHz to 476 MHz (Band 72);

- 430 MHz to 475 MHz (Band 73);

- 1407 MHz to 1490 MHz (Band 74);

- 1412 MHz to 1537 MHz (Band 75);

- 1407 MHz to 1452 MHz (Band 76);

- 678 MHz to 736 MHz (Band 85);

- 390 MHz to 435 MHz (Band 87);

- 392 MHz to 437 MHz (Band 88);

For BS capable of multi-band operation, the total receiver exclusion band shall be the combination of the exclusion bands for each operating band supported by the BS.