3GPP TSG-RAN WG4 Meeting #91		R4-1907107

Reno, US, 13th– 17th May, 2019

Agenda Item:		10.1.1

Source: 		Rohde & Schwarz

Title: 		MU analysis of interpolation near the pole for TRP grids

Document for:	Approval

# Introduction

Previous analysis on the grid resolution for TRP measurements, as shown in [1], were based on simulations with the assumption that all points in the sphere had valid results (i.e. no exclusion/interpolation of results near the pole).

During last meeting, [3] was presented including a proposal to allow the measurements near the pole to be skipped and interpolated instead, but no MU analysis was provided.

This contribution presents additional MU analysis when points near the pole are excluded or interpolated.

# Simulation results

The following simulation results consider the same assumptions made previously in [1] and [2], with the following key assumptions:

- >10,000 antenna random pattern rotations.

- Approximately a constant density distribution of the beam peak within the upper hemisphere assuming the repositioning approach described in [4].

- In addition to the 8x2 antenna array considered as the worst case for the grid study, we also performed simulations for 4x1, 4x2 and 8x1 antenna arrays.

- Only Clenshaw-Curtis weights were analyzed.

The following table summarizes the simulation results for all combinations:

| Number of |  | Mean Error [dB] | Standard Deviation [dB] | Comment | Antenna array |
| --- | --- | --- | --- | --- | --- |
| Latitudes | Longitudes |  |  |  |  |
| 13 | 24 | 0.00 | 0.06 | Reference | 8x2 |
|  |  | 0.00 | 0.04 |  | 8x1 |
|  |  | 0.00 | 0.00 |  | 4x2 |
|  |  | 0.00 | 0.00 |  | 4x1 |
| 12 | 24 | -0.01 | 0.07 | Skipped values zeroed | 8x2 |
|  |  | -0.01 | 0.05 |  | 8x1 |
|  |  | -0.01 | 0.01 |  | 4x2 |
|  |  | -0.01 | 0.01 |  | 4x1 |
|  |  | 0.00 | 0.07 | Skipped values averaged | 8x2 |
|  |  | 0.00 | 0.05 |  | 8x1 |
|  |  | 0.00 | 0.01 |  | 4x2 |
|  |  | 0.00 | 0.01 |  | 4x1 |
| 11 | 24 | -0.08 | 0.15 | Skipped values zeroed | 8x2 |
|  |  | -0.08 | 0.10 |  | 8x1 |
|  |  | -0.09 | 0.12 |  | 4x2 |
|  |  | -0.09 | 0.08 |  | 4x1 |
|  |  | -0.01 | 0.13 | Skipped values averaged | 8x2 |
|  |  | 0.01 | 0.09 |  | 8x1 |
|  |  | 0.00 | 0.08 |  | 4x2 |
|  |  | 0.00 | 0.06 |  | 4x1 |

Table 2-1 Mean Error and Standard Deviation for constant step size measurement grid with 15º step size

Three different types of results are presented for each antenna array:

- Reference: these are the results taking into account all points in the sphere, including the pole, as done in previous simulations.

- Skipped values zeroed: these results consider zero values for the skipped measurement points near the pole.

- Skipped values averaged: these results consider average of the neighboring EIRP values as simple interpolation method for the skipped measurement points near the pole.

As shown in the table, all the simulated combinations yield to a standard deviation of less than 0.25dB, as agreed in [2].

The following figures show the histogram of TRP measurement results for some of the combinations in the previous table:

(a)							(b)

(c)

Figure 2-1 TRP for 8x2 antenna array with 12 latitudes / 24 longitudes

(a) Reference (with 13 latitudes), (b) Skipped values zeroed, (c) Skipped values averaged

(a)							(b)

(c)

Figure 2-2 TRP for 8x2 antenna array with 11 latitudes / 24 longitudes

(a) Reference (with 13 latitudes), (b) Skipped values zeroed, (c) Skipped values averaged

# Summary

According to the results presented in this contribution, the following proposal are made:

Proposal 1: For TRP, the region around the pole (=180º) where the measurements for TRP can be skipped (and interpolated instead) shall be defined based on the analysis of the standard deviation < 0.25 dB.

Proposal 2: For 15º constant step size, the region around the pole (=180º) where the measurements for TRP can be skipped (and interpolated instead) shall be defined as the region beyond 150º in theta.

# References

# 3GPP TR 38.810 V16.2.0 (2019-03)

# R4-1805995, “WF on measurement grids for non-sparse antenna arrays”, Rohde & Schwarz, Anritsu, Intel, Keysight Technologies, RAN4#86bis, April 2018

# R4-1905099, “Draft CR for TR38.810: Interpolation at or near the Pole for TRP”, Keysight Technologies UK Ltd, RAN4#90bis, April 2019.

# R4-1904050, “Draft CR for TR38.810 – UE coordinate system”, Rohde & Schwarz, Keysight Technologies, RAN4#90bis, April 2019.