3GPP TSG-RAN4 Meeting #91	R4-1907188

# Reno, Nevada, United States, 13th May 2019 - 17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to REL-16 TS 38.101-2: addition of NR intra band CA n257C configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Telecom Italia S.p.A. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R16_intra-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 agreed to introduce REL-16 38 series specifications in June 2019 according to the process described in R4-1900047. This Draft CR adds the NR intra band CA n257C configuration |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added NR intra band CA n257C configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR intra band CA n257C is not included in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.1, 5.5A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 series |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

## 5.2	Operating bands

NR is designed to operate in the FR2 operating bands defined in Table 5.2-1.

Table 5.2-1: NR operating bands in FR2

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

## 5.2A	Operating bands for CA

### 5.2A.1	Intra-band CA

NR intra-band contiguous carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1, where all operating bands are within FR2.

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR2

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n257B | n257 |
| CA_n257C | n257 |
| CA_n257D | n257 |
| CA_n257E | n257 |
| CA_n257F | n257 |
| CA_n257G | n257 |
| CA_n257H | n257 |
| CA_n257I | n257 |
| CA_n257J | n257 |
| CA_n257K | n257 |
| CA_n257L | n257 |
| CA_n257M | n257 |
| CA_n260B | n260 |
| CA_n260C | n260 |
| CA_n260D | n260 |
| CA_n260E | n260 |
| CA_n260F | n260 |
| CA_n260G | n260 |
| CA_n260H | n260 |
| CA_n260I | n260 |
| CA_n260J | n260 |
| CA_n260K | n260 |
| CA_n260L | n260 |
| CA_n260M | n260 |
| CA_n260O | n260 |
| CA_n260P | n260 |
| CA_n260Q | n260 |
| CA_n261B | n261 |
| CA_n261C | n261 |
| CA_n261D | n261 |
| CA_n261E | n261 |
| CA_n261F | n261 |
| CA_n261G | n261 |
| CA_n261H | n261 |
| CA_n261I | n261 |
| CA_n261J | n261 |
| CA_n261K | n261 |
| CA_n261L | n261 |
| CA_n261M | n261 |
| CA_n261O | n261 |
| CA_n261P | n261 |
| CA_n261Q | n261 |

<End of change>

<Start of change>

## 5.5	Configurations

## 5.5A	Configurations for CA

### 5.5A.1	Configurations for intra-band contiguous CA

Table 5.5A.1-1: NR CA configurations, bandwidth combination sets and fallback group defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set / Fallback group |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated BW (MHz) | BCS | Fallback group |
|  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |  |
| CA_n257B | CA_n257B | 50 | 400 |  |  |  |  |  |  | 450 | 0 | 1 |
|  |  | 100 | 400 |  |  |  |  |  |  | 500 |  |  |
|  |  | 200 | 400 |  |  |  |  |  |  | 600 |  |  |
|  |  | 400 | 400 |  |  |  |  |  |  | 800 |  |  |
| CA_n257C | CA_n257B | 50 | 400 | 400 |  |  |  |  |  | 850 | 0 | 1 |
|  |  | 100 | 400 | 400 |  |  |  |  |  | 900 |  |  |
|  |  | 200 | 400 | 400 |  |  |  |  |  | 1000 |  |  |
|  |  | 400 | 400 | 400 |  |  |  |  |  | 1200 |  |  |
| CA_n257D | CA_n257D | 50 | 200 |  |  |  |  |  |  | 250 | 0 | 2 |
|  |  | 100 | 200 |  |  |  |  |  |  | 300 |  |  |
|  |  | 200 | 200 |  |  |  |  |  |  | 400 |  |  |
| CA_n257E | CA_n257E | 50 | 200 | 200 |  |  |  |  |  | 450 | 0 |  |
|  |  | 100 | 200 | 200 |  |  |  |  |  | 500 |  |  |
|  |  | 200 | 200 | 200 |  |  |  |  |  | 600 |  |  |
| CA_n257F | CA_n257F | 50 | 200 | 200 | 200 |  |  |  |  | 650 | 0 |  |
|  |  | 100 | 200 | 200 | 200 |  |  |  |  | 700 |  |  |
|  |  | 200 | 200 | 200 | 200 |  |  |  |  | 800 |  |  |
| CA_n257G | CA_n257G | 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n257H | CA_n257H | 100 | 100 | 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n257I | CA_n257I | 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n257J | CA_n257J | 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n257K | CA_n257K | 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n257L | CA_n257L | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n257M | CA_n257M | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |
| CA_n260B | CA_n260B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n260C | CA_n260B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  | 1200 | 0 |  |
| CA_n260D | CA_n260D | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
|  |  | 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |
| CA_n260E | CA_n260E | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 |  |
|  |  | 200 | 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |
| CA_n260F | CA_n260F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 |  |
| CA_n260G | CA_n260G | 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 3 |
|  |  | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |
| CA_n260H | CA_n260H | 100 | 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
|  |  | 50, 100 | 100 | 100 |  |  |  |  |  |  |  |  |
| CA_n260I | CA_n260I | 100 | 100 | 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
|  |  | 50, 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |
| CA_n260J | CA_n260J | 100 | 100 | 100 | 100 | 50, 100 |  |  |  | 500 | 0 |  |
| CA_n260K | CA_n260K | 100 | 100 | 100 | 100 | 100 | 50, 100 |  |  | 600 | 0 |  |
| CA_n260L | CA_n260L | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 |  | 700 | 0 |  |
| CA_n260M | CA_n260M | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 | 800 | 0 |  |
| CA_n260O | CA_n260O | 50, 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n260P | CA_n260P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n260Q | CA_n260Q | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| CA_n261B | CA_n261B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n261C | CA_n261B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  | 8501 | 0 |  |
| CA_n261D | CA_n261D | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
|  |  | 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |
| CA_n261E | CA_n261E | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 |  |
|  |  | 200 | 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |
| CA_n261F | CA_n261F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 |  |
| CA_n261G | CA_n261G | 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 3 |
|  |  | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |
| CA_n261H | CA_n261H | 100 | 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
|  |  | 50, 100 | 100 | 100 |  |  |  |  |  |  |  |  |
| CA_n261I | CA_n261I | 100 | 100 | 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
|  |  | 50, 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |
| CA_n261J | CA_n261J | 100 | 100 | 100 | 100 | 50, 100 |  |  |  | 500 | 0 |  |
| CA_n261K | CA_n261K | 100 | 100 | 100 | 100 | 100 | 50, 100 |  |  | 600 | 0 |  |
| CA_n261L | CA_n261L | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 |  | 700 | 0 |  |
| CA_n261M | CA_n261M | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 | 800 | 0 |  |
|  |  | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |
| CA_n261O | CA_n261O | 50, 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n261P | CA_n261P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n261Q | CA_n261Q | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| NOTE 1: The maximum bandwidth of band n261 is 850MHz |  |  |  |  |  |  |  |  |  |  |  |  |

<End of change>