3GPP TSG-RAN WG4 Meeting #91              	R4-1907287

Reno, US, 13th – 17th May, 2019

Agenda item:	6.12.2.2.1

Source:	Samsung

Title:	Updated simulation results for NR PUSCH

Document for:	Discussion

# 1	Introduction

During RAN4#90b meeting, the remained issue for PUSCH requirements was discussed. Additional test cases for PUSCH are added and captured in WF [1]:

In this contribution, the updated ideal and impairment simulation results are provided.

# 2	Simulation Results

In this section, based on the agreed simulation assumption for NR PUSCH as table 1, table 2. The ideal and impairment simulation results of PUSCH are presented as table 3, to table 11.

Table 1: Simulation assumption for NR PUSCH for CP-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1,2 | 1,2 |
| Number of Rx | 2, 4,8 | 2 |
| Transmission scheme | Identity matrix (TMPI index 0) | n.a. |
| Number of layers | 1, 2 | 1,2 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1(2,11) for type A 1+1(0,10) for type A | 1(0), 1+1(0,8) |
| symbols length | 14 | 10 |
| Time domain resource allocation type | type A, type B | type B |
| MCS index | 2, 16, 20 (20 only for 1Tx) | 2, 16, 20 (20 only for 1 Tx) |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | MCS 2: TDL-B 100ns, 400 Hz MCS 16: TDL-C 300ns, 100 Hz MCS 20: TDL-A 30ns, 10Hz | MCS 2 : TDL-A 30 ns, 300Hz MCS 16: TDL-A 30 ns,300Hz MCS 20: TDL-A 30 ns, 75HZ |
| SCS and BW | 15kHz: 5,10, 20 MHz 30kHz: 10, 20, 40, 100 MHz | 60kHz: 50, 100 MHz 120kHz: 50,100,200 MHz |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | QPSK: Not configured 16 and 64 QAM: (Configured with KPTRS =2, LPTRS =1) and (Not configured, low prority) |

Table 2: Simulation assumption for NR PUSCH for DFT-s-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1,2 | 1 |
| Number of Rx | 2, 4,8 | 1,2 |
| Transmission scheme | Identity matrix (TMPI index 0) | n.a. |
| Number of layers | 1, 2 | 1 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1(2,11) for type A 1+1(0,10) for type B | 1(0), 1+1(0,8) |
| symbols length | 14 | 10 |
| Time domain resource allocation type | type A, type B | type B |
| MCS index | 2, 16, 20 | 2, 16, 20 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | MCS 2: TDL-B 100ns, 400 Hz | MCS 2 : TDL-A 30 ns, 300Hz |
| SCS and BW | 15kHz: 5MHz 30kHz: 10MHz 15kHz: 25 PRB (contig., middle of test BW) 30kHz: 24 PRB (contig., middle of test BW) | 60kHz: 50MHz 120kHz: 50MHz 60kHz: 30 PRB (contig., middle of test BW) 120kHz: 30 PRB (contig., middle of test BW) |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | Not configured |

Table3: The target SNR performance for NR PUSCH with 1Tx2Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR(impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | -2.9 | -0.9 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.53 | 10.53 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.89 | 12.89 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -3.99 | -1.99 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.5 | 10.5 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.37 | 12.37 |
| FR1 mapping type B | 15KHz, 10KHz | TDLB100-400 | MCS2 | 1+1 | -3.21 | -1.21 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.15 | 10.15 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.52 | 12.52 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -4.23 | -2.23 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 7.93 | 9.93 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.3 | 12.3 |
| FR2 with PTRS | 60KHz,100MHz | TDLA30-300 | MCS16 | 1 | 9.93 | 12.43 |
|  |  |  |  | 1+1 | 8.67 | 11.17 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.73 | 14.23 |
|  |  |  |  | 1+1 | 11.59 | 14.09 |
|  | 120KHz, 100MHz | TDLA30-300 | MCS16 | 1 | 9.50 | 12.00 |
|  |  |  |  | 1+1 | 9.06 | 11.56 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.46 | 13.96 |
|  |  |  |  | 1+1 | 11.51 | 14.01 |
| FR2 without PTRS | 60KHz, 100MHz | TDLA30-300 | MCS2 | 1 | -4.43 | -1.93 |
|  |  |  |  | 1+1 | -4.72 | -2.22 |
|  |  | TDLA30-300 | MCS16 | 1 | 9.41 | 11.91 |
|  |  |  |  | 1+1 | 8.28 | 10.78 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.20 | 13.70 |
|  |  |  |  | 1+1 | 11.24 | 13.74 |
|  | 120KHz, 100MHz | TDLA30-300 | MCS2 | 1 | -4.29 | -1.79 |
|  |  |  |  | 1+1 | -4.36 | -1.86 |
|  |  | TDLA30-300 | MCS16 | 1 | 8.80 | 11.30 |
|  |  |  |  | 1+1 | 8.48 | 10.98 |
|  |  | TDLA30-75 | MCS20 | 1 | 10.80 | 13.30 |
|  |  |  |  | 1+1 | 10.90 | 13.40 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -3.01 | -1.01 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.46 | 10.46 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.81 | 12.81 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -3.56 | -1.56 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.4 | 10.4 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.78 | 12.78 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -3.58 | -1.58 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.64 | 10.64 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.45 | 12.45 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -4.51 | -2.51 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.58 | 10.58 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.55 | 12.55 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -4.53 | -2.53 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.49 | 10.49 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 11.24 | 13.24 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | -2.77 | -0.77 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 7.66 | 9.66 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.44 | 12.44 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -2.6 | -0.6 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.03 | 10.3 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.48 | 12.48 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -3.83 | -1.83 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.1 | 10.1 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.4 | 12.4 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -4.16 | -2.16 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 8.01 | 10.01 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 10.4 | 12.4 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | -4.11 | -2.11 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 7.89 | 9.89 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 11.08 | 13.08 |
| FR2 with PTRS | 60KHz, 50MHz | TDLA30-300 | MCS16 | 1 | 9.23 | 11.73 |
|  |  |  |  | 1+1 | 8.51 | 11.01 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.31 | 13.81 |
|  |  |  |  | 1+1 | 11.30 | 13.80 |
|  | 120KHz, 50MHz | TDLA30-300 | MCS16 | 1 | 9.45 | 11.95 |
|  |  |  |  | 1+1 | 8.87 | 11.37 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.36 | 13.86 |
|  |  |  |  | 1+1 | 11.20 | 13.70 |
|  | 120KHz, 200MHz | TDLA30-300 | MCS16 | 1 | 9.55 | 12.05 |
|  |  |  |  | 1+1 | 9.05 | 11.55 |
|  |  | TDLA30-75 | MCS20 | 1 | 11.55 | 14.05 |
|  |  |  |  | 1+1 | 11.54 | 14.04 |
| FR2 without PTRS | 60KHz, 50MHz | TDLA30-300 | MCS2 | 1 | -4.44 | -1.94 |
|  |  |  |  | 1+1 | -4.55 | -2.05 |
|  |  | TDLA30-300 | MCS16 | 1 | 8.99 | 11.49 |
|  |  |  |  | 1+1 | 8.15 | 10.65 |
|  |  | TDLA30-75 | MCS20 | 1 | 10.63 | 13.13 |
|  |  |  |  | 1+1 | 10.62 | 13.12 |
|  | 120KHz, 50MHz | TDLA30-300 | MCS2 | 1 | -3.73 | -1.23 |
|  |  |  |  | 1+1 | -3.99 | -1.49 |
|  |  | TDLA30-300 | MCS16 | 1 | 8.73 | 11.23 |
|  |  |  |  | 1+1 | 8.24 | 10.74 |
|  |  | TDLA30-75 | MCS20 | 1 | 10.71 | 13.21 |
|  |  |  |  | 1+1 | 10.63 | 13.13 |
|  | 120KHz, 200MHz | TDLA30-300 | MCS2 | 1 | -4.23 | -1.73 |
|  |  |  |  | 1+1 | -4.51 | -2.01 |
|  |  | TDLA30-300 | MCS16 | 1 | 8.82 | 11.32 |
|  |  |  |  | 1+1 | 8.39 | 10.89 |
|  |  | TDLA30-75 | MCS20 | 1 | 10.78 | 13.28 |
|  |  |  |  | 1+1 | 10.75 | 13.25 |

Table4: The target SNR performance for NR PUSCH for 1Tx4Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | -7.29 | -5.29 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.63 | 6.63 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 7.23 | 9.23 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -7.51 | -5.51 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.56 | 6.56 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.75 | 8.75 |
| FR1 mapping type B | 15KHz, 10KHz | TDLB100-400 | MCS2 | 1+1 | -6.53 | -4.53 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.25 | 6.25 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 7.03 | 9.03 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -7.42 | -5.42 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.01 | 6.01 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.79 | 8.79 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -7.08 | -5.08 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.56 | 6.56 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 7.21 | 9.21 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -7.08 | -5.08 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.52 | 6.52 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 7.09 | 9.09 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -6.8 | -4.8 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.7 | 6.7 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.73 | 8.73 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -7.7 | -5.7 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.63 | 6.63 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.87 | 8.87 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -7.79 | -5.79 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.54 | 6.54 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 7.55 | 9.55 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | -6.33 | -4.33 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.2 | 6.2 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.94 | 8.94 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -6.16 | -4.16 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.13 | 6.13 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.98 | 8.98 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -7.3 | -5.3 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.16 | 6.13 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.85 | 8.85 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -7.51 | -5.51 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 4.1 | 6.1 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 6.92 | 8.92 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | -7.31 | -5.31 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 3.99 | 5.99 |
|  |  | TDLA30-10 | MCS20 | 1+1 | N.A | N.A |

Table5: The target SNR performance for NR PUSCH with 1 Tx8Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | -9.68 | -7.68 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.59 | 3.59 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.91 | 5.91 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -10.25 | -8.25 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.51 | 3.51 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.51 | 5.51 |
| FR1 mapping type B | 15KHz, 10KHz | TDLB100-400 | MCS2 | 1+1 | -9.62 | -7.62 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.13 | 3.13 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.85 | 5.85 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -10.4 | -8.4 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.74 | 2.74 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.77 | 5.77 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -9.88 | -7.88 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.37 | 3.37 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.92 | 5.92 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -9.47 | -7.47 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.33 | 3.33 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.77 | 5.77 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -9.76 | -7.76 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.67 | 3.67 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.48 | 5.48 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -10.13 | -8.13 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.61 | 3.61 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.58 | 5.58 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -10.36 | -8.36 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 1.68 | 3.68 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 4.17 | 6.17 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | -9.64 | -7.74 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.92 | 2.92 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.81 | 5.81 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -9.21 | -7.21 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.94 | 2.94 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.72 | 5.72 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -10.5 | -8.5 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.99 | 2.99 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.82 | 5.82 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -10.6 | -8.6 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.9 | 2.9 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 3.85 | 5.85 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | -10.3 | -8.3 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 0.85 | 2.85 |
|  |  | TDLA30-10 | MCS20 | 1+1 | 4.15 | 6.15 |

Table6: The target SNR performance for NR PUSCH with 2 Tx2Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | 1.01 | 3.01 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.60 | 18.60 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -0.35 | 1.65 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 18.09 | 20.09 |
| FR1 mapping type B | 15KHz, 10MMHz | TDLB100-400 | MCS2 | 1+1 | 1.85 | 3.85 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.59 | 18.59 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -0.14 | 1.86 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.28 | 18.28 |
| FR2 with PTRS | 60KHz,100MHz | TDLA30-300 | MCS16 | 1 | 6.96 | 9.46 |
|  |  |  |  | 1+1 | 16.53 | 19.03 |
|  | 120KHz, 100MHz | TDLA30-300 | MCS16 | 1 | 19.01 | 21.51 |
|  |  |  |  | 1+1 | 16.81 | 19.31 |
| FR2 without PTRS | 60KHz, 100MHz | TDLA30-300 | MCS2 | 1 | -0.42 | 2.08 |
|  |  |  |  | 1+1 | -1.08 | 1.42 |
|  |  | TDLA30-300 | MCS16 | 1 | NA | NA |
|  |  |  |  | 1+1 | 16.23 | 18.73 |
|  | 120KHz, 100MHz | TDLA30-300 | MCS2 | 1 | -0.14 | 2.36 |
|  |  |  |  | 1+1 | -0.39 | 2.11 |
|  |  | TDLA30-300 | MCS16 | 1 | 17.83 | 20.63 |
|  |  |  |  | 1+1 | 16.31 | 18.81 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -1.2 | -0.8 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.23 | 18.23 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | 0.82 | 2.82 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.49 | 18.49 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | 0.03 | 2.03 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.5 | 18.5 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -0.73 | 1.27 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.5 | 18.5 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -1.01 | 0.99 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.9 | 18.9 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | 0.82 | 2.82 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.21 | 18.21 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -0.1 | 1.9 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.55 | 18.55 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -0.8 | 1.2 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.38 | 18.38 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -0.64 | 1.36 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.37 | 18.37 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | 0.10 | 2.10 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 16.84 | 18.84 |
| FR2 with PTRS | 60KHz, 50MHz | TDLA30-300 | MCS16 | 1 | -13.48 | -10.98 |
|  |  |  |  | 1+1 | 16.54 | 19.04 |
|  | 120KHz, 50MHz | TDLA30-300 | MCS16 | 1 | 18.76 | 21.26 |
|  |  |  |  | 1+1 | 16.86 | 19.36 |
|  | 120KHz, 200MHz | TDLA30-300 | MCS16 | 1 | 19.00 | 21.50 |
|  |  |  |  | 1+1 | 16.65 | 19.15 |
| FR2 without PTRS | 60KHz, 50MHz | TDLA30-300 | MCS2 | 1 | -0.41 | 2.09 |
|  |  |  |  | 1+1 | -1.07 | 1.43 |
|  |  | TDLA30-300 | MCS16 | 1 | NA | NA |
|  |  |  |  | 1+1 | 16.55 | 19.05 |
|  | 120KHz, 50MHz | TDLA30-300 | MCS2 | 1 | -0.30 | 2.20 |
|  |  |  |  | 1+1 | -0.45 | 2.05 |
|  |  | TDLA30-300 | MCS16 | 1 | 17.87 | 20.37 |
|  |  |  |  | 1+1 | 16.29 | 18.79 |
|  | 120KHz, 200MHz | TDLA30-300 | MCS2 | 1 | -0.28 | 2.22 |
|  |  |  |  | 1+1 | -0.48 | 2.02 |
|  |  | TDLA30-300 | MCS16 | 1 | 18.05 | 20.55 |
|  |  |  |  | 1+1 | 16.06 | 18.56 |

Table7: The target SNR performance for NR PUSCH with 2 Tx4Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | -3.2 | -1.2 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.48 | 11.48 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -3.84 | -1.84 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.75 | 11.75 |
| FR1 mapping type B | 15KHz, 10KHz | TDLB100-400 | MCS2 | 1+1 | -4.1 | -2.1 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.46 | 11.46 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -3.92 | -1.92 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.26 | 11.26 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -3.26 | -1.26 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.19 | 11.19 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -3.23 | -1.23 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.22 | 11.22 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -3.35 | -1.35 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.86 | 11.86 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -3.83 | -1.83 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.81 | 11.81 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -4.53 | -2.53 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 10.03 | 12.03 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | -3.23 | -1.23 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.30 | 11.30 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -4.0 | -2.0 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.36 | 11.36 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -4.37 | -2.37 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.33 | 11.33 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -4.13 | -2.13 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.31 | 11.31 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | -4.7 | -2.7 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 9.45 | 11.45 |

Table8: The target SNR performance for NR PUSCH with 2 Tx8Rx with CP-OFDM

| FR | SCS | Channel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR(impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 mapping type A | 15KH, 10MHz | TDLB100-400 | MCS2 | 1+1 | -7.1 | -5.1 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.23 | 7.23 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -6.44 | -4.44 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.26 | 7.26 |
| FR1 mapping type B | 15KHz, 10KHz | TDLB100-400 | MCS2 | 1+1 | -5.47 | -3.47 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.28 | 7.28 |
|  | 30KHz, 40MHz | TDLB100-400 | MCS2 | 1+1 | -6.95 | -4.95 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.15 | 7.15 |
| FR1 mapping type A | 15KHz, 5MH | TDLB100-400 | MCS2 | 1+1 | -5.88 | -3.88 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.19 | 7.19 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -6.9 | -4.9 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.16 | 7.16 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -6.15 | -4.15 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.42 | 7.42 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -6.5 | -4.5 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.28 | 7.28 |
|  | 30KHZ, 100MHz | TDLB100-400 | MCS2 | 1+1 | -7.02 | -5.02 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.34 | 7.34 |
| FR1 mapping type B | 15KHz, 5MHz | TDLB100-400 | MCS2 | 1+1 | -6.61 | -4.61 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.16 | 7.16 |
|  | 15KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -6.9 | -4.9 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.24 | 7.24 |
|  | 30KHz,10MHz | TDLB100-400 | MCS2 | 1+1 | -7.7 | -5.7 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.2 | 7.2 |
|  | 30KHz, 20MHz | TDLB100-400 | MCS2 | 1+1 | -7.14 | -5.14 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.18 | 7.18 |
|  | 30KHz, 100MHz | TDLB100-400 | MCS2 | 1+1 | -6.87 | -4.87 |
|  |  | TDLC300-100 | MCS16 | 1+1 | 5.26 | 7.26 |

Table9: The target SNR performance for NR PUSCH with DFT-s-OFDM

| Number of Antenna | FR | SCS | Chanel | MCS | DMRS configuration | Target SNR (ideal) | Target SNR(impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM 1T2R | FR1 mapping type A | 15KH, 5MHz(25PRB) | TDLB100-400 | MCS2 | 1+1 | -3.82 | -1.82 |
|  |  | 30KHz, 10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -3.77 | -1.77 |
|  | FR1 mapping type B | 15KH, 5MHz(25PRB) | TDLB100-400 | MCS2 | 1+1 | -3.34 | -1.34 |
|  |  | 30KHz, 10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -4.55 | -2.55 |
|  | FR2 without PTRS | 60KHz, 50MHz(30PRB) | TDLA30-300 | MCS2 | 1 | -4.10 | -1.60 |
|  |  |  |  |  | 1+1 | -4.07 | -1.57 |
|  |  | 120KHz, 50MHz(30PRB) | TDLA30-300 | MCS2 | 1 | -3.64 | -1.14 |
|  |  |  |  |  | 1+1 | -3.54 | -1.04 |
| DFT-s-OFDM 1T4R | FR1 mapping type A | 15KHz, 5MH(25PRB) | TDLB100-400 | MCS2 | 1+1 | -6.37 | -4.37 |
|  |  | 30KHZ, 10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -6.73 | -4.73 |
|  | FR1 mapping type B | 15KHz, 5MHz(25PRB) | TDLB100-400 | MCS2 | 1+1 | -7.09 | -5.09 |
|  |  | 30KHz,10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -8.12 | -6.12 |
| DFT-s-OFDM 1T8R | FR1 mapping type A | 15KHz, 5MH(25PRB) | TDLB100-400 | MCS2 | 1+1 | -9.82 | -7.82 |
|  |  | 30KHZ, 10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -9.56 | -7.56 |
|  | FR1 mapping type B | 15KHz, 5MHz(25PRB) | TDLB100-400 | MCS2 | 1+1 | -10.4 | -8.4 |
|  |  | 30KHz,10MHz(24PRB) | TDLB100-400 | MCS2 | 1+1 | -11.3 | -9.3 |

# 3	Conclusion

In this contribution, the updated ideal and impairment simulation results are provided.

# References

R4-1904717, WF on NR PUSCH demodulation requirements, Nokia