3GPP TSG-RAN4 WG4 Meeting #91	R4-1907297

# Reno, Nevada, USA, 13 - 17 May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to FR1 CQI reporting cases in section 6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1: Both wideband and sub-band CQI reporting cases are included under section 6.2.2.2.2, section 6.2.3.1.2 and section 6.2.3.2.2, but the section title are only specifying “Wideband CQI reporting under fading conditions” in the parent sections, it should align with the title of section 6.2.2.1.2; 2: The Measurement channel for cases of 6.2.2.1.2.2, 6.2.2.2.2.2, 6.2.3.1.2.2 and 6.2.3.2.2.2 are still TBD; 3: The Physical channel for CSI report for some cases are still TBD. 4: According to core specification TS 38.214 section 5.2.4, only SP-CSI or aperiodic CSI reporting is applicable for sub-band CQI reporting. 5: Square brackets still exist for SNR values for some cases; 6: K1 values for CQI tests are not exactly same as PDSCH demodulation tests considering some slots are skipped, it is confusing to refer to the K1 values specified in Annex A.1.2 defined for PDSCH demodulation requirements. 7: Some assumptions for the mapping of CQI Index to Information Bit payload calculation in Table A.4-1 and Table A.4-2 are not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1: Removed “Wideband” from section 6.2.2.2.2, section 6.2.3.1.2 and section 6.2.3.2.2 titles and aligned with title of section 6.2.2.1.2; 2: Specified the measurement channel for some cases; 3: Specified physical channel PUSCH for sub-band CQI report cases after changed the CSI reporting type from periodic to aperiodic CSI reporting for sub-band CQI tests and updated the minimum CSI delay from 9.5ms to 6.5ms; 4: Removed the square brackets for SNR values for some cases; 5: Specified the K1 values in the test parameters table to align with FR2 CQI tests and avoid any possible confusions for the referrence to Annex A.1.2. 6: Specified the K1 value for TDD UL-DL pattern FR1.30-1 for FR1 CSI reporting requirements; 7: As per agreements, added notes in Table A.4-1 and Table A.4-2 to specify the some assumptions to derive the mapping of CQI index to Information Bit Payload. 8: Added the missing subband parameters and settings 9: Added the RMC for sub-band CQI tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR1 CQI tests will still be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2, 6.2, Table A.4-1, Table A.4-2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Merged the updates from R4-1905461, R4-1905788 and R4-1906466 |  |  |  |  |  |  |  |  |

6	CSI reporting requirements (Conducted requirements)

6.1	General

This section includes conducted requirements for the reporting of channel state information (CSI).

6.1.1	Applicability of requirements

6.1.1.1	General

The minimum performance requirements are applicable to all FR1 operating bands defined in TS 38.101-1 [6].

The minimum performance requirements in Clause 6 are mandatary for UE supporting NR operation, except test cases listed in Clause 6.1.1.3.

6.1.1.2	Applicability of requirements for different number of RX antenna ports

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2 or 4 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in TS 38.101-1 [6, Clause 7.2]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 6.1.1.2-1.

Table 6.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports only 2RX | CQI | All tests in Clause 6.2.2 |
|  | PMI | All tests in Clause 6.3.2 |
|  | RI | All tests in Clause 6.4.2 |
| UE supports only 4RX or both 2RX and 4RX | CQI | All tests in Clause 6.2.3 |
|  | PMI | All tests in Clause 6.3.3 |
|  | RI | All tests in Clause 6.4.3 |

6.1.1.3	Applicability of requirements for optional UE capabilities

6.1.2	Common test parameters

Parameters specified in Table 6.1.2-1 are applied for all test cases in this section unless otherwise stated.

Table 6.1.2-1: Test parameters for CSI test cases

| Parameter |  |  |  |  | Unit |  | Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  |  |  |  | Transmission scheme 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 3) |  |  |  | RBs |  | 0 |
|  | Subcarrier spacing |  |  |  | kHz |  | 15 or 30 |
| DL BWP configuration #1 | Cyclic prefix |  |  |  |  |  | Normal |
|  | RB offset |  |  |  | RBs |  | 0 |
|  | Number of contiguous PRB |  |  |  | PRBs |  | Maximum transmission bandwidth configuration as specified in TS 38.101-1 [6, Section 5.3.2] for tested channel bandwidth and subcarrier spacing |
| Active DL BWP index |  |  |  |  |  |  | 1 |
| Common serving cell parameters |  | Physical Cell ID |  |  |  |  | 0 |
|  |  | SSB position in burst |  |  |  |  | First SSB in Slot #0 |
|  |  | SSB periodicity |  |  | ms |  | 20 |
| PDCCH configuration |  | Slots for PDCCH monitoring |  |  |  |  | Each slot |
|  |  | Symbols with PDCCH |  |  |  |  | 0,1 |
|  |  | Number of PDCCH candidates and aggregation levels |  |  |  |  | 1/AL8 |
|  |  | DCI format |  |  |  |  | 1_1 |
|  |  | TCI state |  |  |  |  | TCI state #1 |
| Cross carrier scheduling |  |  |  |  |  |  | Not configured |
| PDSCH configuration |  | Mapping type |  |  |  |  | Type A |
|  |  | k0 |  |  |  |  | 0 |
|  |  | Starting symbol (S) |  |  |  |  | 2 |
|  |  | Length (L) |  |  |  |  | 12 |
|  |  | PDSCH aggregation factor |  |  |  |  | 1 |
|  |  | PRB bundling type |  |  |  |  | Static |
|  |  | PRB bundling size |  |  |  |  | 2 |
|  |  | Resource allocation type |  |  |  |  | type 0 |
|  |  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |
|  |  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |
| PDSCH DMRS configuration |  | DMRS Type |  |  |  |  | Type 1 |
|  |  | Number of additional DMRS |  |  |  |  | 1 |
|  |  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |
|  |  | DMRS ports indexes |  |  |  |  | {1000} for Rank1 {1000,1001} for Rank2 {1000,1001,1002} for Rank3 {1000,1001,1002,1003} for Rank4 |
|  |  | Number of PDSCH DMRS CDM group(s) without data |  |  |  |  | 2 |
| PTRS configuration |  | Frequency density (KPT-RS) |  |  |  |  | N/A |
|  |  | Time density (LPT-RS) |  |  |  |  | N/A |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 4 for CSI-RS resource 1 and 3 8 for CSI-RS resource 2 and 4 |
|  |  |  | Number of CSI-RS ports (X) |  |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  |  |  | CDM Type |  |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
|  |  |  | Density (ρ) |  |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  |  |  | CSI-RS periodicity |  | slot |  | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  |  |  | CSI-RS offset |  | slot |  | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  |  |  | Frequency Occupation |  |  |  | Start PRB 0 Number of PRB = BWP size |
|  |  |  | QCL info |  |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition |  |  | Frequency Occupation |  |  |  | Start PRB 0 Number of PRB = BWP size |
|  |  |  | QCL info |  |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition |  |  | Frequency Occupation |  |  |  | Start PRB 0 Number of PRB = BWP size |
| TCI state #0 |  |  | Type 1 QCL information | SSB index |  | SSB #0 |  |  |
|  |  |  |  | QCL Type |  | Type C |  |  |
|  |  |  | Type 2 QCL information | SSB index |  | N/A |  |  |
|  |  |  |  | QCL Type |  | N/A |  |  |
| TCI state #1 |  |  | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |  |  |
|  |  |  |  | QCL Type |  | Type A |  |  |
|  |  |  | Type 2 QCL information | CSI-RS resource |  | N/A |  |  |
|  |  |  |  | QCL Type |  | N/A |  |  |
| Number of HARQ Processes |  |  |  |  |  |  | 4 For FDD 8 for TDD |
| HARQ ACK/NACK bundling |  |  |  |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  |  |  |  | {0,2,3,1} |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  |  | 2 for FDD For FR1.30-1: 8 if mod(i,10) = 0 6 if mod(i,10) = 2 5 if mod(i,10) = 3 5 if mod(i,10) = 4 4 if mod(i,10) = 5 3 if mod(i,10) = 6 Where i is slot index per radio frame with 0~19 |
| Symbols for unused Res |  |  |  |  |  |  | OCNG as specified in A.5 |
| Note 1: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL. Note 2: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 3: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. |  |  |  |  |  |  |  |

6.2	Reporting of Channel Quality Indicator (CQI)

This section includes the requirements for the reporting of channel quality indicator (CQI).

6.2.1	1RX requirements

(Void)

6.2.2	2RX requirements

This sub-clause includes the requirements for reporting of CQI for UE equipped with 2 receiver antennas.

6.2.2.1	FDD

6.2.2.1.1	CQI reporting definition under AWGN conditions

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [12]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

6.2.2.1.1.1	Minimum requirement for periodic CQI reporting

For the parameters specified in Table 6.2.2.1.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.2.1.1.1-1: CQI reporting definition test

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| SNR |  |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 8 |  |  |  |
| Csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 010000 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-2 |  |  |  |

6.2.2.1.2	CQI reporting under fading conditions

6.2.2.1.2.1	Minimum requirement for wideband CQI reporting

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for the frequency non-selective scheduling.

The reporting accuracy of CQI under frequency non-selective fading conditions is determined by the reporting variance, the relative increase of the throughput obtained when the transport format is indicated by the reported CQI compared to the throughput obtained when a fixed transport format is configured according to the reported median CQI, and a minimum BLER using the transport formats indicated by the reported CQI. To account for sensitivity of the input SNR the sub-band CQI reporting under frequency selective fading conditions is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 6.2.2.1.2.1-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time where α% is specified in Table 6.2.2.1.2.1-2;

b)	The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 6.2.2.1.2.1-2;

c)	When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.2.1.2.1-1: Wideband CQI reporting test under frequency non-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| SNR |  |  |  | dB | 6 | 7 | 12 | 13 |
| Propagation channel |  |  |  |  | TDLA30-5 |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 |  |  |  |
| Correlation configuration |  |  |  |  | ULA high |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 8 |  |  |  |
| Csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-1 |  |  |  |

Table 6.2.2.1.2.1-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 20 | 20 |
|  | 1.05 | 1.05 |

6.2.2.1.2.2	Minimum requirement for sub-band CQI reporting

The purpose of the requirements is to verify that the preferred sub-bands can be used for frequency-selective scheduling under the frequency-selective fading conditions.

The accuracy of sub-band channel CQI reporting under the frequency-selective fading conditions is determined by a double-sided percentile of  the reported differential CQI offset level 0 per sub-band, and the relative increase of the throughput obtained when transmitting the transport format indicated by the corresponding reported sub-band CQI on a randomly selected sub-band among the sub-bands with the highest reported differential CQI offset level compared to the throughput when transmitting a fixed transport format according to the wideband CQI median on a randomly selected sub-band among all the sub-bands.

For the parameters specified in Table 6.2.2.1.2.2-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A sub-band differential CQI offset level of 0 shall be reported at least α% of the time but less than β% of the time for each sub-band, where α and β are specified in Table 6.2.2.1.2.2-2;

b)	The ratio of the throughput obtained when transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level and that obtained when transmitting the transport format indicated by the reported wideband CQI median on a randomly selected sub-band among all the sub-bands shall be ≥ γ, where γ is specified in Table 6.2.2.1.2.2-2;

c)	When transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level, the average BLER for the indicated transport format shall be greater than or equal to 0.02.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD.

Table 6.2.2.1.2.2-1: Sub-band CQI reporting test under frequency-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| SNR |  |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  |  | Two tap model specified in Annex B.2.4 with a=1, fD = 5Hz, and τd=0.45μs |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 |  |  |  |
| Correlation configuration |  |  |  |  | As per Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Aperiodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Subband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report interval and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | 0 |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUSCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | TBD |  |  |  |

Table 6.2.2.1.2.2-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
| α [%] | 2 | 2 |
| β [%] | 55 | 55 |
|  | 1.05 | 1.05 |

6.2.2.2	TDD

6.2.2.2.1	CQI reporting definition under AWGN conditions

6.2.2.2.1.1	Minimum requirement for periodic CQI reporting

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS38.214 [12]. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median.

For the parameters specified in Table 6.2.2.2.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.2.2.1.1-1: CQI reporting definition test

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 010000 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-4 |  |  |  |

6.2.2.2.2	CQI reporting under fading conditions

6.2.2.2.2.1	Minimum requirement for wideband CQI reporting

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for the frequency non-selective scheduling.

The reporting accuracy of CQI under frequency non-selective fading conditions is determined by the reporting variance, the relative increase of the throughput obtained when the transport format is indicated by the reported CQI compared to the throughput obtained when a fixed transport format is configured according to the reported median CQI, and a minimum BLER using the transport formats indicated by the reported CQI.

For the parameters specified in Table 6.2.2.2.2.1-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time where α% is specified in Table 6.2.2.2.2.1-2;

b)	The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 6.2.2.2.2.1-2;

c)	When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.2.2.2.1-1: Wideband CQI reporting test under frequency non-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  | dB | 6 | 7 | 12 | 13 |
| Propagation channel |  |  |  |  | TDLA30-5 |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 |  |  |  |
| Correlation configuration |  |  |  |  | ULA high |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-3 |  |  |  |

Table 6.2.2.2.2.1-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 20 | 20 |
|  | 1.05 | 1.05 |

6.2.2.2.2.2	Minimum requirement for sub-band CQI reporting

The purpose of the requirements is to verify that the preferred sub-bands can be used for frequency-selective scheduling under the frequency-selective fading conditions.

The accuracy of sub-band channel CQI reporting under the frequency-selective fading conditions is determined by a double-sided percentile of  the reported differential CQI offset level 0 per sub-band, and the relative increase of the throughput obtained when transmitting the transport format indicated by the corresponding reported sub-band CQI on a randomly selected sub-band among the sub-bands with the highest reported differential CQI offset level compared to the throughput when transmitting a fixed transport format according to the wideband CQI median on a randomly selected sub-band among all the sub-bands.

For the parameters specified in Table 6.2.2.2.2.2-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A sub-band differential CQI offset level of 0 shall be reported at least α% of the time but less than β% of the time for each sub-band, where α and β are specified in Table 6.2.2.2.2.2-2;

b)	The ratio of the throughput obtained when transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level and that obtained when transmitting the transport format indicated by the reported wideband CQI median on a randomly selected sub-band among all the sub-bands shall be ≥ γ, where γ is specified in Table 6.2.2.2.2.2-2;

c)	When transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level, the average BLER for the indicated transport format shall be greater than or equal to 0.02.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each available downlink transmission instance for TDD.

Table 6.2.2.2.2.2-1: Sub-band CQI reporting test under frequency-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  |  | Two tap model specified in Annex B.2.4 with a=1, fD = 5Hz, and τd=0.1125μs |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 |  |  |  |
| Correlation configuration |  |  |  |  | As per Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Aperiodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Subband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report interval and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | 0 |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUSCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | TBD |  |  |  |

Table 6.2.2.2.2.2-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
| α [%] | 2 | 2 |
| β [%] | 55 | 55 |
|  | 1.05 | 1.05 |

6.2.3	4RX requirements

This sub-clause includes the requirements for reporting of CQI for UE equipped with 4 receiver antennas.

6.2.3.1	FDD

6.2.3.1.1	CQI reporting definition under AWGN conditions

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS38.214 [12]. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median.

6.2.3.1.1.1	Minimum requirement for period CQI reporting

For the parameters specified in Table 6.2.3.1.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.2.1.1.1-1: CQI reporting definition test

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| SNR |  |  |  | dB | 5 | 6 | 11 | 12 |
| Propagation channel |  |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 010000 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-2 |  |  |  |

6.2.3.1.2	CQI reporting under fading conditions

6.2.3.1.2.1	Minimum requirement for wideband CQI reporting

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for the frequency non-selective scheduling.

The reporting accuracy of CQI under frequency non-selective fading conditions is determined by the reporting variance, the relative increase of the throughput obtained when the transport format is indicated by the reported CQI compared to the throughput obtained when a fixed transport format is configured according to the reported median CQI, and a minimum BLER using the transport formats indicated by the reported CQI.

For the parameters specified in Table 6.2.3.1.2.1-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time where α% is specified in Table 6.2.3.1.2.1-2;

b)	The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 6.2.3.1.2.1-2;

c)	When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.3.1.2.1-1: Wideband CQI reporting test under frequency non-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| SNR |  |  |  | dB | 3 | 4 | 9 | 10 |
| Propagation channel |  |  |  |  | TDLA30-5 |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 |  |  |  |
| Correlation configuration |  |  |  |  | XP High |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-1 |  |  |  |

Table 6.2.3.1.2.1-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 5 | 5 |
|  | 1.05 | 1.05 |

6.2.3.1.2.2	Minimum requirement for sub-band CQI reporting

The purpose of the requirements is to verify that the preferred sub-bands can be used for frequency-selective scheduling under the frequency-selective fading conditions.

The accuracy of sub-band channel CQI reporting under the frequency-selective fading conditions is determined by a double-sided percentile of  the reported differential CQI offset level 0 per sub-band, and the relative increase of the throughput obtained when transmitting the transport format indicated by the corresponding reported sub-band CQI on a randomly selected sub-band among the sub-bands with the highest reported differential CQI offset level compared to the throughput when transmitting a fixed transport format according to the wideband CQI median on a randomly selected sub-band among all the sub-bands.

For the parameters specified in Table 6.2.3.1.2.2-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A sub-band differential CQI offset level of 0 shall be reported at least α% of the time but less than β% of the time for each sub-band, where α and β are specified in Table 6.2.3.1.2.2-2;

b)	The ratio of the throughput obtained when transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level and that obtained when transmitting the transport format indicated by the reported wideband CQI median on a randomly selected sub-band among all the sub-bands shall be ≥ γ, where γ is specified in Table 6.2.3.1.2.2-2;

c)	When transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level, the average BLER for the indicated transport format shall be greater than or equal to 0.02.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD.

Table 6.2.3.1.2.2-1: Sub-band CQI reporting test under frequency-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| SNR |  |  |  | dB | 5 | 6 | 11 | 12 |
| Propagation channel |  |  |  |  | Two tap model specified in Annex B.2.4 with a=1, fD = 5Hz, and τd=0.45μs |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 |  |  |  |
| Correlation configuration |  |  |  |  | As per Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Aperiodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Subband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report interval and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | 0 |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUSCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | TBD |  |  |  |

Table 6.2.3.1.2.2-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
| α [%] | 2 | 2 |
| β [%] | 55 | 55 |
|  | 1.05 | 1.05 |

6.2.3.2	TDD

6.2.3.2.1	CQI reporting definition under AWGN

6.2.3.2.1.1	Minimum requirement for CQI periodic reporting

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS38.214 [12]. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median.

For the parameters specified in Table 6.2.3.2.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.3.2.1.1-1: CQI reporting definition test

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  | dB | 5 | 6 | 11 | 12 |
| Propagation channel |  |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 010000 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-4 |  |  |  |

6.2.3.2.2	CQI reporting under fading conditions

6.2.3.2.2.1	Minimum requirement for wideband CQI reporting

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for the frequency non-selective scheduling.

The reporting accuracy of CQI under frequency non-selective fading conditions is determined by the reporting variance, the relative increase of the throughput obtained when the transport format is indicated by the reported CQI compared to the throughput obtained when a fixed transport format is configured according to the reported median CQI, and a minimum BLER using the transport formats indicated by the reported CQI.

For the parameters specified in Table 6.2.3.2.2.1-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time where α% is specified in Table 6.2.3.2.2.1-2;

b)	The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 6.2.3.2.2.1-2;

c)	When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.3.2.2.1-1: Wideband CQI reporting test under frequency non-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  | dB | 3 | 4 | 9 | 10 |
| Propagation channel |  |  |  |  | TDLA30-5 |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 |  |  |  |
| Correlation configuration |  |  |  |  | XP High |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-3 |  |  |  |

Table 6.2.3.2.2.1-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | 5 | 5 |
|  | 1.05 | 1.05 |

6.2.3.2.2.2	Minimum requirement for sub-band CQI reporting

The purpose of the requirements is to verify that the preferred sub-bands can be used for frequency-selective scheduling under the frequency-selective fading conditions.

The accuracy of sub-band channel CQI reporting under the frequency-selective fading conditions is determined by a double-sided percentile of  the reported differential CQI offset level 0 per sub-band, and the relative increase of the throughput obtained when transmitting the transport format indicated by the corresponding reported sub-band CQI on a randomly selected sub-band among the sub-bands with the highest reported differential CQI offset level compared to the throughput when transmitting a fixed transport format according to the wideband CQI median on a randomly selected sub-band among all the sub-bands.

For the parameters specified in Table 6.2.3.2.2.2-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A sub-band differential CQI offset level of 0 shall be reported at least α% of the time but less than β% of the time for each sub-band, where α and β are specified in Table 6.2.3.2.2.2-2;

b)	The ratio of the throughput obtained when transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level and that obtained when transmitting the transport format indicated by the reported wideband CQI median on a randomly selected sub-band among all the sub-bands shall be ≥ γ, where γ is specified in Table 6.2.3.2.2.2-2;

c)	When transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level, the average BLER for the indicated transport format shall be greater than or equal to 0.02.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each available downlink transmission instance for TDD.

Table 6.2.3.2.2.2-1: Sub-band CQI reporting test under frequency-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  | dB | 5 | 6 | 11 | 12 |
| Propagation channel |  |  |  |  | Two tap model specified in Annex B.2.4 with a=1, fD = 5Hz, and τd=0.1125μs |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 |  |  |  |
| Correlation configuration |  |  |  |  | As per Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Aperiodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Subband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report interval and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | 0 |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUSCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | TBD |  |  |  |

Table 6.2.2.1.2.2-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
| α [%] | 2 | 2 |
| β [%] | 55 | 55 |
|  | 1.05 | 1.05 |

6.3	Reporting of Precoding Matrix Indicator (PMI)

The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reports compared to the case when the transmitter is using random precoding, respectively. When the transmitter uses random precoding, for each PDSCH allocation a precoder is randomly generated and applied to the PDSCH. A fixed transport format (FRC) is configured for all requirements.

The requirements for transmission mode 1 with higher layer parameter codebookType set to ‘typeI-SinglePanel’ are specified in terms of the ratio:

In the definition of γ, for 4TX and 8TX PMI requirements, is 90 % of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

6.3.1	1RX requirements

(Void)

6.3.2	2RX requirements

6.3.2.1	FDD

6.3.2.1.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.1-2.

Table 6.3.2.1.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 2 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Section Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset |  | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.1 FDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the eNB downlink before slot#(n+3). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.2.1.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

6.3.2.1.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.1.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.2-2.

Table 6.3.2.1.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 2 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Section Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.2 |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the eNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.2.1.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

6.3.2.2	TDD

6.3.2.2.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.2.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.1-2.

Table 6.3.2.2.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configuration |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 2 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Section Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 10/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 5.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.1 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot #n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the eNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.2.2.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

6.3.2.2.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.2.2-1, and using the downlink physical channels specified in Annex TBD, the minimum requirements are specified in Table 6.3.2.2.2-2.

Table 6.3.2.2.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 2 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Section Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 10/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.2 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the eNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.2.2.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

6.3.3	4RX requirements

6.3.3.1	FDD

6.3.3.1.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.1-2.

Table 6.3.3.1.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 4 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Section Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.1 FDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the eNB downlink before slot#(n+3). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.3.1.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

6.3.3.1.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.1.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.2-2.

Table 6.3.3.1.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 4 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Section Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.2 FDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the eNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.3.1.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

6.3.3.2	TDD

6.3.3.2.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.2.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.1-2.

Table 6.3.3.2.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configuration |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 4 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Section Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset |  | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 10/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 5.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.1 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the eNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.3.2.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.3 |

6.3.3.2.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.2.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.2-2.

Table 6.3.3.2.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 4 (N1,N2) = (4,1) |
| Beamforming Model |  |  | As specified in Section Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannnelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 10/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.2 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the eNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.3.2.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.5 |

6.4	Reporting of Rank Indicator (RI)

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

6.4.1	1RX requirements

(Void)

6.4.2	2RX requirements

6.4.2.1	FDD

The minimum performance requirement in Table 6.4.2.1-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.2.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.2.1-2.

Table 6.4.2.1-1: RI Test (FDD)

| Parameter |  |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 | 10 | 10 |
| Subcarrier spacing |  |  |  | kHz | 15 | 15 | 15 |
| Duplex Mode |  |  |  |  | FDD | FDD | FDD |
| SNR |  |  |  | dB | 0 | 20 | 20 |
| Propagation channel |  |  |  |  | TDLA30-5 | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  |  | ULA Low 2x2 | ULA Low 2x2 | ULA High 2x2 |
| Beamforming Model |  |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 4 | 4 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (9,-) | (9,-) | (9,-) |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 | 5/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 | 2 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 | 5/1 | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | Pattern 0 | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) | (4,9) | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 | 5/1 | 5/1 |
| ReportConfigType |  |  |  |  | Periodic | Periodic | Periodic |
| CQI-table |  |  |  |  | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  |  |  | RB | 8 | 8 | 8 |
| csi-ReportingBand |  |  |  |  | 1111111 | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 | 5/1 | 5/1 |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  |  | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | N/A | N/A | N/A |
|  |  | CodebookSubsetRestriction |  |  | 010000 for fixed rank 2, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank |
|  |  | RI Restriction |  |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  |  |  | PUCCH | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  |  | ms | 8 | 8 | 8 |
| Maximum number of HARQ transmission |  |  |  |  | 1 | 1 | 1 |
| RI Configuration |  |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI |

Table 6.4.2.1-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 |
| 2 | 1.0 | N/A | N/A |

6.4.2.2	TDD

The minimum performance requirement in Table 6.4.2.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.2.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.2.2-2.

Table 6.4.2.2-1: RI Test (TDD)

| Parameter |  |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 | 40 | 40 |
| Subcarrier spacing |  |  |  | kHz | 30 | 30 | 30 |
| Duplex Mode |  |  |  |  | TDD | TDD | TDD |
| TDD Slot Configuration |  |  |  |  | FR1.30-1 | FR1.30-1 | FR1.30-1 |
| SNR |  |  |  | dB | 0 | 20 | 20 |
| Propagation channel |  |  |  |  | TDLA30-5 | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  |  | ULA Low 2x2 | ULA Low 2x2 | ULA High 2x2 |
| Beamforming Model |  |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 4 | 4 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (9,-) | (9,-) | (9,-) |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 | 2 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | Pattern 0 | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) | (4,9) | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 |
| ReportConfigType |  |  |  |  | Periodic | Periodic | Periodic |
| CQI-table |  |  |  |  | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  |  |  | RB | 16 | 16 | 16 |
| csi-ReportingBand |  |  |  |  | 1111111 | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 | 10/1 | 10/1 |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  |  | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | N/A | N/A | N/A |
|  |  | CodebookSubsetRestriction |  |  | 010000 for fixed rank 2, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank |
|  |  | RI Restriction |  |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  |  |  | PUCCH | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 | 9.5 | 9.5 |
| Maximum number of HARQ transmission |  |  |  |  | 1 | 1 | 1 |
| RI Configuration |  |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI |

Table 6.4.2.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 |
| 2 | 1.0 | N/A | N/A |

6.4.3	4RX requirements

6.4.3.1	FDD

The minimum performance requirement in Table 6.4.3.1-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.3.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.3.1-2.

Table 6.4.3.1-1: RI Test (FDD)

| Parameter |  |  |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 | 10 | 10 | 10 |
| Subcarrier spacing |  |  |  | kHz | 15 | 15 | 15 | 15 |
| Duplex Mode |  |  |  |  | FDD | FDD | FDD | FDD |
| SNR |  |  |  | dB | -2 | 16 | 16 | 22 |
| Propagation channel |  |  |  |  | TDLA30-5 | TDLA30-5 | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  |  | ULA Low 2x4 | ULA Low 2x4 | ULA High 2x4 | ULA Low 4x4 |
| Beamforming Model |  |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 4 | 4 | 4 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (9,-) | (9,-) | (9,-) | (9,-) |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 | 5/1 | 5/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 | 2 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) | Row 4 (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 | 5/1 | 5/1 | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | Pattern 0 | Pattern 0 | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) | (4,9) | (4,9) | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 | 5/1 | 5/1 | 5/1 |
| ReportConfigType |  |  |  |  | Periodic | Periodic | Periodic | Periodic |
| CQI-table |  |  |  |  | Table 2 | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  | not configured | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | not configured | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband | Wideband |
| Sub-band Size |  |  |  | RB | 8 | 8 | 8 | 8 |
| csi-ReportingBand |  |  |  |  | 1111111 | 1111111 | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 | 5/1 | 5/1 | 5/1 |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  |  | 1 | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | N/A | N/A | N/A | (2,1) |
|  |  | CodebookSubsetRestriction |  |  | 010000 for fixed rank 2, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 11111111 |
|  |  | RI Restriction |  |  | N/A | N/A | N/A | 00000010 for fixed Rank 2 and 00001111 for follow RI |
| Physical channel for CSI report |  |  |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  |  | ms | 8 | 8 | 8 | 8 |
| Maximum number of HARQ transmission |  |  |  |  | 1 | 1 | 1 | 1 |
| RI Configuration |  |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 2 and follow RI |

Table 6.4.3.1-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 | N/A |
| 2 | 0.9 | N/A | N/A | 0.9 |

6.4.3.2	TDD

The minimum performance requirement in Table 6.4.3.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.3.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.3.2-2.

Table 6.4.3.2-1: RI Test (TDD)

| Parameter |  |  |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 | 40 | 40 | 40 |
| Subcarrier spacing |  |  |  | kHz | 30 | 30 | 30 | 30 |
| Duplex Mode |  |  |  |  | TDD | TDD | TDD | TDD |
| TDD Slot Configuration |  |  |  |  | FR1.30-1 | FR1.30-1 | FR1.30-1 | FR1.30-1 |
| SNR |  |  |  | dB | -2 | 16 | 16 | 22 |
| Propagation channel |  |  |  |  | TDLA30-5 | TDLA30-5 | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  |  | ULA Low 2x4 | ULA Low 2x4 | ULA High 2x4 | ULA Low 4x4 |
| Beamforming Model |  |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 4 | 4 | 4 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (9,-) | (9,-) | (9,-) | (9,-) |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 | 2 | 2 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) | Row 4 (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | Pattern 0 | Pattern 0 | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) | (4,9) | (4,9) | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 | 10/1 |
| ReportConfigType |  |  |  |  | Periodic | Periodic | Periodic | Periodic |
| CQI-table |  |  |  |  | Table 2 | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  | not configured | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | not configured | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband | Wideband |
| Sub-band Size |  |  |  | RB | 16 | 16 | 16 | 16 |
| csi-ReportingBand |  |  |  |  | 1111111 | 1111111 | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 | 10/1 | 10/1 | 10/1 |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  |  | 1 | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | N/A | N/A | N/A | (2,1) |
|  |  | CodebookSubsetRestriction |  |  | 010000 for fixed rank 2, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 11111111 |
|  |  | RI Restriction |  |  | N/A | N/A | N/A | 00000010 for fixed Rank 2 and 00001111 for follow RI |
| Physical channel for CSI report |  |  |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 | 9.5 | 9.5 | 9.5 |
| Maximum number of HARQ transmission |  |  |  |  | 1 | 1 | 1 | 1 |
| RI Configuration |  |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 2 and follow RI |

Table 6.4.3.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| 1 | N/A | 1.05 | 0.9 | N/A |
| 2 | 0.9 | N/A | N/A | 0.9 |

<End of change>