3GPP TSG-RAN WG4 Meeting #91	  	R4-1907300

Reno, USA, 13th – 17th May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-4 on CSI reporting requirements for Interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2019-05-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Section on interworking requirements is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added sections for applicability for optional and mandatory with capability signaling features - Added section heading to sections with ‘Void’ title - Delete Editors’ note - Align wording with Demodulation interworking requirements - Add Void in sections with no requirements - Delete content in sections on EN-DC including FR1 and FR2 NR carriers |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interworking requirements are incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2,10.3,10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 38.521-4 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change #1

### 10.1.1	Applicability of requirements

The following applicability rules are specified for demodulation performance requirements for interworking:

-	For UEs supporting both SA and NSA,

-	The performance requirements specified in Section 6 will be verified only for SA.

-	The performance requirements specified in Section 8 will be verified only for SA.

-	The FR1 EN-DC test cases with the NR TDD DL-UL configurations which are not aligned with LTE’s can be tested on the corresponding EN-DC band combinations where UE supports simultaneous transmission and reception.

#### 10.1.1.1	Applicability of requirements for optional UE features

For UE which supports optional UE features the additional performance requirements from Table 10.1.1.1-1 should be applied.

Table 10.1.1.1-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR1 FDD | PDSCH | 6.2.3.1.1 .1 Minimum requirement for CQI periodic reporting 6.3.3.1.2 Single PMI with 8TX TypeI-SinglePanel Codebook 6.4.3.1 (Test 1 - 4) | EN-DC |
|  | FR1 TDD | PDSCH | 6.2.3.2.1.1 Minimum requirement for CQI periodic reporting 6.3.3.2.2 Single PMI with 8TX TypeI-SinglePanel Codebook 6.4.3.2 (Test 1 - 4) | EN-DC |
|  | FR2 TDD | PDSCH | 8.2.2.2.1.1 Minimum requirement for periodic CQI reporting 8.4.2.2 (Test 1 – 3) | EN-DC |

#### 10.1.1.2	Applicability of requirements for mandatory UE features with capability signalling

End of Change #1

Start of Change #2

## 10.2	Reporting of Channel Quality Indicator (CQI)

## 10.2A	Reporting of Channel Quality Indicator (CQI) for CA

## (Void)

## 10.2B	Reporting of Channel Quality Indicator (CQI) for DC

### 10.2B.1	EN-DC

#### 10.2B.1.1	EN-DC within FR1

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR CQI reporting requirements are specified in Section 6.2. During the test, only the performance based on NR requirements on the NR cell(s) shall be verified.

#### 10.2B.1.2	EN-DC including FR2 NR carrier

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR CQI reporting requirements are specified in Section 8.2. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified.

#### 10.2B.1.3	EN-DC including FR1 and FR2 NR carriers

### (Void)

### 10.2B.2	NR DC between FR1 and FR2

## 10.3	Reporting of Precoding Matrix Indicator (PMI)

## 10.3A	Reporting of Precoding Matrix Indicator (PMI) for CA

(Void)

## 10.3B	Reporting of Precoding Matrix Indicator (PMI) for DC

### 10.3B.1	EN-DC

#### 10.3B.1.1	EN-DC within FR1

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PMI reporting requirements are specified in Section 6.3. During the test, only the performance based on NR requirements on the NR cell(s) shall be verified.

#### 10.3B.1.2	EN-DC including NR FR2 carrier

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PMI reporting requirements are specified in Section 8.3. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified.

#### 10.3B.1.3	EN-DC including FR1 and FR2 NR carriers

(Void)

### 10.3B.2	NR DC between FR1 and FR2

## 10.4	Reporting of Rank Indicator (RI)

## 10.4A	Reporting of Rank Indicator (RI) for CA

## (Void)

## 10.4B	Reporting of Rank Indicator (RI) for DC

### 10.4B.1	EN-DC

#### 10.4B.1.1	EN-DC within FR1

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR RI reporting requirements are specified in Section 6.4. During the test, only the performance based on NR requirements on the NR cell(s) shall be verified.

#### 10.4B.1.2	EN-DC including NR FR2 carrier

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR RI reporting requirements are specified in Section 8.4. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified.

#### 10.4B.1.3	EN-DC including FR1 and FR2 NR carriers

(Void)

### 10.4B.2	NR DC between FR1 and FR2

End of Change #2