3GPP TSG-RAN4 Meeting #91 	R4-1907386

# Reno, US, 13 – 17 May, 2019

Title:	LS to RAN2 on SRS-RSRP measurement resource configuration

Response to:

Release:	Rel-16

Work Item:	NR_CLI_RIM-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

Name:	Jin-Yup Hwang

E-mail Address:	jinyup.hwang@lge.com

1. Overall Description:

RAN4 has discussed SRS-RSRP measurement resource configuration and has agreed to change some parameters (highlighted yellow part) as following table:

| Parameter name in specification | Final decision in RAN4 |
| --- | --- |
| SRS-scs | 15, 30, 60 kHz for FR1 60, 120 kHz for FR2 |
| nrofSRS-Ports | 1 |
| transmissionComb | n2 comboffset (0,1), cyclicShift (0,…,7) n4 comboffset (0,..,3), cyclicShift (0,…,11) |
| resourceMapping | startPosition INTEGER (0,…,5) nrofSymbols ENUMERATED {n1} repetitionFactor ENUMERATED {n1} |
| freqDomainPosition | 0,…,67 |
| freqDomainShift | 0,…,268 |
| freqHopping note1 | c-SRS INTEGER (0,…,63) b-SRS INTEGER (0,…,3) b-hop INTEGER (0,…,3) |
| groupOrSequenceHopping | neither |
| SRS-Measurement-PeriodicityAndOffset (only periodic) | sl1 NULL, sl2 INTEGER(0..1), sl4 INTEGER(0..3), sl5 INTEGER(0..4), sl8 INTEGER(0..7), sl10 INTEGER(0..9), sl16 INTEGER(0..15), sl20 INTEGER(0..19), sl32 INTEGER(0..31), sl40 INTEGER(0..39), sl64 INTEGER(0..63), sl80 INTEGER(0..79), sl160 INTEGER(0..159), sl320 INTEGER(0..319), sl640 INTEGER(0..639) |
| Note 1: b-hop and b-SRS should be configured with condition b-hop > b-SRS. The condition means no frequency hopping. |  |

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above RAN4 agreement for SRS-RSRP measurement resource configuration for their further work.

3. Date of Next TSG-RAN4 Meetings:

3GPP RAN4 #92 		26 - 30 August 2019, 		, Slovenia

3GPP RAN4 #92BIS 		14 - 18 October 2019, 		, China