3GPP TSG-RAN WG4 Meeting #91												  	         R4-190xxxx

Reno, USA, 13 – 17 May, 2019

Source: 	LG Uplus

Title: 					TP to TR 38.716-02-00 CA_n78-n257 with 2UL

Agenda Item: 		9.2.3

Document for:	Approval

1	Introduction

This TP introduces NR Carrier Aggregation CA_n78A-n257 with 2UL into the TR.

| NR CA configuration | Uplink Configuration | BCS | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n78A-n257G | CA_n78A-n257A | 0 | Yunsung Kim, LG Uplus | kimy0011@lguplus.co.kr | LG Electronics, Huawei, HiSilicon | new | (completed) DL_n78A-n257G_UL_n78A (completed) DL_n78A-n257G_UL_n257A |
| CA_n78A-n257H | CA_n78A-n257A | 0 | Yunsung Kim, LG Uplus | kimy0011@lguplus.co.kr | LG Electronics, Huawei, HiSilicon | new | (completed) DL_n78A-n257H_UL_n78A (completed) DL_n78A-n257H_UL_n257A |
| CA_n78A-n257I | CA_n78A-n257A | 0 | Yunsung Kim, LG Uplus | kimy0011@lguplus.co.kr | LG Electronics, Huawei, HiSilicon | new | (completed) DL_n78A-n257I_UL_n78A (completed) DL_n78A-n257I_UL_n257A |
| CA_n78A-n257J | CA_n78A-n257A | 0 | Yunsung Kim, LG Uplus | kimy0011@lguplus.co.kr | LG Electronics, Huawei, HiSilicon | new | (completed) DL_n78A-n257J_UL_n78A (completed) DL_n78A-n257J_UL_n257A |
| CA_n78A-n257K | CA_n78A-n257A | 0 | Yunsung Kim, LG Uplus | kimy0011@lguplus.co.kr | LG Electronics, Huawei, HiSilicon | new | (completed) DL_n78A-n257K_UL_n78A (completed) DL_n78A-n257K_UL_n257A |
| CA_n78A-n257L | CA_n78A-n257A | 0 | Yunsung Kim, LG Uplus | kimy0011@lguplus.co.kr | LG Electronics, Huawei, HiSilicon | new | (completed) DL_n78A-n257L_UL_n78A (completed) DL_n78A-n257L_UL_n257A |
| CA_n78A-n257M | CA_n78A-n257A | 0 | Yunsung Kim, LG Uplus | kimy0011@lguplus.co.kr | LG Electronics, Huawei, HiSilicon | new | (completed) DL_n78A-n257M_UL_n78A (completed) DL_n78A-n257M_UL_n257A |

2. Text Proposal

-------------------------- Start of the TP --------------------------

8.9	CA_n78-n257

8.9.1	Common for 1 band UL and 2 bands UL CA

8.9.1.1		Operating bands for CA

Table 8.9.1.1-1: CA band combination of band n78+n257

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.9.1.2		Channel bandwidths per operating band for CA

Table 8.9.1.2-1: Supported bandwidths per CA band combination of band n78+n257

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n78A-n257G | CA_n78A-n257A | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257H | CA_n78A-n257A | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257I | CA_n78A-n257A | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257J | CA_n78A-n257A | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257K | CA_n78A-n257A | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257L | CA_n78A-n257A | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257M | CA_n78A-n257A | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

8.9.1.3		UE co-existence studies

The studies for 1 band UL for the CA band combination of band n78 + n257 have been already completed and captured into TR 37.865-01-01. It is expected that only 7th order harmonic of Band n78 UL may fall into Band n257 DL according to TR 37.865-01-01.

8.9.1.4		∆TIB and ∆RIB values

For CA_n78-n257, the TIB,c and RIB,c values are already specified in TR37.865-01-01.

8.9.1.5 		REFSENS requirements

The studies for 1 band UL for the CA band combination of band n78 + n257 have been already completed and captured into TR 37.865-01-01. There are no need for additional REFSENS requirements for 1 band UL according to TR 37.865-01-01.

8.9.2	Specific for 2 bands UL CA

8.9.2.1		UE co-existence studies

The studies for 2 band UL for the CA band combination of band n78 + n257 have been already completed and captured into TR 37.866-00-02.

8.9.2.2		REFSENS requirements

The studies for 2 band UL for the CA band combination of band n78 + n257 have been already completed and captured into TR 37.866-00-02. There are no need for additional REFSENS requirements for 2 band UL according to TR 37.866-00-02.

-------------------------- End of the TP --------------------------