3GPP TSG-RAN WG4 Meeting#91	R4-1907456

Reno, US, 13th– 17th May, 2019

Agenda Item:	9.10.2

Source:	CATT

Title:	TP for 38.716-03-01: UE requirements for CA_n3A-n41A-n79A

Document for:	Approval

- Introduction

This contribution provides a text proposal on UE requirements for inter-band CA_n3A-n41A-n79A.

- Discussion

The followings are proposed for this combination

- ∆TIB,c and ∆RIB,c values are derived based those values for the 3 fall back modes. If the ∆TIB,c and ∆RIB,c values for a specific bands are difference between different combinations, the largest value will be used for this combination.

- Due to cross band isolation issues, sensitivity degradation is allowed for Band n3 when there is uplink transmission in Band n41.

- Conclusion

It is proposed to approve the text proposal provided in the annex.

- Annex

----- Start of TP -----

6.1	CA_n3A-n41A-n79A

- 6.1.1	Operating bands for CA

Table 6.1.1-1: 3DL Inter-band CA operating bands

| sNR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n41-n79 | n3 | 1710MHz | – | 1780MHz | 1805MHz | – | 1880MHz | FDD |
|  | n41 | 2496MHz | – | 2690MHz | 2496MHz | – | 2690MHz | TDD |
|  | n79 | 4400MHz | – | 5000MHz | 4400MHz | – | 5000MHz | TDD |

- 6.1.2	Channel bandwidths per operating band for CA

Table 6.1.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n41A-n79A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| CA_n3A-n41A-n79A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 1 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of  TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

- 6.1.3	Co-existence studies

Table 6.1.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. It can be seen that the 3rd order harmonic of Band n3 will fall into a small portion of Band n79.

Table 6.1.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 |  |  |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |  |  |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 1320 | 15000 |  |  |

Table 6.1.3-2 gives harmonic mixing issue for CA with Band n3, n41 and n79. It is seen that a small part of Band n79 Tx frequency coincides with Band n41x3.

Table 6.1.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 |  |  |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |  |  |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 1320 | 15000 |  |  |

For single uplink, the UE coexistence is already considered for these bands in TS 38.101-1 [3].

- 6.1.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band X, Y and Z, the TIB,c and RIB,c  values are shown in table 6.1.4-1 and  table 6.1.4-2, respectively.

Table 6.1.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3A-n41A-n79A | n3 | 0.3 |
|  | n41 | 0.31 |
|  |  | 0.82 |
|  | n79 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. |  |  |

Table 6.1.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3A-n41A-n79A | n3 | 0 |
|  | n41 | 0.5 |
|  | n79 | 0.5 |

- 6.1.5	REFSENS requirements

For this combination, sensitivity degradation is allowed for Band n3 when uplink transmission ocure in Band n41 due to cross band isolation issues. The reference sensitivity exceptions are specified in Table 6.1.5-1 with the uplink configuration specified in Table 6.1.5-2.

# Table 6.1.5-1: MSD due to cross band isolation for CA CA_n3A-n41A-n79A

| NR CA Configuration | NR UL band | NR DL band | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n3A-n41A-n79A | n3 | n41 |  | 0.7 | 0.7 | 0.7 |  |  | 0.7 | 0.7 | 0.7 | 0.7 |  | 0.7 |
|  | n41 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | - | - | - | - | - | - |

# Table 6.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR UL band | NR DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n3 | n41 | 15kHz |  | 501 | 501 | 501 |  |  | 501 | 501 | 501 | 501 |  | 501 |
| n41 | n3 | 30 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

----- Start of TP -----