3GPP TR 37.716-41-11 V0.3.0 (2019-08)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of 4 bands LTE inter-band Carrier aggregation (CA) (4 Down Link (DL) / 1 Up Link (UL)) and 1 NR band (1 DL / 1 UL)

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

§§
Contents

1	Scope	5

2	References	15

3	Definitions, symbols and abbreviations	15

3.1	Definitions	15

3.2	Symbols	16

3.3	Abbreviations	16

4	Background	16

4.1	TR Maintenance	16

5	EN-DC of 4 LTE band (4DL/1UL) + 1 NR band: Specific Band Combination Part	16

5.1	Inter-band EN-DC within FR1	16

5.1.1	Eksample DC_1A-2A-3A-4A_n5A	16

5.1.2	DC_1-3-41-42_n78	17

5.1.3	DC_1-3-18-42_n78	18

5.1.4	DC_3-19-21-42_n77	19

5.1.5	DC_3-19-21-42_n78	20

5.1.6	DC_3-19-21-42_n79	21

5.1.7	DC_1-3-41-42_n79	22

5.1.8	DC_1-3-18-42_n79	23

5.1.9	DC_1-3-41-42_n77	24

5.1.10	DC_1-3-18-42_n77	25

5.1.11	1-3-7-28_n78	26

5.1.12	DC_1-3-5-41_n79	27

5.2	Inter-band EN-DC including FR2	28

5.2.1	Eksample DC_1A-2A-3A-4A_n257A	28

5.2.2	DC_2A-12A-30A-66A_n260	29

5.2.3	DC_2A-5A-30A-66A_n260	30

5.2.4	DC_1-3-41-42_n257	31

5.2.5	DC_1-3-18-42_n257	33

5.2.6	DC_1A-3A-5A-7A_n257	35

5.2.6	DC_1A-3A-5A-7A-7A_n257	35

5.2.7	DC_3-19-21-42_n257	36

Annex A: Change history	38

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for Dual Connectivity (EN-DC) of 4 LTE bands (4DL/1UL) and 1 NR band (1DL/1UL) under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address Dual Connectivity (EN-DC) of 4 LTE band (4DL/1UL) and 1 NR band (1DL/1UL) for the Rel-16 band combinations. in Table 1-1 and Table 1-2. The co-existence analysis and RF front end requirements such as Delta RIB,C and TIB,C are described based on the band combination basis since such information have no difference between the EN-DC configulations consisting with the same E-UTRA band and the same NR band.

Table 1-1: Release 16 EN-DC of 4 LTE band (4DL/1UL) and 1 NR band (1DL/1UL) within FR1

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-41A-42A_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-41A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-41C-42A_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-41C-42C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-18A-42A_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_3A-19A-21A-42A_n77A | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A-42A_n77C | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A-42C_n77A | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A-42C_n77C | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A-42A_n78A | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A-42A_n78C | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A-42C_n78A | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A-42C_n78C | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A-42A_n79A | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-21A-42A_n79C | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-21A-42C_n79A | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-21A-42C_n79C | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-3A-41A-42A_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-41A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-41C-42A_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-41C-42C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-18A-42A_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DC_1A-3A-18A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DC_1A-3A-41A-42A_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41C-42A_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41C-42C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-18A-42A_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-7A-28A_n78A DC_1A-3A-7C-28A_n78A DC_1A-3C-7A-28A_n78A DC_1A-3C-7C-28A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-5A-41A_n79A | DC_1A_n79A DC_3A_n79A DC_5A_n79A DC_41A_n79A |

Table 1-2: Release 16 EN-DC of 4 LTE band (4DL/1UL) and 1 NR band (1DL/1UL) including FR2

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-12A-30A-66A_n260 | DC_2A_n260A DC_12A_n260A DC_30A_n260A DC_66A_n260A |
| DC_2A-5A-30A-66A_n260 | DC_2A_n260A DC_5A_n260A DC_30A_n260A DC_66A_n260A |
| DC_1A-3A-41A-42A_n257A | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-41A-42A_n257F | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-41A-42A_n257M | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-41A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-41A-42C_n257F | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-41A-42C_n257M | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-41C-42A_n257A | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-41C-42A_n257F | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-41C-42A_n257M | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-41C-42C_n257A | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-41C-42C_n257F | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-41C-42C_n257M | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-18A-42A_n257A | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A |
| DC_1A-3A-18A-42A_n257F | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A |
| DC_1A-3A-18A-42A_n257M | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A |
| DC_1A-3A-18A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A |
| DC_1A-3A-18A-42C_n257F | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A |
| DC_1A-3A-18A-42C_n257M | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A |
| DC_1A-3A-5A-7A_n257D DC_1A-3A-5A-7A_n257E DC_1A-3A-5A-7A_n257F DC_1A-3A-5A-7A_n257G DC_1A-3A-5A-7A_n257H DC_1A-3A-5A-7A_n257I DC_1A-3A-5A-7A_n257J DC_1A-3A-5A-7A_n257K DC_1A-3A-5A-7A_n257L DC_1A-3A-5A-7A_n257M | DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A |
| DC_1A-3A-5A-7A-7A_n257D DC_1A-3A-5A-7A-7A_n257E DC_1A-3A-5A-7A-7A_n257F DC_1A-3A-5A-7A-7A_n257G DC_1A-3A-5A-7A-7A_n257H DC_1A-3A-5A-7A-7A_n257I DC_1A-3A-5A-7A-7A_n257J DC_1A-3A-5A-7A-7A_n257K DC_1A-3A-5A-7A-7A_n257L DC_1A-3A-5A-7A-7A_n257M | DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A |
| DC_3A-19A-21A-42A_n257A | DC_3A_n257A DC_19A_n257A DC_21A_n257A |
| DC_3A-19A-21A-42A_n257D | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D |
| DC_3A-19A-21A-42A_n257E | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D |
| DC_3A-19A-21A-42A_n257F | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D |
| DC_3A-19A-21A-42C_n257A | DC_3A_n257A DC_19A_n257A DC_21A_n257A |
| DC_3A-19A-21A-42C_n257D | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D |
| DC_3A-19A-21A-42C_n257E | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D |
| DC_3A-19A-21A-42C_n257F | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D |

This TR contains only band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Background

The present document is a technical report for Dual Connectivity (EN-DC) of 4 LTE band (4DL/1UL) and 1 NR band (4DL/1UL) under Rel-16 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	EN-DC of 4 LTE band (4DL/1UL) + 1 NR band: Specific Band Combination Part

<Editor’s note: The requirements for specific band combinations shall be described according to the same mannter as specified in TS38.101-3.>

## 5.1	Inter-band EN-DC within FR1

### 5.1.1	Eksample DC_1A-2A-3A-4A_n5A

#### 5.1.1.1	Operating bands for EN-DC

### <Editor’s note: If you need a note use same note numbering as in TS 38-101-3>

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-2-3-4_n5 | CA_1-2-3-4 | n5 |  |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.1.1.2	Configuration for EN-DC

### <Editor’s note: If you need a note use same note numbering as in TS 38-101-3>

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-2A-3A-4A_n5A | DC_1A_n5A DC_2A_n5A DC_3A_n5A DC_4A_n5A | CA_1A-2A-3A-4A | n5A |

#### 5.1.1.3	∆TIB and ∆RIB values

<Editor’s note: rows in ΔTIB,c andΔRIB tables shall be deleted if ΔTIB,c andΔRIB values are zero. >

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-2-3-4_n5 | 1 |  |
|  | 2 |  |
|  | 3 |  |
|  | 4 |  |
|  | n5 |  |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-2-3-4_n5 | 1 |  |
|  | 2 |  |
|  | 3 |  |
|  | 4 |  |
|  | n5 |  |

### 5.1.2	DC_1-3-41-42_n78

#### 5.1.2.1	Operating bands for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-41-42_n78 | CA_1-3-41-42 | n78 | DC_3_n78 |

#### 5.1.2.2	Configuration for EN-DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-41A-42A_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A | CA_ 1A-3A-41A-42A | n78A |
| DC_1A-3A-41A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A | CA_ 1A-3A-41A-42C | n78A |
| DC_1A-3A-41C-42A_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A | CA_ 1A-3A-41C-42A | n78A |
| DC_1A-3A-41C-42C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A | CA_ 1A-3A-41C-42C | n78A |

#### 5.1.2.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC(five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

### 5.1.3	DC_1-3-18-42_n78

#### 5.1.3.1	Operating bands for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-18-42_n78 | CA_1-3-18-42 | n78 | DC_3_n78 |

#### 5.1.3.2	Configuration for EN-DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-18A-42A_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A | CA_ 1A-3A-18A-42A | n78A |
| DC_1A-3A-18A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A | CA_ 1A-3A-18A-42C | n78A |

#### 5.1.3.3	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of DC_1-3-19-42_n78 can be applied.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC(five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

### 5.1.4	DC_3-19-21-42_n77

#### 5.1.4.1	Operating bands for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-19-21-42_n77 | CA_3-19-21-42 | n77 | DC_3_n77 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.1.4.2	Configuration for EN-DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-19A-21A-42A_n77A | DC_3A_n77A DC_19A_n77A DC_21A_n77A | CA_3A-19A-21A-42A | n77A |
| DC_3A-19A-21A-42A_n77C | DC_3A_n77A DC_19A_n77A DC_21A_n77A | CA_3A-19A-21A-42A | CA_n77C |
| DC_3A-19A-21A-42C_n77A | DC_3A_n77A DC_19A_n77A DC_21A_n77A | CA_3A-19A-21A-42C | n77A |
| DC_3A-19A-21A-42C_n77C | DC_3A_n77A DC_19A_n77A DC_21A_n77A | CA_3A-19A-21A-42C | CA_n77C |

#### 5.1.4.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-21-42_n77 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-21-42_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

### 5.1.5	DC_3-19-21-42_n78

#### 5.1.5.1	Operating bands for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-19-21-42_n78 | CA_3-19-21-42 | n78 | DC_3_n78 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.1.5.2	Configuration for EN-DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-19A-21A-42A_n78A | DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_3A-19A-21A-42A | n78A |
| DC_3A-19A-21A-42A_n78C | DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_3A-19A-21A-42A | CA_n78C |
| DC_3A-19A-21A-42C_n78A | DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_3A-19A-21A-42C | n78A |
| DC_3A-19A-21A-42C_n78C | DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_3A-19A-21A-42C | CA_n78C |

#### 5.1.5.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-21-42_n78 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-21-42_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

### 5.1.6	DC_3-19-21-42_n79

#### 5.1.6.1	Operating bands for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-19-21-42_n79 | CA_3-19-21-42 | n79 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.1.6.2	Configuration for EN-DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-19A-21A-42A_n79A | DC_3A_n79A DC_19A_n79A DC_21A_n79A | CA_3A-19A-21A-42A | n79A |
| DC_3A-19A-21A-42A_n79C | DC_3A_n79A DC_19A_n79A DC_21A_n79A | CA_3A-19A-21A-42A | CA_n79C |
| DC_3A-19A-21A-42C_n79A | DC_3A_n79A DC_19A_n79A DC_21A_n79A | CA_3A-19A-21A-42C | n79A |
| DC_3A-19A-21A-42C_n79C | DC_3A_n79A DC_19A_n79A DC_21A_n79A | CA_3A-19A-21A-42C | CA_n79C |

#### 5.1.6.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-21-42_n79 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-21-42_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |

### 5.1.7	DC_1-3-41-42_n79

#### 5.1.7.1	Operating bands for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-41-42_n79 | CA_1-3-41-42 | n79 | No |

#### 5.1.7.2	Configuration for EN-DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-41A-42A_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A | CA_ 1A-3A-41A-42A | n79A |
| DC_1A-3A-41A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A | CA_ 1A-3A-41A-42C | n79A |
| DC_1A-3A-41C-42A_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A | CA_ 1A-3A-41C-42A | n79A |
| DC_1A-3A-41C-42C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A | CA_ 1A-3A-41C-42C | n79A |

#### 5.1.7.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n79 | 0 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |

### 5.1.8	DC_1-3-18-42_n79

#### 5.1.8.1	Operating bands for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-18-42_n79 | CA_1-3-18-42 | n79 | No |

#### 5.1.8.2	Configuration for EN-DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-18A-42A_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A | CA_ 1A-3A-18A-42A | n79A |
| DC_1A-3A-18A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A | CA_ 1A-3A-18A-42C | n79A |

#### 5.1.8.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n79 | 0 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |

### 5.1.9	DC_1-3-41-42_n77

#### 5.1.9.1	Operating bands for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-41-42_n77 | CA_1-3-41-42 | n77 | DC_1_n77 DC_3_n77 |

#### 5.1.9.2	Configuration for EN-DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-41A-42A_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A | CA_ 1A-3A-41A-42A | n77A |
| DC_1A-3A-41A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A | CA_ 1A-3A-41A-42C | n77A |
| DC_1A-3A-41C-42A_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A | CA_ 1A-3A-41C-42A | n77A |
| DC_1A-3A-41C-42C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A | CA_ 1A-3A-41C-42C | n77A |

#### 5.1.9.3	∆TIB and ∆RIB values

The same values of DC_1-3-41-42_n78 can be applied.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

### 5.1.10	DC_1-3-18-42_n77

#### 5.1.10.1	Operating bands for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-18-42_n77 | CA_1-3-18-42 | n77 | DC_1_n77 DC_3_n77 |

#### 5.1.10.2	Configuration for EN-DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-18A-42A_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A | CA_ 1A-3A-18A-42A | n77A |
| DC_1A-3A-18A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A | CA_ 1A-3A-18A-42C | n77A |

#### 5.1.10.3	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of DC_1-3-18-42_n78 can be applied.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

### 5.1.11	1-3-7-28_n78

#### 5.1.11.1	Operating bands for DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-7-28_n781 | CA_1-3-7-28 | n78 | DC_3_n78 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.1.11.2	Configuration for DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-7A-28A_n78A DC_1A-3A-7C-28A_n78A DC_1A-3C-7A-28A_n78A DC_1A-3C-7C-28A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A | CA_1A-3A-7A-28A CA_1A-3A-7C-28A CA_1A-3C-7A-28A CA_1A-3C-7C-28A | n78A |

#### 5.1.11.3	∆TIB and ∆RIB values

For DC_1-3-7-28_n78 the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

#### 5.1.11.4	REFSENS requirements

No further MSD are needed to be specified for DC_1A-3A-7A-28A_n78A, DC_1A-3A-7C-28A_n78A, DC_1A-3C-7A-28A_n78A and DC_1A-3C-7C-28A_n78A.

### 5.1.12	DC_1-3-5-41_n79

#### 5.1.12.1	Operating bands for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-5-41_n79 | CA_1-3-5-41 | n79 | No |

#### 5.1.12.2	Configurations for EN-DC

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A-41A_n79A | DC_1A_n79A DC_3A_n79A DC_5A_n79A DC_41A_n79A | CA_1A-3A-5A-41A | n79A |

#### 5.1.12.3	∆TIB and ∆RIB values

For DC_1A-3A-5A-41A_n79A, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-5-41_n79 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 41 | 0.51 |
|  |  | 0.82 |
|  | n79 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.3B.3.3.4-1: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB, c [dB] |
| --- | --- | --- |
| DC_1-3-5-41_n79 | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n79 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

## 5.2	Inter-band EN-DC including FR2

### 5.2.1	Eksample DC_1A-2A-3A-4A_n257A

#### 5.2.1.1	Operating bands for EN-DC

### <Editor’s note: If you need a note use same note numbering as in TS 38-101-3>

Table 5.2B.5.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-2-3-4_n257 | CA_1-2-3-4 | n257 |  |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.2.1.2	Configuration for DC

### <Editor’s note: If you need a note use same note numbering as in TS 38-101-3>

Table 5.5B.5.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-2A-3A-4A_n257A | DC_1A_n257A DC_2A_n257A DC_3A_n257A DC_4A_n257A | CA_1A-2A-3A-4A | n257A |

#### 5.2.1.3	∆TIB and ∆RIB values

<Editor’s note: rows in ΔTIB,c andΔRIB tables shall be deleted if ΔTIB,c andΔRIB values are zero. >

Table 6.2B.4.2.4.4-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-2-3-4_n257 | 1 |  |
|  | 2 |  |
|  | 3 |  |
|  | 4 |  |
|  | n257 |  |

Table 7.3B.3.4.4-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-2-3-4_n257 | 1 |  |
|  | 2 |  |
|  | 3 |  |
|  | 4 |  |
|  | n257 |  |

### 5.2.2	DC_2A-12A-30A-66A_n260

#### 5.2.2.1	Operating bands for EN-DC

Table 5.2B.5.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-12-30-66_n260 | CA_2-12-30-66 | n260 |  |

#### 5.2.2.2	Configuration for DC

Table 5.5B.5.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-12A-30A-66A_n260 | DC_2A_n260A DC_12A_n260A DC_30A_n260A DC_66A_n260A | CA_2A-12A-30A-66A | n260A |

#### 5.2.2.3		∆TIB and ∆RIB values

Table 6.x.4-1: ΔTIB,c

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12-30-66_n260 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| EN-DC band | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12-30-66_n260 | 2 | 0.4 |
|  | 12 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n260 | 0 |

### 5.2.3	DC_2A-5A-30A-66A_n260

#### 5.2.3.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5-30-66_n260 | CA_2-5-30-66 | n260 |  |

#### 5.2.3.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-5A-30A-66A_n260 | DC_2A_n260A DC_5A_n260A DC_30A_n260A DC_66A_n260A | CA_2A-5A-30A-66A | n260A |

#### 5.2.3.3		∆TIB and ∆RIB values

Table 6.x.4-1: ΔTIB,c

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-30-66_n260 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| EN-DC band | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-30-66_n260 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n260 | 0 |

### 5.2.4	DC_1-3-41-42_n257

#### 5.2.4.1	Operating bands for EN-DC

Table 5.2B.5.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-41-42_n257 | CA_1-3-41-42 | n257 |  |

#### 5.2.4.2	Configuration for EN-DC

Table 5.5B.5.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-41A-42A_n257A | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41A-42A | n257A |
| DC_1A-3A-41A-42A_n257F | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41A-42A | n257A |
| DC_1A-3A-41A-42A_n257M | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41A-42A | n257A |
| DC_1A-3A-41A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41A-42C | n257A |
| DC_1A-3A-41A-42C_n257F | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41A-42C | n257A |
| DC_1A-3A-41A-42C_n257M | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41A-42C | n257A |
| DC_1A-3A-41C-42A_n257A | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41C-42A | n257A |
| DC_1A-3A-41C-42A_n257F | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41C-42A | n257A |
| DC_1A-3A-41C-42A_n257M | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41C-42A | n257A |
| DC_1A-3A-41C-42C_n257A | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41C-42C | n257A |
| DC_1A-3A-41C-42C_n257F | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41C-42C | n257A |
| DC_1A-3A-41C-42C_n257M | DC_1A_n257A DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 1A-3A-41C-42C | n257A |

#### 5.2.4.3	∆TIB and ∆RIB values

Table 6.2B.4.2.4.4-1: ΔTIB,c due to EN-DC(five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n257 | 0 |

Table 7.3B.3.4.4-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | 42 | 0.5 |
|  | n257 | 0 |

### 5.2.5	DC_1-3-18-42_n257

#### 5.2.5.1	Operating bands for EN-DC

Table 5.2B.5.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-18-42_n257 | CA_1-3-18-42 | n257 |  |

#### 5.2.5.2	Configuration for EN-DC

Table 5.5B.5.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-18A-42A_n257A | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A | CA_ 1A-3A-18A-42A | n257A |
| DC_1A-3A-18A-42A_n257F | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A | CA_ 1A-3A-18A-42A | n257A |
| DC_1A-3A-18A-42A_n257M | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A | CA_ 1A-3A-18A-42A | n257A |
| DC_1A-3A-18A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A | CA_ 1A-3A-18A-42C | n257A |
| DC_1A-3A-18A-42C_n257F | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A | CA_ 1A-3A-18A-42C | n257A |
| DC_1A-3A-18A-42C_n257M | DC_1A_n257A DC_3A_n257A DC_18A_n257A DC_42A_n257A | CA_ 1A-3A-18A-42C | n257A |

#### 5.2.5.3	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of DC_1-3-19-42_n257 can be applied.

Table 6.2B.4.2.4.4-1: ΔTIB,c due to EN-DC(five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-42_n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n257 | 0 |

Table 7.3B.3.4.4-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18-42_n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | 42 | 0.5 |
|  | n257 | 0 |

### 5.2.6	DC_1A-3A-5A-7A_n257

#### 5.2.6.1	Operating bands for DC

Table 5.2B.5.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-5-7_n257 | CA_1-3-5-7 | n257 | No |

#### 5.2.6.2	Configuration for DC

Table 5.5B.5.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A-7A_n257D DC_1A-3A-5A-7A_n257E DC_1A-3A-5A-7A_n257F DC_1A-3A-5A-7A_n257G DC_1A-3A-5A-7A_n257H DC_1A-3A-5A-7A_n257I DC_1A-3A-5A-7A_n257J DC_1A-3A-5A-7A_n257K DC_1A-3A-5A-7A_n257L DC_1A-3A-5A-7A_n257M | DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | CA_1A-3A-5A-7A | CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

#### 5.2.6.3	∆TIB and ∆RIB values

Table 6.2B.4.2.4.4-1: ΔTIB,c due to EN-DC(five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-5-7_n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
|  | n257 | 0 |

Table 7.3B.3.4.3-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-5-7_n257 | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

### 5.2.6	DC_1A-3A-5A-7A-7A_n257

#### 5.2.6.1	Operating bands for DC

Table 5.2B.5.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-5-7-7_n257 | CA_1-3-5-7-7 | n257 | No |

#### 5.2.6.2	Configuration for DC

Table 5.5B.5.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A-7A-7A_n257D DC_1A-3A-5A-7A-7A_n257E DC_1A-3A-5A-7A-7A_n257F DC_1A-3A-5A-7A-7A_n257G DC_1A-3A-5A-7A-7A_n257H DC_1A-3A-5A-7A-7A_n257I DC_1A-3A-5A-7A-7A_n257J DC_1A-3A-5A-7A-7A_n257K DC_1A-3A-5A-7A-7A_n257L DC_1A-3A-5A-7A-7A_n257M | DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | CA_1A-3A-5A-7A-7A | CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

#### 5.2.6.3	∆TIB and ∆RIB values

Table 6.2B.4.2.4.4-1: ΔTIB,c due to EN-DC(five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3A-5-7-7_n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
|  | n257 | 0 |

Table 7.3B.3.4.3-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-5-7-7_n257 | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

### 5.2.7	DC_3-19-21-42_n257

#### 5.2.7.1	Operating bands for EN-DC

Table 5.2B.5.4-1: Band combinations EN-DC (five bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-19-21-42_n257 | CA_3-19-21-42 | n257 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.2.7.2	Configuration for EN-DC

Table 5.5B.5.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-19A-21A-42A_n257A | DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_3A-19A-21A-42A | n257A |
| DC_3A-19A-21A-42A_n257D | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D | CA_3A-19A-21A-42A | CA_n257D |
| DC_3A-19A-21A-42A_n257E | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D | CA_3A-19A-21A-42A | CA_n257E |
| DC_3A-19A-21A-42A_n257F | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D | CA_3A-19A-21A-42A | CA_n257F |
| DC_3A-19A-21A-42C_n257A | DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_3A-19A-21A-42C | n257A |
| DC_3A-19A-21A-42C_n257D | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D | CA_3A-19A-21A-42C | CA_n257D |
| DC_3A-19A-21A-42C_n257E | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D | CA_3A-19A-21A-42C | CA_n257E |
| DC_3A-19A-21A-42C_n257F | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D | CA_3A-19A-21A-42C | CA_n257F |

#### 5.2.7.3	∆TIB and ∆RIB values

Table 6.2B.4.2.4.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-21-42_n257 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |

Table 7.3B.3.4.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-21-42_n257 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-08 | 3GPP RAN4#88 | R4-1811505 |  |  |  | Initial TR skeleton | 0.0.1 |
| 2018-10 | 3GPP RAN4#88 | R4-1811507 R4-1811508 R4-1811509 R4-1811510 |  |  |  | TP for TR 37.716-41-11 DC of 3-19-21-42_n77 TP for TR 37.716-41-11 DC of 3-19-21-42_n78 TP for TR 37.716-41-11 DC of 3-19-21-42_n79 TP for TR 37.716-41-11 DC of 3-19-21-42_n257 | 0.1.0 |
|  | 3GPP RAN4#88bis | R4-1813777 R4-1813778 R4-1812805 R4-1812806 R4-1812891 R4-1812909 R4-1813765 |  |  |  | TP for TR 37.716-41-11: DC_1-3-41-42_n78 TP for TR 37.716-41-11: DC_1-3-18-42_n78 TP for TR 37.716-41-11: 2A-12A-30A-66A_n260 TP for TR 37.716-41-11: 2A-5A-30A-66A_n260 TP for TR 37.716-41-11: DC_1-3-41-42_n257 TP for TR 37.716-41-11: DC_1-3-18-42_n257 TP for TR 37.716-41-11 DC_4Bands_n257 | 0.1.0 |
| 2019-02 | 3GPP RAN4#89 | R4-1814939 R4-1814945 R4-1816164 R4-1816165 R4-1816174 R4-1816225 |  |  |  | TP for TR 37.716-41-11: DC_1-3-41-42_n79 TP for TR 37.716-41-11: DC_1-3-18-42_n79 TP for TR 37.716-41-11: DC_1-3-41-42_n77 TP for TR 37.716-41-11: DC_1-3-18-42_n77 Addition of EN-DC configuration DC_1A-3A-7A-28A_n78A, DC_1A-3A-7C-28A_n78A and DC_1A-3C-7A-28A_n78A DC band combination of Band 1, 3, 5, 41 and n79 | 0.2.0 |
| 2019-08 | 3GPP RAN4#92 | R4-1904454 |  |  |  | Update to table containing combinations in this TR TP to TR 37.716-41-11: Addition of EN-DC configuration DC_1A-3C-7C-28A_n78A | 0.2.0 |