3GPP TSG-RAN WG4 Meeting # 91	R4-1907643

# Reno, Nevada, USA,  13 - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0042 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02: updating the FR2 OTA transmit ON/OFF column in Table 12.10.1-1 (12.10.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 13-05-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This is a revision of R4-1906955 in which the column for FR2 OTA TX ON/OFF is removed. During the RAN4 #90bis meeting, conclusions were reached on OTA testing methodology for FR2 transmitter OFF power and transient period in which the test requirement is verified by EIRP measurements in the beam peak direction within a reference beam direction pair. This is a revision of R4-1906955 following online discussions and comments. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The FR2 OTA transmit ON/OFF column in Table 12.10.1-1 is updated to reflect the agreement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Table 12.10.1-1 captures incorrect OTA test procedures for the FR2 OTA transmit ON/OFF requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-1906955 |  |  |  |  |  |  |  |  |

<Start of modified section>

### 12.10.1	General

Refer to subclause 10.8 in TR 37.843 [26] for further details.

A summary of TRP measurement procedures and their applicability to different OTA BS requirements is shown in Table 12.10.1-1. Note, OTA FR2 transmit ON/OFF power is excluded from the table although the core requirement is specified as TRP because conformance is verified through EIRP measurements. Refer to subclause 9.5 for more details.

Table 12.10.1-1 - Applicability of TRP measurement methods

|  | OTA base station output power |  | OTA unwanted emissions: Adjacent channel leakage ratio (ACLR) (Note 1) | OTA Operating band unwanted emission (OBUE) | OTA transmitter spurious emissions OTA (Note 2) |
| --- | --- | --- | --- | --- | --- |
| Full sphere using reference steps (accurate) | X |  | X | X | X |
| Full sphere using sparse sampling (overestimate) |  |  |  |  | X (Note 3) |
| Two cuts + Pattern multiplication (accurate) (Note 4) | X |  | X | X |  |
| Two/three cuts (overestimate) |  |  | X | X | X (Note 3) |
| Beam-based directions | X |  | X | X |  |
| Peak method |  |  |  | X | X |
| Equal sector with peak average |  |  |  | X | X |
| Pre-scan |  |  | X | X | X |
| NOTE 1: Two TRP measurements are needed. NOTE 2: Pre-scan is needed to identify the frequencies of interest. Pre-scan can also be applied to ACLR, OBUE and SEM. NOTE 3: At harmonic frequencies the use of this method is FFS due to risk of high beamforming gain NOTE 4: Pattern multiplication is conditional NOTE: If box is blank the method is not excluded but the methodology has not been described in subclause 10.8, TR 37.843; if a suitable analysis is shown the method may be applied. |  |  |  |  |  |

The summation error is based on a reasonable number of test directions and the minimum beam width, which is in turn dependent on antenna size and frequency, i.e., the reference steps see Figure 10.8.2.2-1 in TR 37.843 [26]. It has been agreed that a reasonable trade-off between accuracy and sampling is achieved when the SE is 0,75 dB for FR1 BS (which is adopted from OTA AAS BS) and 1,2 dB for FR2 BS. For FR2 BS, the beams are expected to be narrower than an FR1 BS and hence the SE is higher.

<End of modified section>