3GPP TSG-RAN WG4 Meeting #91	R4-190xxxx

# Reno, US, 13th– 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.113 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.113 (subclause 3.1,3.2,3.3,6.1,6.2,9.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some abbreviations were missed. Some texts are not suitable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct some texts. Also correct and add some abbreviations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Abbreviations and symbols may be incongruous symbols with TS 38.104. During the immunity test, the “mobile” simulator instead by UE Simulator. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1,3.2,3.3,6.1,6.2,9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------------Start of first change-------------

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

ancillary equipment: electrical or electronic equipment, that is intended to be used with a receiver or transmitter

NOTE:	It is considered as an ancillary equipment if:

-	the equipment is intended for use with a receiver or transmitter to provide additional operational and/or control features to the radio equipment, (e.g. to extend control to another position or location); and

-	the equipment cannot be used on a stand-alone basis to provide user functions independently of a receiver or transmitter; and

-	the receiver or transmitter, to which it is connected, is capable of providing some intended operation such as transmitting and/or receiving without the ancillary equipment (i.e. it is not a sub-unit of the main equipment essential to the main equipment basic functions).

antenna port: for EMC purposes, port for connection of an antenna used for intentional transmission and/or reception of radiated RF energy, equivalent to an RF antenna connector/TAB connector in TS 37.105 [25]

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors.

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB.

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB.

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB.

channel bandwidth: the RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements

continuous phenomena: electromagnetic disturbance, the effects of which on a particular device or equipment cannot be resolved into a succession of distinct effects (IEC 60050-161 [9])

enclosure port: physical boundary of the equipment through which electromagnetic fields may radiate or impinge

NOTE:	In the case of integral antenna equipment, this port is inseparable from the antenna port.

exclusion band: frequency range(s) not subject to test or assessment

integral antenna: antenna designed for permanent connection to the equipment and considered part of the enclosure port

NOTE:	An integral antenna may be fitted internally or externally.

lower RF bandwidth edge: the frequency of the lower edge of the Base Station RF bandwidth, used as a frequency reference point for transmitter and receiver requirements

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

port: particular interface of EUT used for EMC requirements testing purposes

NOTE:	Any connection point on EUT intended for connection of cables to or from EUT during the EMC testing is considered as a port.

EXAMPLE 1:	Examples of ports for BS type 1-C and BS type 1-H are as presented in figure 3.1-1:

Figure 3.1-1: Examples of ports for BS type 1-C and BS type 1-H

EXAMPLE 2:	Examples of ports for BS type 1-O and BS type 2-O (i.e. with no antenna ports) are as presented in figure 3.1-2:

Figure 3.1-2: Examples of ports for BS type 1-O and BS type 2-O

receiver exclusion band: band of frequencies over which no tests of radiated immunity of a receiver are made, and expressed relative to the BS receive band

signal port: port intended for the interconnection of components of an EUT, or between an EUT and associated equipment and used in accordance with relevant functional specifications (for example for the maximum length of cable connected to it)

TAB connector: transceiver array boundary connector

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transceiver unit: active unit consisting of transmitter and/or receiver which transmits and/or receives radio signals, and which may include passive RF filters

telecommunication port: ports which are intended to be connected to telecommunication networks (e.g. public switched telecommunication networks, integrated services digital networks), local area networks (e.g. Ethernet, Token Ring) and similar networks

NOTE:	Telecommunication port is called "wired network port" in ETSI EN 301 489-1 [23]

transient phenomena: pertaining to or designating a phenomenon or a quantity which varies between two consecutive steady states during a time interval short compared with the time-scale of interest (IEC 60050-161 [9])

transmitter exclusion band: band of frequencies over which no tests of radiated immunity of a transmitter are made and is expressed relative to the carrier frequencies used (the carrier frequencies of the base stations activated transmitter(s))

upper RF bandwidth edge: the frequency of the upper edge of the Base Station RF bandwidth, used as a frequency reference point for transmitter and receiver requirements

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

β	Bandwidth

BWChannel	Channel bandwidth

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

FDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

FUL,low	The lowest frequency of the uplink operating band

FUL,high	The highest frequency of the uplink operating band

ΔfRIexclusion	Maximum offset of the Radiated Immunity exclusion band from the uplink operating band edge for test without spatial exclusion

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AC	Alternating Current

AMN	Artificial Mains Network

BC	Band Category

BS            Base Station

CA            Carrier Aggregation

CDN	Coupling/Decoupling Network

CS	Capability Set

DC	Direct Current

DL            Downlink

e.i.r.p          equivalent isotropic radiated power

EMC	Electromagnetic Compatibility

e.r.p           effective radiated power

ESD	Electrostatic Discharge

EUT	Equipment Under Test

FR	Frequency Range

FRC	Fixed Reference Channel

NC           Non Contiguous

NG           Next Generation

NGC          Next Generation Core

NR            New Radio

NR-ARFN      NR Absolute Radio Frequency Channel Number

NRTC	NR Test Configuration

NTC	Test Configuration for Non-contiguous operation

RAT	Radio Access Technology

RF	Radio Frequency

RIB	Radiated Interface Boundary

RMS	Root Mean Square

SC            Single Carrier

SDL	Supplementary Downlink

TC	Test Configuration

UE            User Equipment

UL            Uplink

--------------End of first change-------------

Unchanged Omitted

--------------Start of second change-------------

# 6	Performance criteria

## 6.1	Performance criteria for continuous phenomena for BS

The test should, where possible, be performed using a bearer with the characteristics of data rate and throughput defined in Table 6.1-1 and Table 6.1-2. If the test is not performed using one of these bearers (for example, of none of them are supported by the BS), the characteristics of the bearer used shall be recorded in the test report.

The throughput in Table 6.1-1 and Table 6.1-2 is stated relative to the maximum throughput of the FRC.

The BS uplink and downlink paths shall each meet the performance criteria defined in Table 6.1-1 and Table 6.1-2 during the test. If the Uplink and Downlink paths are evaluated as a one loop then the criteria is two times the throughput reduction shown in Table 6.1-1 and Table 6.1-2 (i.e. throughput > 90 % instead of throughput > 95 %). After each test case BS shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.1-1: FR1 erformance criteria for continuous phenomena for BS

| NRchannel bandwidth [MHz] | Sub-carrier spacing [kHz] | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 in Annex a.1 in TS 38.104 [2] | Throughput > 95 %, no loss of service |
| 10, 15 | 30 | G-FR1-A1-2 in Annex a.1 in TS 38.104 [2] |  |
| 10, 15 | 60 | G-FR1-A1-3 in Annex a.1 in TS 38.104 [2] |  |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 in Annex a.1 in TS 38.104 [2] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 in Annex a.1 in TS 38.104 [2] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 in Annex a.1 in TS 38.104 [2] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.1-2: FR2 performance criteria for continuous phenomena for BS

| NR channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 in Annex a.1 in TS 38.104 [2] | Throughput > 95 %, no loss of service |
| 50 | 120 | G-FR2-A1-2 in Annex a.1 in TS 38.104 [2] |  |
| 100, 200, 400 | 120 | G-FR2-A1-3 in Annex a.1 in TS 38.104 [2] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

## 6.2	Performance criteria for transient phenomena for BS

The test should be, where possible, performed using a bearer with the characteristics of data rate and throughput defined in Table 6.2-1 and Table 6.2-2. If the test is not performed using one of these bearers (for example, of none of them are supported by the BS), the characteristics of the bearer used shall be recorded.

The throughput in Table 6.2-1 and Table 6.2-2 is stated relative to the maximum throughput of the FRC.

The BS Uplink and Downlink paths shall each meet the performance criteria defined in Table 6.2-1 and Table 6.2-2 during the test. If the uplink and downlink paths are evaluated as a one loop then the criteria is two times the throughput reduction shown in Table 6.2-1 and Table 6.2-2 (i.e. throughput <90 % temporarily instead of throughput < 95 % temporarily). After each test case BS shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.2-1: FR1 performance criteria for transient phenomena for BS

| NR channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 in Annex a.1 in TS 38.104 [2] | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 10, 15 | 30 | G-FR1-A1-2 in Annex a.1 in TS 38.104 [2] |  |
| 10, 15 | 60 | G-FR1-A1-3 in Annex a.1 in TS 38.104 [2] |  |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 in Annex a.1 in TS 38.104 [2] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 in Annex a.1 in TS 38.104 [2] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 in Annex a.1 in TS 38.104 [2] |  |
| NOTE 1: The performance criteria, Throughput < 95 % temporarily, however the communication link shall be maintained, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, Throughput < 90 % temporarily, however the communication link shall be maintained, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.2-2: FR2 performance criteria for transient phenomena for BS

| NR channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 in Annex a.1 in TS 38.104 [2] | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 50 | 120 | G-FR2-A1-2 in Annex a.1 in TS 38.104 [2] |  |
| 100, 200, 400 | 120 | G-FR2-A1-3 in Annex a.1 in TS 38.104 [2] |  |
| NOTE 1: The performance criteria, Throughput < 95 % temporarily, however the communication link shall be maintained, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, Throughput < 90 % temporarily, however the communication link shall be maintained, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

--------------End of second change-------------

Unchanged Omitted

--------------Start of third change-------------

# 9	Immunity

## 9.1	Test configurations

This subclause defines the configurations for immunity tests as follows:

-	the equipment shall be tested under normal test conditions as specified in the functional standards;

-	during the test, the RF output power may be reduced to a power level sufficient for establishing and maintaining the required communication link;

-	the test configuration shall be as close to normal intended use as possible;

-	if the equipment is part of a system, or can be connected to ancillary equipment, then it shall be acceptable to test the equipment while connected to the minimum configuration of ancillary equipment necessary to exercise the ports;

-	if the equipment has a large number of ports, then a sufficient number shall be selected to simulate actual operation conditions and to ensure that all the different types of termination are tested;

-	the test conditions, test configuration and mode of operation shall be recorded in the test report;

-	ports which in normal operation are connected shall be connected to an ancillary equipment or to a representative piece of cable correctly terminated to simulate the input/output characteristics of the ancillary equipment. In case of BS type 1-C and BS type 1-H, antenna ports shall be correctly terminated.

-	ports which are not connected to cables during normal operation, e.g. service connectors, programming connectors, temporary connectors etc. shall not be connected to any cables for the purpose of EMC testing. Where cables have to be connected to these ports, or interconnecting cables have to be extended in length in order to exercise the EUT, precautions shall be taken to ensure that the evaluation of the EUT is not affected by the addition or extension of these cables;

-	immunity tests on the entire NR BS shall be performed by establishing communication links at the radio interface (e.g. with the UE simulator) and the NG interface (e.g. with an NGC simulator) and evaluating the throughput (see figures 9.1-1 and 9.1-2);

-	immunity tests shall be performed on both the uplink and downlink paths. The tests shall also include both the radio interface and the NG interface. Throughput evaluation may be carried out at either interface, where appropriate, and the measurements for the uplink and downlink paths may be carried out as a single path looped at either the radio interface or NG interface. In case of looping is used care have to be taken that the throughput information doesn't change due to looping;

-	for NR BS capable of multi-band operation, communication links shall be established in such a way that all operating band(s) are activated during the test according to the applicable test configurations in subclause 4.5. Performance assessment may be done separately for each operating band.

Figure 9.1-1: Communication link set up for BS type 1-C/BS type 1-H immunity measurement

Figure 9.1-2: Communication link set up for BS type 1-O/BS type 2-O immunity measurement

### 9.1.1	(Void)

### 9.1.2	(Void)

### 9.1.3	(Void)

### 9.1.4	(Void)

### 9.1.5	(Void)

## 9.2	RF electromagnetic field (80 MHz to 6000 MHz)

The test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

### 9.2.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the presence of a radio frequency electromagnetic field disturbance at the enclosure.

### 9.2.2	Test method and level

The test method shall be in accordance with IEC 61000-4-3 [18]:

-	For transmitters, receivers and transceivers the following requirements shall apply:

-	The test level shall be 3 V/m amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	The stepped frequency increments shall be 1 % of the momentary frequency;

-	The test shall be performed over the frequency range 80 MHz - 6000 MHz; with the exception of the exclusion band for receivers (see subclause 4.4);

-	Responses in stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see subclause 4.3;

-	The frequencies selected during the test shall be recorded in the test report.

-	In the range of angles except the operational range of angles of the BS type 1-O and 2-O antenna (i.e. except for the half sphere around the EUT radiating direction as depicted on figure 9.2.2-1) and for the frequency range above 690 MHz (according to the test method in ETSI EN 301 489-50 [28]), the EMC RF electromagnetic field immunity requirement applies.

Figure 9.2.2-1: EMC RF electromagnetic field immunity requirement testing directions for 1-O and 2-O BS (horizontal plane depicted)

### 9.2.3	Performance criteria

Base station:

The performance criteria of subclause 6.1 shall apply.

Ancillary equipment:

The performance criteria of subclause 6.3 shall apply.

--------------End of third change-------------