3GPP TSG-RAN WG4 Meeting #91	R4-1907741

Reno, US, 13th– 17th May, 2019

Agenda item:	6.12.1.2.2

Source: 		NTT DOCOMO, INC.

Title: 	Simulation results for SDR test in FR2

Document for:	Discussion

1. Introduction

This contribution presents our simulation results for SDR test case 10 (FR2, 120 kHz SCS) [1,2].

2. Simulation results

We follow simulation assumptions in Table. A1 and A2, which are identical to the agreed simulation assumptions of SDR test [1,2]. We show simulation results of case 10 (FR2, 120 kHz SCS) for MCS range agreed in RAN4 #90bis. Based on the results, we have following proposals.

Proposal:  For test case 10, alignment results and impairment results are as follows.

| Rank | MCS | Case 10 (FR2, 120 kHz SCS) |  |
| --- | --- | --- | --- |
|  |  | Alignment results | Impairment results |
| 2 | MCS13 | 5.7 | 7.4 |
| 2 | MCS14 | 6.8 | 8.5 |
| 2 | MCS15 | 7.7 | 9.4 |
| 2 | MCS16 | 8.4 | 10.1 |
| 2 | MCS17 | 9.0 | 11.0 |
| 2 | MCS18 | 9.6 | 11.6 |
| 2 | MCS19 | 10.6 | 12.6 |
| 2 | MCS20 | 11.6 | 13.6 |
| 2 | MCS21 | 12.5 | 14.5 |
| 2 | MCS22 | 13.7 | 15.7 |
| 2 | MCS23 |  |  |
| 2 | MCS24 | 15.8 | 17.9 |
| 2 | MCS25 | 17.2 | 19.3 |
| 2 | MCS26 | 18.9 | 21.0 |
| 2 | MCS27 | 20.3 | 22.4 |

3. Conclusion

This contribution presents our simulation results for SDR test for case 10 (FR2, 120 kHz SCS). Our proposals are presented as follows.

Proposal:  For test case 10, alignment results and impairment results is as follows.

| Rank | MCS | Case 10 (FR2, 120 kHz SCS) |  |
| --- | --- | --- | --- |
|  |  | Alignment results | Impairment results |
| 2 | MCS13 | 5.7 | 7.4 |
| 2 | MCS14 | 6.8 | 8.5 |
| 2 | MCS15 | 7.7 | 9.4 |
| 2 | MCS16 | 8.4 | 10.1 |
| 2 | MCS17 | 9.0 | 11.0 |
| 2 | MCS18 | 9.6 | 11.6 |
| 2 | MCS19 | 10.6 | 12.6 |
| 2 | MCS20 | 11.6 | 13.6 |
| 2 | MCS21 | 12.5 | 14.5 |
| 2 | MCS22 | 13.7 | 15.7 |
| 2 | MCS23 |  |  |
| 2 | MCS24 | 15.8 | 17.9 |
| 2 | MCS25 | 17.2 | 19.3 |
| 2 | MCS26 | 18.9 | 21.0 |
| 2 | MCS27 | 20.3 | 22.4 |

References

[1] 3GPP, NTT DoCoMo, R4-1904755 WF on SDR test cases, April. 2019.

[2] 3GPP, NTT DoCoMo, R4-1902402 WF on SDR requirements, Mar. 2019.

Appendix

Table A1: List of simulation cases for SDR test

| Test case | FR | SCS | Duplex mode | CBW | MIMO config. | Max rank | Max modulation order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | FR1 | 15kHz | FDD | 20MHz | 2x2MIMO | Rank 2 | 64QAM |
| Case 2 | FR1 | 15kHz | FDD | 20MHz | 2x2MIMO | Rank 2 | 256QAM |
| Case 3 | FR1 | 15kHz | FDD | 20MHz | 4x4MIMO | Rank 4 | 64QAM |
| Case 4 | FR1 | 15kHz | FDD | 20MHz | 4x4MIMO | Rank 4 | 256QAM |
| Case 5 | FR1 | 30kHz | TDD | 100MHz | 2x2MIMO | Rank 2 | 64QAM |
| Case 6 | FR1 | 30kHz | TDD | 100MHz | 2x2MIMO | Rank 2 | 256QAM |
| Case 7 | FR1 | 30kHz | TDD | 100MHz | 4x4MIMO | Rank 4 | 64QAM |
| Case 8 | FR1 | 30kHz | TDD | 100MHz | 4x4MIMO | Rank 4 | 256QAM |
| Case 9 | FR2 | 60kHz | TDD | 100MHz | 2x2MIMO | Rank2 | 64QAM |
| Case 10 | FR2 | 120kHz | TDD | 100MHz | 2x2MIMO | Rank2 | 64QAM |

Table A2: General simulation assumption

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH transmission scheme |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  | dB | N/A |
| Channel bandwidth |  | MHz | See slide #11 |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | First SSB in Slot #0 |
|  | SSB periodicity | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  | 2 |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | Full allocation |
|  | Subcarrier spacing | kHz | See slide #11 |
|  | Cyclic prefix |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |
|  | Symbols with PDCCH |  | Symbols #0 |
|  | Number of PRBs in CORESET |  | [TBD] |
|  | Number of PDCCH candidates and aggregation levels |  | [TBD] |
|  | DCI format |  | [TBD] |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | Wideband for FR1 and FR2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
|  | Antenna ports indexes |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 - 1003} for 4 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers CCs |
| PTRS configuratioaan |  |  | For FR1: PTRS is not configured For FR2: 1 port, per 2PRB in frequency domain, per symbol in time domain |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 6 for CSI-RS resource 1 and 3 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  | 1 |
|  | CDM Type |  | No |
|  | Density (ρ) |  | 3 |
|  | CSI-RS periodicity |  | 20ms |
|  | CSI-RS offset |  | 10ms from SSB |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | Number of CSI-RS ports (X) |  | Same as Tx ports |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | CSI-RS periodicity |  | 20ms |
|  | CSI-RS offset |  | 0ms from SSB |
| Maximum number of code block groups for ACK/NACK feedback |  |  | 1 |
| Maximum number of HARQ transmission |  |  | 4 |
| HARQ ACK/NACK bundling |  |  | Multiplexed |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| Precoding configuration |  |  | SP Type I, Random per slot with PRB bundling granularity |
| Symbols for all unused REs |  |  | OCNG Annex A.5 |
| Propagation condition |  |  | Static propagation condition for FR1 and FR2 |