3GPP TSG-RAN WG4 Meeting RAN4#91                                       R4-1907786

Reno, NV, USA, 13- 17 May 2019

Agenda Item:   9.23

Source:        Ericsson, Huawei

Title:          TP for TR 38.887: Regulatory situation in 37-43.5 GHz frequency range

Document for:  Approval

# Introduction

This paper is a text proposal to TR 38.887 on Regulatory situation in 37-43.5 GHz range:

# Reference

[1] https://www.itu.int/en/ITU-R/conferences/wrc/2019/Pages/reg-prep.aspx

[2] CPM19-2 report: https://www.itu.int/en/mediacentre/Pages/2019-CM02.aspx

[3] Radio Regulation 2016: https://www.itu.int/pub/R-REG-RR

# Text Proposal for NR TR 38.887

## <Start of Text Proposal>

# 4	Background

WRC-19 agenda item 1.13, Resolution 238 (WRC-15) identifies 37-40.5 GHz, 42.5-43.5 GHz, which have allocations to the mobile service on a primary basis; and 40.5-42.5 GHz which may require additional allocations to the mobile service on a primary basis, among other as IMT candidate bands. Consequently, after WRC-15 there have been extensive discussions and studies on the suitability of these bands for IMT allocation in different regulatory organizations. From the WRC-19 preparations, it is evident that many administrations across all ITU regions are interested in the IMT allocation of the whole or part of the 37-43.5 GHz frequency ranges. The position of regulatory bodies and regional forums regarding these ranges are summarized below:

Mobile Primary allocation

No Mobile allocation

APT (Asia-Pacific Telecommunity) members support identification of the frequency bands 37-40.5 GHz, 40.5-42.5 GHz and 42.5-43.5 GHz, or portions thereof, for IMT considering that protection of the incumbent services in these and adjacent frequency bands should be ensured. APT Members recognize that different administrations would implement IMT in different portions of the 37-43.5 GHz frequency range for IMT, and a global identification for IMT in the 37-43.5 GHz band, or portions thereof, would allow each country/region to implement IMT in different portions of the band in accordance with their national/regional considerations, while still facilitating the benefits of economies of scale.

ASMG (Arab Spectrum Management Group): supports IMT identification within the frequency bands 40.5-42.5GHz and 42.5-43.5GHz.

CEPT (European Conference of Postal and Telecommunications): proposes an IMT identification for 40.5-43.5 GHz. This is a priority band for CEPT and already specified for future harmonization in Europe. Whilst CEPT will not propose IMT identification at WRC-19 and has no intention of using 37-40.5 GHz for IMT, CEPT will not oppose a global IMT identification for the full 37-43.5 GHz range.

Some countries in CITEL including the US, Canada and Brazil support IMT identification for part of 37-43.5 GHz spectrum range.

Position of the RCC (Regional Commonwealth in the field of Communications) administrations on frequency bands 37.0−40.5 GHz and 40.5−42.5 GHz included in Resolution 238 (WRC-15): to be specified taking into account the need to protect both passive and active services.

ATU (African Telecommunications Union) supports the 40GHz band (37-40.5, 40.5-42.5 and 42.5-43.5 GHz frequency range) for IMT identification under Resolution 238 (WRC15).

## 4.1        Regulatory situation

Table 4.1-1 is the extract of the Radio Regulation (RR) table of allocations providing the services allocated in the 36-47 GHz frequency range.

Table 4.1.1 Allocation information in the 36-47GHz frequency range [2]

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 36-37 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) 5.149 5.550A |  |  |
| 37-37.5 FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) 5.547 |  |  |
| 37.5-38 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 38-39.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 39.5-40 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) 5.516B MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile Mobile-satellite (space-to-Earth) 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 |
| 41-42.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 5.551F 5.551H 5.551I |  |  |
| 42.5-43.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 MOBILE except aeronautical mobile RADIO ASTRONOMY 5.149 5.547 |  |  |
| 43.5-47 MOBILE 5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE 5.554 |  |  |

## <End of Text Proposal>