TSG-RAN Working Group 4 (Radio) meeting #91	R4-1907796

Reno, US, 13th – 17th May 2019

Source:	Ericsson

Title:	TP to TR 38.820: Addition of PA trends in subclause 6.3.1

Agenda item:	10.4.7

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#90bis in Xian) a TR skeleton was approved in [1]. In [2] a text proposal with technical information about PA trends was presented. At the time is was not approved. In this contribution a revised version is presented for approval.

This is a revised version of R4-1906161.

# Discussion

In [3], the trends for power amplifier semi-conductor technologies in terms of achievable peak saturated power and power added efficiency was presented. Due to complex dependency between output power, ACLR/unwanted emission and efficiency, it is important to properly document theses aspects including performance and feasibility during the SI to ensure that proper considerations when WI for standardizing specific frequency bands in 7-24 GHz range is taken.

The intention with this paper is to propose text to capture the power amplifier trends for different technologies. At previous meeting a text proposal was presented in [2]. In this contribution a revised version is presented with the following changes;

- Fonts and styles are updated to according to 3GPP drafting rules.

- Addition of reference in clause 3.

- Change marks enabled.

- The text is improved to be appropriate for TR.

# Conclusion

It is proposed that the attached text proposal is included in TR 38.820.

# References

[1]	R4-1903328, “TR 38.820: updated TR skeleton for 7-24GHz SI”, Huawei, RAN4#90bis

[2]	R4-1903465, “TP to TR: 7-24 GHz power amplifier trends”, Ericsson, RAN4#90bis

[3]	R4-1900832, “7-24 GHz technical aspects and considerations”, Ericsson, RAN4#90

TEXT PROPOSAL:

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.913: “Study on Scenarios and Requirements for Next Generation Access Technologies”

[3]	Hua Wang et al., "Power Amplifiers Performance Survey 2000-Present," Available: https://gems.ece.gatech.edu/PA_survey.html

TEXT PROPOSAL:

# 6	Study on general aspects

Editor’s note: consider aspects common to BS and UE

## 6.1	General

## 6.2	Selection of example frequencies

Editor’s note: Identify representative example frequencies for study

## 6.3	RF technology considerations in the 7-24 GHz range

Editor’s note: This section is intended to capture conclusions on the performance of key technologies in the BS and UE in the range, such as power amplifiers, filters, receivers, phase noise etc.

### 6.3.1	PA trends

For 7-24 GHz frequency range, it is essential to perform in-depth analysis of power amplifiers based on different semi-conductor technologies. In addition, complex relation between output power, ACLR (linearity) and efficiency is an important aspect considering the performance and feasibility for example frequencies.

The PA technology and trends in this paper is based on professor Hua Wang at Georgia Tech [3] work where a large power amplifier survey consisting of more than 2000 data points were published in 2008. Wangs database covers published results from year 2000 and beyond, both from the open literature as well as commercial amplifiers from various vendors.

Based on this material a more comprehensive analysis of achievable power amplifier performance has been investigated and presented in this chapter. The analysis covers the peak output power and power added efficiency. It should be noted that for all presented characteristics in this chapter, the results are based on peak power, non-linearized power amplifiers without considering the bandwidth impact to show the trends with respect to frequency for different technologies.

#### 6.3.1.1	Saturated peak output power

In Figure 6.3.1.1-1, a scatter diagram of saturated output power as a function of operating frequency for Silicon technologies is shown. The technologies are grouped in three categories: LDMOS, CMOS and SiGe. The attainable output power at a given operating frequency is limited by the saturated electron velocity and the breakdown field strength in a given semiconductor material. This is captured in Johnsons’ figure of merit which states that the maximum output power will decrease with 20 dB/decade as the operating frequency is increased. The black line 1 conveys the limitation due to Johnsons’ figure of merit.

Figure 6.3.1.1-1: Saturated output power versus frequency for LDMOS, CMOS and SiGe

Figure 6.3.1.1-2 presents the maximum output power as function of operating frequency for two compound semiconductor technologies: GaAs and GaN. Higher output power can be achieved but would necessitate excessive power combining associated with decreased operating efficiency.

Figure 6.3.1.1-2: Saturated output power versus frequency for GaAs and GaN

#### 6.3.1.2	Efficiency

In the previous section the output power capability was analyzed using published power amplifier data. In this section the same dataset will be used to study the efficiency of available semiconductor technologies.

In Figure 6.3.1.2-1 (left), a scatter diagram of peak power added efficiency as function of operating frequency for power amplifiers made using Silicon transistors (LDMOS, CMOS and SiGe). In Figure 6.3.1.2-1 (right) the same parameters are plotted for compound semiconductor transistors (GaAs and GaN). As expected the efficiency is mainly dependent on the operating frequency and not the transistor technology. The wide spread of data is mainly due to different power levels and different amplifier architectures (e.g. class-A, class-AB, Doherty etc.).

Figure 6.3.1.2-1: Peak power added efficiency versus frequency

## 6.4	Key parameters over the 7-24 GHz range

Editor’s note: Conclusions on ranges for some key parameters from a technology capability perspective (Output power, ACLR, noise figure etc.)

## 6.5	Deployment scenarios

Editor’s note: Considerations on possible deployment scenarios in this range, considering technology capabilities

## 6.6	System parameters for 7-24 GHz range

Editor’s note: Study applicable maximum bandwidths and SCS for SSB/PBCH block and control/data

## 6.7	Requirements classification

Editor’s note: conducted vs. OTA requirements. Might require splitting to UE and BS discussions.

- Study how to define a boundary frequency and/or boundary conditions, including extending FR1 upwards and/or FR2 downwards in frequency, or alternatively introduce new frequency range(s), so that it is clear where conducted requirement and/or OTA requirement will apply for both BS and UE, which requirements apply and how the requirements are derived for BS. Note there may be some grey frequency range for which conducted and/or OTA requirement will apply.

- Study shall take into account agreed design of RF specifications for UE (different specifications for FR1 and FR2) and BS (different specification for conducted and radiated test requirements)