3GPP TSG-RAN WG4 #91	R4-1907802

Reno, USA, 13th – 17th May 2019

Source: 	Ericsson

Title:  	TP to TR 38.820: Inclusion of NF Background Information for BS requirements

Agenda Item:	10.4.7

Document for:	Approval

# Introduction

This contribution provides some background information on typical noise figure values to assist in providing a basis for RF receiver requirements based on discussion in [1].  It is noteworthy that the analysis in [1] was based on assumed example frequencies of 11 GHz, 16 GHz and 22 GHz. The new example frequencies of 10 GHz, 15 GHz and 20 GHz implies some possible but limited reduction is Noise Figure due to slightly reduced losses in routing, switches etc but does not affect the typical NF value compared to previous assumption on example frequencies.

Since this TR is for the SI it should therefore capture technology aspects which would help to provide information on the parameters.  The inclusion of the text would also give background information on considerations why the NF configured would be different within the 7-24 GHz range.

# Proposal

It is proposed that the text in this paper is included in TR 38.820

# References

[1]	R4-1904070, “7-24 GHz Receiver Aspects”, Ericsson

# Text Proposal

## 5.5	Key parameters over the 7-24 GHz range

Editor’s note: Conclusions on ranges for some key parameters from a technology capability perspective (Output power, ACLR, noise figure etc.)

### 5.5.x	Noise figure

Receiver Noise Figure (NF) is one of the essential metrics for determining receiver requirements. For determining typical NF value some fundamental principles should be considered:

- The NF is not given by the LNA alone, but also by bandwidth, linearity and dynamic range dependencies as there is a delicate balance which should be considered when future requirements are specified.

- A full RF receiver chain all the way up to radiating elements should be addressed as all parts in the chain would contribute to the overall receiver performance including switch (for TDD), routing and filter losses, etc.

- For some compact and highly integrated building practices with many transceivers and antennas, the power efficiency and heat dissipation in small area/volume is necessary needs to be considered.

  - As an example, in the context of NF, it might be possible to reduce the noise contribution from ADC by using more bits, but this would have significant implication in terms of power consumption and heat dissipation aspect as a single added bit to ADC would result in four times higher power consumption. In general the ADC power consumption is proportional to

  - BW2 · DRACD

where DRACD is the ADC dynamic range.

Table 5.5.x-1	Typical noise figure for 7-24 GHz example frequencies

| Example frequency (GHz) | Typical NF values for NR BS (dB) | Typical NF values for NR UE |
| --- | --- | --- |
| 10 | 7 | TBD |
| 15 | 8 | TBD |
| 20 | 9 | TBD |

### 5.5.x	NF Analysis for NR BS

Considering recent state-of-the-art LNA only noise figure publications, Figure 5.5.x-1 presents the LNA only noise figure for different semi-conductor technologies over frequency based on published data [x].

Figure 5.5.x-1	LNA only NF trend over frequency

The typical noise figure values presented in table 5.5.x-1 are based on assumption of pre-LNA noise figure of ~1.8 dB and per example frequency calculation of losses such as routing losses, etc.