3GPP TSG-RAN WG4 #91	R4-190xxxx

Reno, United States, 13th – 17th May, 2019

Agenda item:	8.5

Source: 	Qualcomm Incorporated

Title: 	IAB Ad Hoc meeting minutes

Document for:	Approval

# General (Agenda Item: 8.5.1)

| R4-1905451: Discussion on TS skeleton and terminology Samsung | Proposal: IAB-MT and IAB-DU with tentative definition as below are suggested to be used in IAB RAN4 RF performance TS. IAB-MT: the [subset] function as UE in IAB node to communicate with its parent node. IAB-DU: the partial functions as gNB in IAB node to communicate with UE or child node |
| --- | --- |
| R4-1905452: Discussion Rel-16 IAB scope Samsung | Proposal 1: it’s expected the introduction of Rel-16 IAB node shall have no impact on below existing BS and UE specifications from RF performance perspective. - TS38.104 - TS38.101-1/2/3/4 Observation 1: the FDM and SDM resource sharing among backhaul and access links may be not in Rel-16 scope. Observation 2: simultaneously transmission of IAB-MT and IAB-DU, simultaneously reception of IAB-MT and IAB-DU seems not precluded from Rel-16 IAB scope, which is also related to slot format of IAB-MT and IAB-DU. Observation 3: according to OTA “case 1” timing of IAB-DU DL transmission, there would be misalignment between reception timing of IAB-MT DL and IAB-DU UL, between transmission timing of IAB-MT UL and IAB-DU DL. Observation 4: according to legacy definition of synchronized TDD operation, the definition of synchronized operation between IAB system and legacy NR system needs further discussion. Observation 5: even though the scenarios considered in CLI co-existence study, such as interference from gNB to gNB and interference from UE to UE, is not in IAB scope, the conclusion of CLI co-existence study can be considered for IAB as well. |
| R4-1905905: IAB definitions Huawei | Abbreviation; Description; Notes. BH; Backhaul; from 38.874. IAB; Integrated Access and Backhaul; from 38.874. DU; Distributed Unit; RAN2 logical entity in both IAB node and IAB donor (architecture 1a), talks to the UE and MT over the Uu interface. Can be mapped to either the same of different RF HW as MT.. CU; Central Unit; RAN2 logical entity in the IAB donor only (architecture 1a) talks to DU and NGC via the NG interface. MT; IAB Mobile Terminal; RAN2 logical entity in IAB node only (architecture 1a) talks to DU over the Uu interface. Can be mapped to either the same of different RF HW as DU.. Term; Definition; Notes. IAB Donor; RAN node which provides UE’s interface to core network and wireless backhauling functionality to IAB-nodes. [Tentative definition to be agreed for 38.300-->gNB that provides network access to UEs via a network of backhaul and access links]; Definition from 38.874 Contains DU and CU functions, start of the IAB chain, can only be a IAB parent? Can be standard BS?. IAB node; RAN node that supports wireless access to UEs and wirelessly backhauls the access traffic. [Tentative definition to be agreed for 38.300--->RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes.]; Definition from 38.874 Contains DU and MT functions, an IAB node can be both a parent and child depending on its place in the chain (for FDD the HW would be different due to the duplex frequency?). IAB parent; 1) An IAB node that uses RF resources like a BS , i.e. TX on DL, RX on UL 2) IAB-node-MT’s next hop neighbour node; the parent node can be IAB-node or IAB-donor-DU. IAB child; 1) An IAB node that uses RF resources like a UE , i.e. RX on DL, TX on UL 2) IAB-node-DU’s next hop neighbour node; the child node is also an IAB-node. |
| R4-1905906: IAB node antenna directional requirements Huawei | In this paper the antenna definition for the IAB node has been discussed and the antenna layout for both the micro BS/IAB node in both a dense urban and homogeneous micro urban has been highlighted. The IAB node uses 3 panels at 120° from each other, only 1 panel is assumed active at any time, the appropriate pannel is chosen for the direction of the link. The IAB node antenna can be described as follows: For FR2 (30GHz) - 3 panel omni antenna Antenna has 3 panels at 120º as shown in figure 5 Fig.5 3 panel omni antenna structure Each panel of the antenna is described as follows: Table 1: FR2 BS antenna modelling for Urban micro scenario Parameter; Values. Antenna element vertical radiation pattern (dB); AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=130°,SLAV=30dB. Antenna element horizontal radiation pattern (dB); AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=130°,Am=30dB. Combining method for 3D antenna element pattern (dB); A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am. Maximum directional gain of an antenna element GE,max; 3 dBi (assuming 1.8dB loss). (Mg, Ng, M, N, P) note; For 30GHz: (1, 1, 8, 16, 2). (dv, dh); (0.5λ, 0.5λ). |
| R4-1905907: Capturing the IAB background Huawei | This paper briefly discussed the need for a RAN4 specific TR for IAB, and expresses our opinion that it is necessary. |
| R4-1906019: Operating scenarios for IAB AT&T | Proposal 1: In case of TDM operation the following mapping of IAB node transmission/reception to a TDD pattern (DL/UL/F resources) is assumed: • IAB DU transmission / IAB MT reception: DL resources, F resources if indicated as DL by DCI • IAB MT transmission / IAB DU reception: UL resources, F resources if indicated as UL by DCI Proposal 2: Adopt the following parameters for co-existence simulations based on Table A.1-1 of TR 38.874: Parameters; Homogeneous scenario (urban micro). Layout; Hex. Grid 19 sites Number of IAB-donors (Ndonor) 3 Number of IAB-nodes is 19 – Ndonor. Inter-BS distance; 200m. Topology formation; Max RSRP of candidate parent nodes. Carrier frequency; 30GHz. Duplex mode; TDD. Aggregated system bandwidth (access + backhaul); Up to 400MHz (DL+UL). Simulation bandwidth; Per CC BW is 100MHz. Large-scale channel parameters; - Micro-to-UE: UMi-Street canyon - Micro-to-Micro: UMi-Street canyon (hUE =10m) - UE-to-UE: UMi-Street canyon (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 of TR38.802 The path loss for links between the IAB-node and candidate serving IAB-nodes/donors is determined based on N =3 independent large-scale channel realizations (taking into account LOS/NLOS probability and shadow fading). The realization that results in the minimum pathloss between the IAB-node and the associated serving IAB-node/donor is selected.. Fast fading parameters; - Micro to Micro: UMi-Street canyon O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0. BS Tx power; 33dBm EIRP should not exceed 68 dBm (*). BS antenna configurations; See Table A.2.1-4 of TR38.802. At least for the purpose of IAB evaluations, when the IAB-node has multiple panels, access and backhaul traffic can be sent on any panel, subject to the per IAB-node half duplex constraint.. BS antenna height; 10 m. BS antenna element gain + connector loss; See Table A.2.1-4 of TR38.802. BS receiver noise figure; 7dB. |
| R4-1906617: Regulation aspect for IAB operation within adjacent TDD network Ericsson | Observation#1: It clearly shows that in scenario 1, the IAB node transmit in Uplink time slot of the TDD pattern while scenario 2 has no uplink transmission during Uplink time slot of TDD pattern. Observation#2: IAB node in Scenario 1 will transmit on the uplink time slot and it will interfere the other BS receiving. Observation-3: ECC regulatory requirement does not introduce more stricter requirement than SEM defined by 3GPP when the TDD networks operated by different operator are synchronized. Observation-4: No additional regulatory requirement IAB node for scenario 2. Observation-5: For scenario 1 (or in general, transmits with greater power or beamforming gain than a UE) and use the uplink part of the TDD pattern, the IAB-BS interference will occur and the stricter ECC regulation will apply for multiple operator coexistence. Observation-6: If non synchronized TDD or uplink subframes would be used then to fulfill the regulator requirement, either vastly increased cost on the IAB node due to strict emissions requirements or carefully planned and likely infeasible increasing the separation distance to other operator’s BS are the options. None of them are attractive Observation-7: It is difficult for IAB to operate on the unsynchronized TDD without geographical separation with the baseline requirements (assuming an ACIR of 28 dB). Proposal-1: Derive RAN4 requirements based on the assumptions of synchronized TDD and that the IAB BH is configured in the downlink part of TDD pattern as baseline assumption (scenario 2) for FR2 operation. Proposal-2: Investigate if the minimal geographical separation between IAB from one operation and BS from another operation could be enforced so the IAB BH on operate in downlink part of the TDD pattern limit can be removed. |
| R4-1906618: On radio architecture discussion of IAB node Ericsson | Observation-1: Dedicated IAB-MT RF equipment will increase the cost both on hardware and test perspective. Proposal-1: The shared hardware architecture for IAB MT and DU should be prioritized in consideration of the RF spec of IAB node. |
| R4-1906621: Further Scope discussion of RF work for IAB Ericsson | Obseration-1: SDM operation is supported in clause 7.3.3. By SDM operation, it means the simultaneously transmission in DU and MT alternatively simultaneous reception in the DU and the MT. Obseration-2: The DL and UL in Table 7.3.3-2, is from backhaul link or access link perspective, DL means backhaul link from IAB parent to IAB child or access link from IAB node to UE. Obseration-3: There is no discussion in clause 7.3.3 on the TDD pattern mapping to the uplink/downlink time resources. Proposal-1: The TDD pattern mapping to the downlink/uplink time resource is vendor specific. |
| R4-1907183: IAB node radio transmission and reception Qualcomm Incorporated | Proposal 1. Introduce a new specification in the 38.1xx series that covers all the requirements applicable to IAB nodes. Proposal 2: The sections of the 38.1xx spec, covering IAB requirements, should pursue the following skeleton: 1. Scope 2. References 3. Definitions, etc 4. General 5. Operating bands and channel arrangement 6. Transmitter characteristics 7. Receiver characteristics 8. Radio resource management support 9. Performance requirements Proposal 3: RAN4 approves the work plan shown in section 3 of the contribution. |
| R4-1907187: Workplan for IAB WI Qualcomm Incorporated | As rapporteur, in this contribution, we recommend a work plan for RAN1, RAN2, RAN3 and RAN4. The workplan captures interdependences among tasks and timeline for RAN1/2/3/4. This workplan is submitted to all RAN working groups. This workplan is a revision of a prior version submitted to RAN2 #105. |
| R4-1906700: Scenarios for IAB network deployment Qualcomm Incorporated | Proposal 1: Adopt the IAB network deployment illustrated in this paper for capacity enhancement as one deployment scenario to analyze. Proposal 2: Adopt the IAB network deployment illustrated in this paper for coverage enhancement as one deployment scenario to analyze. |
| R4-1906701: Scenarios for IAB network operation Qualcomm Incorporated | Proposal 1: Adopt the following Scenario #1 as baseline scenario for coexistence analysis. Half duplex constraint will apply to all the IAB nodes of the network. Scenario #1: • IAB MT using the UL parts of the TDD pattern for IAB transmission • IAB MT using the DL parts of the TDD pattern for IAB receiving • IAB DU using the DL parts of the TDD pattern for IAB transmission • IAB DU using the UL parts of the TDD pattern for IAB receiving Proposal 2: The following half duplex constraints will apply to all the IAB nodes of the network: • MT and DU operations (resource usage) are TDM’d • DU is able to support multiple cells (and therefore multiple sectors) only if the cells are synchronized, i.e. different sectors of the same site cannot be simultaneously transmitting and receiving. Proposal 3: RAN4 to assume the same TDD pattern between two adjacent channel operators. Observation 1: Scenario 2 halves IAB network capacity that may not be desirable especially for links up in the topology chain. Observation 2: When operating an IAB network in DL parts of TDD pattern, the receiving IAB nodes will be subject to interference from NR BSs that may be operating in adjacent channel. |

# Co-existence study (Agenda Item: 8.5.2)

| R4-1905453: Discussion on IAB co-existence Samsung | Observation 1: the cases of DL to UL interference and UL to DL interference are out of IAB scope and have already been covered by CLI co-existence study. Proposal 1: for IAB RAN4 co-existence study purpose, the same TDD slot format shall be assumed in the system. - For DL reception of victim, interference comes from DL transmission only - For UL reception of victim, interference comes from UL transmission only Proposal 2: It is proposed to consider totally 4 cases illustrated in figure 2- figure 5 and summarized in table 1 for IAB co-existence study. |
| --- | --- |
| R4-1905532: Discussion on NR IAB deployment scenarios CMCC | Proposal 1: It is proposed to introduce FR1 NR bands n41 and n79 in the IAB node specification. NR operating band; Uplink (UL) and Downlink (DL) operating band FUL,low – FUL,high FDL,low – FDL,high; Duplex Mode. n41; 2496 MHz – 2690 MHz; TDD. n79; 4800 MHz – 5000 MHz; TDD. Proposal 2: It is proposed to prioritize the co-existence study for n79 in FR1 and specify the ACLR and ACS requirements. Proposal 3: RAN4 to agree to the following co-existence simulation cases for n79 deployment scenarios with synchronization operation assumption. cases; network topology; aggressor; victim; note. 1; the aggressor and the victim belong to different network with overlapping geographical areas; Macro IAB donor; Micro IAB node. 2; indoor IAB node. 3; Micro IAB node; Micro IAB node; Only for multiple hops. 4; Micro IAB node; Macro IAB donor. 5; indoor IAB node. |
| R4-1905533: Discussion on IAB simulation CMCC | Proposal 1: RAN4 to adopt the simulation steps listed in this paper. |
| R4-1905902: Discuss IAB link budget Huawei | This paper looks at a simple link budget for the IAB dense urban and urban micro scenarios. The throughput and the co-channel interference is not the purpose of the RAN4 study but it is useful to have a general understanding of the link budget. For LOS there is excess power in both the IAB parent and IAB node, however when considering NLOS it is not possible to always maintain the maximum throughput with the specified output power. There are a number of statistical parameters therefore which will influence the throughput • LOS probability • Shadow fading loss • Antenna direction and pointing loss • IAB routing The analysis also does not consider co-channel interference (although if nodes have higher power then co-channel interference should increase linearly). |
| R4-1905903: IAB discussion on FR2 co-existence simulation scenarios Huawei | Where possible it is sensible that the co-channel analysis is based on the co-channel work already carried out. As such 2 scenarios have been investigated based on the co-channel evaluation documented in TR 38.874. • Dense Urban - Heterogeneous scenario • Urban Micro – homogenous solution. The dense urban scenario can be used to investigate eth effect of the IAB network on an adjacent macro network, where the worst case is that the adjacent network is coordinated with the IAB macro network. The homogenous micro network can be used to investigate the interference between a micro IAB network and an adjacent micro network where the 2 networks are uncoordinated and have a random offset from each other. |
| R4-1905904: IAB FR2 co-existence simulation scenarios Huawei | This paper gives the suggested simulation parameters for the Dense urban and urban micro IAB co-existence simulations. |
| R4-1906020: Co-existence scenarios for IAB AT&T | Proposal 1: Adopt the following parameters for co-existence simulations based on Table A.1-1 of TR 38.874: Parameters; Homogeneous scenario (urban micro). Layout; Single layer Micro layer: Hex. Grid 19 sites Number of IAB-donors (Ndonor) 3 Number of IAB-nodes is 19 – Ndonor. Inter-BS distance; 200m. Topology formation; Max RSRP of candidate parent nodes. Carrier frequency; 30GHz. Duplex mode; TDD. Aggregated system bandwidth (access + backhaul); Up to 400MHz (DL+UL). Simulation bandwidth; Per CC BW is 100MHz. Large-scale channel parameters; - Micro-to-UE: UMi-Street canyon - Micro-to-Micro: UMi-Street canyon (hUE =10m) - UE-to-UE: UMi-Street canyon (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 of TR38.802 The path loss for links between the IAB-node and candidate serving IAB-nodes/donors is determined based on N =3 independent large-scale channel realizations (taking into account LOS/NLOS probability and shadow fading). The realization that results in the minimum pathloss between the IAB-node and the associated serving IAB-node/donor is selected.. Fast fading parameters; - Micro to Micro: UMi-Street canyon O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0. BS Tx power; 33dBm EIRP should not exceed 68 dBm (*). BS antenna configurations; See Table A.2.1-4 of TR38.802. At least for the purpose of IAB evaluations, when the IAB-node has multiple panels, access and backhaul traffic can be sent on any panel, subject to the per IAB-node half duplex constraint.. BS antenna height; 10 m. BS antenna element gain + connector loss; See Table A.2.1-4 of TR38.802. BS receiver noise figure; 7dB. Proposal 2: For the purpose of co-existence evaluations, the IAB donor nodes can be assumed to be always active and the probability that given IAB node is active should be a function of hop order and subject to the half-duplex constraint. FFS: The activity distribution, pending evaluations based on the TBD simulation assumptions. |
| R4-1906619: Initial consideration on coexisting assumption of IAB network Ericsson | Observation-1: The backhaul and access link coexistence scheme is different for scenario 1 and 2. Proposal-1: the backhaul and access link coexistence scheme for multiple hop IAB node need to be agreed. Proposal-2: the baseline scenario for scenario 1 [4] could be the normal access link operation at uplink time slot of TDD pattern without backhaul link enabled. Proposal-3: The interference scenario for scenario 1 [4] will be 2nd and fourth IAB node transmitting at uplink time slot of TDD pattern, the first (IAB donor) and third IAB node is receiving. Proposal-4: the baseline scenario for scenario 2 [4] could be IAB network without macro BS layer and enable the backhaul link. Proposal-5: The interference scenario for scenario 2 [4] will be the case where macro BS layer is added so interference occur when IAB node is receiving. Proposal-6: Hexagonal network topology is recommended to use for coexistence study. Proposal-7: IAB other coexisting other parameter should be based on [9] for the coexisting simulation assumption. |
| R4-1906620: on Antenna of IAB node Ericsson | Observation-1: The IAB antenna deployment needs to fit both access link and backhaul link requirements. Observation-2: The directivity of the AAS antenna has maximum value in the center but minimum value at the edge of the elevation or azimuth range. The difference could be as much as 10dB. Observation-3: In the deployment of the IAB antennas, the directivity of beam needs to be considered so the maximum directivity beam can be used to gain backhaul link coverage. Proposal-1: Consider at least similar height IAB antenna deployment scenarios as starting point for coexisting studies. |
| R4-1906713: Simulation assumption for IAB coexistence study ZTE Corporation | Proposal 1: RAN4 should further discuss whether all coexistence scenarios in Table 2.1-1 should be evaluated or not. Proposal 2 : RAN4 should further discuss the network layout other than HetNet and Urban Micro scenarios if needed. Proposal 3: RAN4 should further discuss the IAB node orientation for coexistence simulation. Proposal 4 : RAN4 should further discuss multi-hop and single-hop for FR1 and FR2 scenarios. Proposal 5 : maximum RSRP based method should be used for Topology formation. Proposal 6: maximum RSRP is uesd for NR UE association. Proposal 7: for legacy NR UE, propose to use the following parameters for simulation. - CLx-ile = 88 + 10*log10(200/X), where X is UL transmission BW (MHz) - γ = 1 For IAB MT in the IAB scenarios, Pmax and target SNR for IAB DU should be further investigated as there are possibilities with higher Pmax and target SNR to achieve better capacity in the backhaul link. Proposal 8: for IAB MT, to further discuss the target SNR, maximum output power and Rmin. Proposal: to use the legacy shannon equation and parameters used for mmWave NR coexistence study. |
| R4-1906730: RF architecture for IAB node ZTE Corporation | In this contribution, we want to share some further considerations on the IAB antenna structure and IAB RF architecture f: or the RF requirement discussion. |
| R4-1906881: IAB scenarios and simulation assumptions Nokia, Nokia Shanghai Bell | Observation 1: When the victim and aggressor are both configured with dynamic TDD there is no restriction on the IAB MT. Observation 2: Donor to IAB-Node and parent IAB-node to child IAB-node transmissions are similar. Observation 3: Child IAB-Node to parent IAB-node and child IAB-node to IAB-Donor transmission are similar. Proposal 1: IAB-MT RF coexistence study is based on micro deployment scenario Proposal 2: Urban macro scenario with coordinated deployment can be considered if IAB-MT transmissions take place during DL slots. Proposal 3: The IAB performance should be assessed against the baseline performance of the victim network with a NR access network acting as the aggressor Proposal 4: In a coordinated aggressor system, restrictions on the IAB MT should be investigated. Proposal 5: IAB-MT is transmits with a probability of 15%. Proposal 6: Bursty traffic model is used for IAB-MT transmissions. Proposal 7: No multi-hop scenario needs to be simulated, as long as IAB-donor and IAB-DU parameters are sufficiently similar. Proposal 8: Consider parameters in Table 2 to be used in RF co-existence study Proposal 9: Same antenna configuration is used for IAB-MT, IAB-DU and IAB-Donor |
| R4-1906882: On BS classes supported in IAB-node access link Nokia, Nokia Shanghai Bell | Proposal 1: IAB-node specification shall support all BS classes IAB access link in FR2. |
| R4-1906702: Simulation methodology and assumptions for IAB co-existence study Qualcomm Incorporated | Proposal 1: Employ RSRP based formation of network topology. Proposal 2: RAN4 to agree on a common simulation methodology and adopt the simulation steps listed in Section 3 of this paper. Proposal 3: RAN4 to agree on the simulation assumptions listed in Section 4 of this paper. |

# Discussion

## Network deployment scenarios

Layout 1:

| Layout for nodes | IAB network: Two layer Macro layer: Hex. Grid (3 sector) (all macro BSs are IAB-donors) 19 sites Micro layer: 1 micro BSs per macro BS 1. Random drop (All micro BSs are all outdoor and are IAB-nodes) 2. Drop micro nodes in a circle with center at 40m and radius of 20m Victim network 1. Macro layer: Hex grid 3 sector coordinated layout (0% grid shift) – Rel.15 legacy network 2. Same as aggressor Orientation: Highest gain towards donor |
| --- | --- |
| Inter-BS distance | Macro layer: 200m FR2, 500m FR1 |
| Minimum distance between Micro BS | 40m |
| Minimum distance between Macro BS and UE | 10m |
| Minimum distance between Micro BS and UE | 10m |

Layout 2: only FR2

Tri-sectorial hexagonal micro layout with 1 donor node. IAB nodes are connected to donor node through single or multiple hops

| Layout for nodes | IAB network: Micro layer: Hex tri-sectorial Number of IAB-donors (Ndonor): 1 Number of IAB-nodes is 19 – Ndonor Victim network 1. Micro layer: Hex tri-sectorial – grid shift derived from minimum distance(20meters and 40 meters) 2. same as agressor IAB-node and Micro BS are assumed to have 3 panels with 120 degree shift relative to each other. Use the same assumption as RAN1: The path loss for links between the IAB-node and candidate serving IAB-nodes/donors is determined based on N =3 independent large-scale channel realizations (taking into account LOS/NLOS probability and shadow fading). The realization that results in the minimum pathloss between the IAB-node and the associated serving IAB-node/donor is selected. |
| --- | --- |
| Inter-BS distance | Micro layer: 200m |

Frequency range:

30GHz for FR2

4.9GHz for FR1(beamforming is assumed)

Baseline assumption is aggressor and victim are synchronous.

Discussion

Nokia: Selection of the layout should be discussed together with the network operation

E///: for micro deployments, based on CLI experience, we do not get any relevant results because they are not close enough. Would be good to understand what is the minimum separation needed. Just a PL based analysis does not capture the co-channel interference that influences the result so some sort of hybrid analysis might be useful.

QC: we want to see the impact of the backhauling to the victim.

Huw: QC is talking about the macro+micro(1st in listy here), E/// is talking about homogeneous micro. Would be useful to identify the worst case offset instead of just studying with random drops.

E///: this is micro-micro and macro-macro.  In this case the nodes are not moving so the question is how to plan the deployment such that we avoid interference.

Nok: this also relates to the timing that the nodes use. Depending on the timing, the deployment scenario will change.

Huw: we should discuss the minimum distance between nodes.

SS: we would prefer to have only a single micro

ZTE: we could use 19 cells not just 7

AT&T: how do we enforce that it is single hop? It’s RSRP based

E///: for capacity enhancement, multi-hop will have some limitations. In this scenario it would make sense to have a single hop. Another point to consider is what shift should be assumed and how many micro nodes should we have?

AT&T: if we focus on the homogeneous scenario which is simpler, discussion for capacity enhancement might be easier

ZTE: multi hop vs. single hop is still under discussion. We only need to consider the worst case for this study so we should think whether we need to do multi hop or not. Maybe we can start with single hop

Huw: agree with ZTE, we are trying to find the worst interference case. If we have a bad link between nodes then multi-hop will likely be better. If we put them at a minimum distance we will limit the scenarios, with multi hop the beams will point in different directions and this might be a worse case.

Nok: for multi-hop we need to consider how to model the co-ex. We don’t necessarily need to model the traffic for the multiple hops.

SS: we have HD restrictions.  If we consider multi-hop, not all backhaul links will be active at the same time.

AT&T: if we go with single hop it is not clear if we need 3 nodes because they anyway have to be TDM-ed

E///: we need to focus on what the purpose for this is, to quantify interference it might be enough not to have multi-hop. We need to understand the deployment constraints.

QC: for the micro scenario, single hop might be enough. Not all nodes will be transmitting if we do multi hop

SS: we prefer single hop

E///: 19 might be better to capture overall interference

Agreement: 19 sites

E///: we look at co-ex with UEs and this is statistical, with IAB we could have some consistent interference

Huw: this is assuming that the IAB node is acting as a UE, it would interference with victim BS, they could be close to UEs and interfere with them. This would also be the difference between considering multi hop or single hop.

SS: for the scenario suggested by E///, we could do some deterministic analysis

Nok: some sort of minimum distance makes sense, this is a planned deployment. Interference towards UE will not change much if the access link comes from a gNB or from a IAB node.

E///: UEs at different distances from the network nodes will have different co-channel interference also so not clear what is the worst case scenario

AT&T: agree with Nokia, interference to other UEs will depend on the multiplexing.

Agreement:

Layout 1 :All micro IAB nodes are IAB-MTs and study will investigate at least interference on UL to the macro gNB receivers.

Layout 2: IAB nodes transmitting BH in UL slots or DL slots

AT&T: DL interference is no difference than a wired macro

SS: we also agree that we should focus on the IAB-MT

Nok: do we multiplex UEs and IAB nodes

AT&T: we need to understand what is the interference from IAB nodes, not UEs. That’s the existing network

SS: we talked about UL, Nok is talking about DL.

Victim system:

1. Legacy Rel.15 NR(no IAB nodes) : All micro IAB nodes are IAB-MTs and study will investigate at least interference on UL to the macro gNB receivers

2. Same layout as aggressor with IAB nodes deployed: study will investigate at least IAB-MTs DL reception

## IAB operation mode

Proposal: RAN4 to assume the same TDD pattern between two adjacent channel operators.

Scenario 1:

Proposal: In case of TDM operation the following mapping of IAB node transmission/reception to a TDD pattern (DL/UL/F resources) is assumed:

- IAB DU transmission / IAB MT reception: DL time slots

- IAB MT transmission / IAB DU reception: UL time slots

Scenario 2: only Layout 2

Proposal: In case of TDM operation the following mapping of IAB node transmission/reception to a TDD pattern (DL/UL/F resources) is assumed:

- IAB DU transmission / IAB MT reception: DL time slots

- IAB MT transmission / IAB DU reception: DL time slots

Proposal: The following half duplex constraints will apply to all the IAB nodes of the network:

- 1. MT and DU operations (resource usage) are TDM-ed at each site

- 2. Each site is only Tx-ing or Rx-ing on different interfaces – Layout 2 and Scenario 2

- DU is able to support multiple cells (and therefore multiple sectors) only if the cells are synchronized, i.e. different sectors of the same site cannot be simultaneously transmitting and receiving.

Proposal: Consider the cases summarized in table below for IAB co-existence study.

Case #2 and #3 applies:

Replace table with new one based on above agreements:

Table 1. Aggressor: IAB Network, Victim: NR Network

| Layout | Scenario | Aggressor system | Victim system |
| --- | --- | --- | --- |
| 1 | 1 | UL: (IAB-MT) -> (IAB-Donor) | UL: UE -> gNB |
| 2 | 1 | UL: (IAB-MT) & UE ->(IAB-DU/IAB-Donor) | UL: UE -> gNB |
|  |  | DL: (IAB-DU/IAB-Donor)->(IAB-MT) & UE | DL: gNB->UE |
|  | 2 | DL: (IAB-MT) ->(IAB-DU/IAB-Donor) & DL: (IAB-DU/IAB-Donor)->(IAB-MT & UE) | DL: gNB->UE |
| Assumption for all scenarios: |  | 1 MT and 3 DUs per site (1 DU per sector) |  |

Table 2. Aggressor: NR Network, Victim: IAB Network

| Layout | Scenario |  | Aggressor system |  |  | Victim system |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 |  | UL: UE -> gNB |  |  | UL: (IAB-MT) -> (IAB-Donor) |  |
|  |  |  | DL: gNB->UE |  |  | DL: (IAB-Donor)->(IAB-MT) |  |
| 2 | 1 |  | UL: UE -> gNB |  |  | UL: (IAB-MT) & UE ->(IAB-DU/IAB-Donor) |  |
|  |  |  | DL: gNB->UE |  |  | DL: (IAB-DU/IAB-Donor)->(IAB-MT) & UE |  |
|  | 2 | DL: gNB->UE | DL: (IAB-MT) ->(IAB-DU/IAB-Donor) & DL: (IAB-DU/IAB-Donor)->(IAB-MT & UE) |
| Assumption for all scenarios: |  |  |  | 1 MT and 3 DUs per site (1 DU per sector) |  |  |  |

Table 3. Aggressor: IAB Network , Victim: IAB Network

| Layout | Scenario | Aggressor system | Victim system |
| --- | --- | --- | --- |
| 1 | 1 | UL: (IAB-MT) -> (IAB-Donor) | UL: (IAB-MT) -> (IAB-Donor) |
| 2 | 1 | UL: (IAB-MT) & UE ->(IAB-DU/IAB-Donor) | UL: (IAB-MT) & UE ->(IAB-DU/IAB-Donor) |
|  |  | DL: (IAB-DU/IAB-Donor)->(IAB-MT) & UE | DL: (IAB-DU/IAB-Donor)->(IAB-MT) & UE |
|  | 2 | DL: (IAB-MT) ->(IAB-DU/IAB-Donor) & DL: (IAB-DU/IAB-Donor)->(IAB-MT & UE) | DL: (IAB-MT) ->(IAB-DU/IAB-Donor) & DL: (IAB-DU/IAB-Donor)->(IAB-MT & UE) |
| Assumption for all scenarios: |  | 1 MT and 3 DUs per site (1 DU per sector) |  |

Discussion

## Simulation assumptions for co-existence study

### Link level assumptions

Target SNR and maximum output power

IAB node-MT:

- SNR target: 22dB [upper limit of shannon curve]

- γ = 1

Legacy NR UE:

- SNR target: 15 dB

- γ = 1

Power control

MT for UL transmissions: Yes

DU for DL transmission: No

Map SINR into throughput with the legacy shannon equation

### Traffic model

Proposal : Full buffer at active IAB nodes subject to half duplex contraints for simulation alignment.

Traffic model and probability of MT transmission to be updated in RAN4#92

### Pathloss model

Proposal 1: Re-use the pathloss model between parent/donor IAB node DU and child IAB node MT agreed in RAN1 in TR 38.874

The path loss for links between the IAB-node and candidate serving IAB-nodes/donors is determined based on N independent large-scale channel realizations (taking into account LOS/NLOS probability and shadow fading). The realization that results in the minimum pathloss between the IAB-node and the associated serving IAB-node/donor is selected.

N=3 intra-operator serving cell

N=1 for all others

### System level parameters

| Parameters | Homogeneous scenario (urban micro) |
| --- | --- |
| Duplex mode | TDD |
| Simulation bandwidth | 100MHz for FR1, 200MHz for FR2 |
| Number of UEs in the network | FR2: 1 active UE/sector FR1: 3 active UEs/sector |

### gNB/IAB node parameters

| gNB Tx power | 33 dBm for FR2 macro and micro 46 dBm for FR1 | IAB node Tx power | 33 dBm for FR2, PC is TBD for Scenario 2 for MT link 38dBm for FR1 (medium range power limit) |
| --- | --- | --- | --- |
| gNB antenna height | 25m for macro cells and 10m for micro cells | IAB node antenna height | 10m |
| gNB receiver noise figure | 10dB for FR2 5dB for FR1 | IAB node receiver noise figure | 10dB forFR2 5dB for FR1 |

gNB/IAB node antenna configuration:

## FR1

Table 4. FR1 BS antenna modelling for macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=65°,SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=130°,Am=30dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi (assuming 1.8dB loss) |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 8, 8, 1) Note 1,2 |
| (dv, dh) | (0.8λ, 0.5λ) |
| Mechanical down tilt | 10° |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: single polarization simulated under the assumption of polarization match. |  |

Table 5. FR1 BS antenna modelling for micro

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=130°,SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=130°,Am=30dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 8, 8, 1) Note 1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Mechanical down tilt | 0° |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: single polarization simulated under the assumption of polarization match. |  |

## FR2

Table 6. FR2 BS antenna modelling for macro

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=130°,SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=130°,Am=30dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 1) Note 1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Mechanical down tilt | 10° |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: single polarization simulated under the assumption of polarization match. |  |

Table 7. FR2 BS antenna modelling for micro

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=130°,SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=130°,Am=30dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 1) Note 1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Mechanical down tilt | 0° |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: single polarization simulated under the assumption of polarization match. |  |

|  | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=130°,SLAV=30dB |
| --- | --- |
|  | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=130°,Am=30dB |
|  | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |

## 3.3.6 UE assumptions

| UE Tx power (dBm) | FR2:22.4dBm EIRP (13.4dBm conducted) FR1: 23dBm(conducted) |
| --- | --- |
| UE noise figure (dB) | 10 |

UE antenna configuration for FR2

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=130 or°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=130°,Am=25dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 1) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

UE antenna configuration for FR1

Isotropic antenna with 0dBi gain

## 3.3.7 Simulation methodology

Layout 1: Optimum orientation between parent and child

Layout 2: Antenna orientation based on planned macro layout

Topology is based on RSRP(based on PL and antenna gain)

FFS activity factor