3GPP TSG-RAN WG4 Meeting #91	R4-1907860

# Reno, Nevada, US, 13 May – 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0022 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to add NR intra-band FR2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R16_intra-Core |  |  |  |  |  | Date: |  |  | 2019-05-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding approved NR Intra-band FR2 combinations from RAN4 91 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the following: CA n257C Simplification of Table 5.5A.2-2 Missing fallbacks for n260 and n261 This CR is written towards Draft_38101-2_REL16_postRAN4_90 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Approved NR Intra-band FR2 combinations are not added |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 series |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

### 5.2A.1	Intra-band CA

NR intra-band contiguous carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1, where all operating bands are within FR2.

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR2

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n257B | n257 |
| CA_n257C | n257 |
| CA_n257D | n257 |
| CA_n257E | n257 |
| CA_n257F | n257 |
| CA_n257G | n257 |
| CA_n257H | n257 |
| CA_n257I | n257 |
| CA_n257J | n257 |
| CA_n257K | n257 |
| CA_n257L | n257 |
| CA_n257M | n257 |
| CA_n260B | n260 |
| CA_n260C | n260 |
| CA_n260D | n260 |
| CA_n260E | n260 |
| CA_n260F | n260 |
| CA_n260G | n260 |
| CA_n260H | n260 |
| CA_n260I | n260 |
| CA_n260J | n260 |
| CA_n260K | n260 |
| CA_n260L | n260 |
| CA_n260M | n260 |
| CA_n260O | n260 |
| CA_n260P | n260 |
| CA_n260Q | n260 |
| CA_n261B | n261 |
| CA_n261C | n261 |
| CA_n261D | n261 |
| CA_n261E | n261 |
| CA_n261F | n261 |
| CA_n261G | n261 |
| CA_n261H | n261 |
| CA_n261I | n261 |
| CA_n261J | n261 |
| CA_n261K | n261 |
| CA_n261L | n261 |
| CA_n261M | n261 |
| CA_n261O | n261 |
| CA_n261P | n261 |
| CA_n261Q | n261 |

---Text omitted---

## 5.5	Configurations

## 5.5A	Configurations for CA

### 5.5A.1	Configurations for intra-band contiguous CA

Table 5.5A.1-1: NR CA configurations, bandwidth combination sets and fallback group defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set / Fallback group |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated BW (MHz) | BCS | Fallback group |
|  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |  |
| CA_n257B | CA_n257B | 50 | 400 |  |  |  |  |  |  | 450 | 0 | 1 |
|  |  | 100 | 400 |  |  |  |  |  |  | 500 |  |  |
|  |  | 200 | 400 |  |  |  |  |  |  | 600 |  |  |
|  |  | 400 | 400 |  |  |  |  |  |  | 800 |  |  |
| CA_n257C | CA_n257B | 50 | 400 | 400 |  |  |  |  |  | 850 | 0 | 1 |
|  |  | 100 | 400 | 400 |  |  |  |  |  | 900 |  |  |
|  |  | 200 | 400 | 400 |  |  |  |  |  | 1000 |  |  |
|  |  | 400 | 400 | 400 |  |  |  |  |  | 1200 |  |  |
| CA_n257D | CA_n257D | 50 | 200 |  |  |  |  |  |  | 250 | 0 | 2 |
|  |  | 100 | 200 |  |  |  |  |  |  | 300 |  |  |
|  |  | 200 | 200 |  |  |  |  |  |  | 400 |  |  |
| CA_n257E | CA_n257E | 50 | 200 | 200 |  |  |  |  |  | 450 | 0 |  |
|  |  | 100 | 200 | 200 |  |  |  |  |  | 500 |  |  |
|  |  | 200 | 200 | 200 |  |  |  |  |  | 600 |  |  |
| CA_n257F | CA_n257F | 50 | 200 | 200 | 200 |  |  |  |  | 650 | 0 |  |
|  |  | 100 | 200 | 200 | 200 |  |  |  |  | 700 |  |  |
|  |  | 200 | 200 | 200 | 200 |  |  |  |  | 800 |  |  |
| CA_n257G | CA_n257G | 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n257H | CA_n257H | 100 | 100 | 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n257I | CA_n257I | 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n257J | CA_n257J | 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n257K | CA_n257K | 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n257L | CA_n257L | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n257M | CA_n257M | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |
| CA_n260B | CA_n260B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n260C | CA_n260B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  | 1200 | 0 |  |
| CA_n260D | CA_n260D | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
|  |  | 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |
| CA_n260E | CA_n260E | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 |  |
|  |  | 200 | 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |
| CA_n260F | CA_n260F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 |  |
| CA_n260G | CA_n260G | 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 3 |
|  |  | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |
| CA_n260H | CA_n260H | 100 | 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
|  |  | 50, 100 | 100 | 100 |  |  |  |  |  |  |  |  |
| CA_n260I | CA_n260I | 100 | 100 | 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
|  |  | 50, 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |
| CA_n260J | CA_n260J | 100 | 100 | 100 | 100 | 50, 100 |  |  |  | 500 | 0 |  |
| CA_n260K | CA_n260K | 100 | 100 | 100 | 100 | 100 | 50, 100 |  |  | 600 | 0 |  |
| CA_n260L | CA_n260L | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 |  | 700 | 0 |  |
| CA_n260M | CA_n260M | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 | 800 | 0 |  |
| CA_n260O | CA_n260O | 50, 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n260P | CA_n260P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n260Q | CA_n260Q | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| CA_n261B | CA_n261B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n261C | CA_n261B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  | 8501 | 0 |  |
| CA_n261D | CA_n261D | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
|  |  | 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |
| CA_n261E | CA_n261E | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 |  |
|  |  | 200 | 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |
| CA_n261F | CA_n261F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 |  |
| CA_n261G | CA_n261G | 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 3 |
|  |  | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |
| CA_n261H | CA_n261H | 100 | 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
|  |  | 50, 100 | 100 | 100 |  |  |  |  |  |  |  |  |
| CA_n261I | CA_n261I | 100 | 100 | 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
|  |  | 50, 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |
| CA_n261J | CA_n261J | 100 | 100 | 100 | 100 | 50, 100 |  |  |  | 500 | 0 |  |
| CA_n261K | CA_n261K | 100 | 100 | 100 | 100 | 100 | 50, 100 |  |  | 600 | 0 |  |
| CA_n261L | CA_n261L | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 |  | 700 | 0 |  |
| CA_n261M | CA_n261M | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 | 800 | 0 |  |
|  |  | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |
| CA_n261O | CA_n261O | 50, 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n261P | CA_n261P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n261Q | CA_n261Q | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| NOTE 1: The maximum bandwidth of band n261 is 850MHz |  |  |  |  |  |  |  |  |  |  |  |  |

---Text omitted---

Table 5.5A.2-1: NR CA configurations  with single CA bandwidth class defined for intra-band non-contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | SCS | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |
| CA_n257(2A) | - | 60 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |
| CA_n260(2A) | - | 60 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |
| CA_n260(3A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |
| CA_n260(4A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  |  |  |  |  | 1600 |  |
| CA_n260(5A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  |  |  |  | 2000 |  |
| CA_n260(6A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  |  |  | 2400 |  |
| CA_n260(7A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  |  | 26001 |  |
| CA_n260(8A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  | 1600 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  | 26501 |  |
| CA_n260(9A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  | 1800 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  | 26002 | 0 |
| CA_n260(10A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  | 2000 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  | 25502 | 0 |
| CA_n260(2D) | - | 60 | 50, 100, 200 | 200 | 50, 100, 200 | 200 |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | 120 | 50, 100, 200 | 200 | 50, 100, 200 | 200 |  |  |  |  |  |  |  |  |  |  | 800 |  |
| CA_n260(2G) | - | 60 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
|  |  | 120 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 400 |  |
| CA_n260(3G) | - | 60 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | 120 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 |  |
| CA_n260(4G) | - | 60 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  | 800 | 0 |
|  |  | 120 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  | 800 |  |
| CA_n260(2H) | - | 60 | 100 | 100 | 50, 100 | 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | 120 | 100 | 100 | 50, 100 | 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 |  |
| CA_n260(2O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 400 |  |
| CA_n260(3O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 |  |
| CA_n260(4O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  | 800 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  | 800 |  |
| CA_n260(2P) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 |  |
| CA_n260(3P) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 900 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 900 |  |
| CA_n260(4P) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 1200 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 1200 |  |
| CA_n260(2Q) | - | 60 | 50, 100 | 50, 100, | 50, 100 | 50, 100 | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  |  |  | 800 | 0 |
|  |  | 120 | 50, 100 | 50, 100, | 50, 100 | 50, 100 | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  |  |  | 800 |  |
| CA_n261(2A) | - | 60 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |
| CA_n261(3A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  |  |  |  |  |  | 7001 |  |
| CA_n261(4A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  |  |  |  |  |  |  |  | 7001 |  |
| CA_n261(2D) | - | 60 | 50, 100, 200 | 200 | 50, 100, 200 | 200 |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | 120 | 200 | 50, 100, 200 | 200 | 50, 100, 200 |  |  |  |  |  |  |  |  |  |  | 800 |  |
| CA_n261(2G) | - | 60 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
|  |  | 120 | 50, 100 | 100 | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |  | 400 |  |
| CA_n261(3G) | - | 60 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | 120 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 |  |  |  |  |  |  |  |  | 600 |  |
| CA_n261(4G) | - | 60 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 |  |  |  |  |  |  | 7001 | 0 |
|  |  | 120 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 | 50, 100 | 100 |  |  |  |  |  |  | 7001 |  |
| CA_n261(2H) | - | 60 | 100 | 100 | 50, 100 | 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | 120 | 100 | 100 | 50, 100 | 100 | 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 |  |
| CA_n261(2I) | - | 60 | 100 | 100 | 100 | 50, 100 | 100 | 100 | 100 | 50, 100 |  |  |  |  |  |  | 600 | 0 |
|  |  | 120 | 100 | 100 | 100 | 50, 100 | 100 | 100 | 100 | 50, 100 |  |  |  |  |  |  | 600 |  |
| CA_n261(2O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 400 |  |
| CA_n261(3O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 |  |
| CA_n261(4O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  | 7001 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  | 7001 |  |
| CA_n261(5O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 6501 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 6501 |  |
| CA_n261(6O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 6001 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  | 6001 |  |
| A_n261(7O) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 5501 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 5501 |  |
| CA_n261(2P) | - | 60 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | 120 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 600 |  |
| CA_n261(2Q) | - | 60 | 50, 100 | 50, 100, | 50, 100 | 50, 100 | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  |  |  | 800 | 0 |
|  |  | 120 | 50, 100 | 50, 100, | 50, 100 | 50, 100 | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  |  |  | 800 |  |
| NOTE 1: The maximum bandwidth of band n261 is 850MHz and a non-contiguous gap is in between NR component carriers NOTE 2: The maximum bandwidth of band n260 is 3000MHz, and a 50MHz gap is in between non-contiguous CCs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.5A.2-2: NR CA configurations and bandwidth combination sets for intra-band non-contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | CBW (MHz) |  | CBW (MHz) |  | CBW (MHz) | CBW (MHz) | CBW (MHz) |  | CBW (MHz) |  | CBW (MHz) |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |
| CA_n260(A-D) | - | n260 |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260D |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-D) | - | CA_n260(2A) |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260D |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2D) | - | n260 |  | CA_n260(2D) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(2D) |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2D) | - | CA_n260(2A) |  |  |  | CA_n260(2D) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(2D) |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-D-O) | - | n260 |  | CA_n260D |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260D |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-D-O) | - | CA_n260(2A) |  |  |  | CA_n260D |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260D |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-D-2O) | - | n260 |  | CA_n260D |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260D |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-D-2O) | - | CA_n260(2A) |  |  |  | CA_n260D |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260D |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-G) | - | n260 |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260G |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-G) | - | CA_n260(2A) |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260G |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2G) | - | n260 |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(2G) |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2G) | - | CA_n260(2A) |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(2G) |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2G-O) | - | CA_n260(2A) |  |  |  | CA_n260(2G) |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260(2A) |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2G-2O) | - | CA_n260(2A) |  |  |  | CA_n260(2G) |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(2A) |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n260(2G) |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |
| CA_n260(3A-2G) | - | CA_n260(3A) |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(2G) |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-G) | - | CA_n260(4A) |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 | 0 |
|  |  | CA_n260G |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-2G) | - | CA_n260(4A) |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  | 2000 | 0 |
|  |  | CA_n260(2G) |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2G-2O) | - | n260 | CA_n260(2G) |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-G-2O) |  | CA_n260(2A) |  |  | CA_n260G |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(3A-G) | - | CA_n260(3A) |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260G |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2H) | - | n260 |  | CA_n260(2H) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(2H) |  |  |  |  |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-H) | - | CA_n260(2A) |  |  |  | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1100 | 0 |
|  |  | CA_n260H |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2H) | - | CA_n260(2A) |  |  |  | CA_n260(2H) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260(2H) |  |  |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-H) | - | n260 |  | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n260H |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-O) | - | n260 |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260O |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-O-2P) | - | n260 | CA_n260O |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-O-P) | - | CA_n260(2A) |  |  | CA_n260O |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  | 1300 | 0 |
| CA_n260(2A-O-2P) | - | CA_n260(2A) |  |  | CA_n260O |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(2A-2O-P) | - | CA_n260(2A) |  |  | CA_n260(2O) |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  | 1500 | 0 |
| CA_n260(A-O-Q) | - | n260 | CA_n260O |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(A-O-2Q) | - | n260 | CA_n260O |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(2A-O-Q) | - | CA_n260(2A) |  |  | CA_n260O |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(2A-O-2Q) | - | CA_n260(2A) |  |  | CA_n260O |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 | 0 |
| CA_n260(2A-2O-Q) | - | CA_n260(2A) |  |  | CA_n260(2O) |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(2A-O) | - | CA_n260(2A) |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2O) | - | n260 |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2O-P) | - | n260 | CA_n260(2O) |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  | 1100 | 0 |
| CA_n260(A-2O-2P) | - | n260 | CA_n260(2O) |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(A-2O-Q) | - | n260 | CA_n260(2O) |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(A-2O-2Q) | - | n260 | CA_n260(2O) |  |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(2A-2O) | - | CA_n260(2A) |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2O-2P) | - | CA_n260(2A) |  |  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  | 1800 | 0 |
|  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  | CA_n260(2A) |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |
| CA_n260(2A-2O-2Q) | - | CA_n260(2A) |  |  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  | 2000 | 0 |
|  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2A) |  | CA_n260(2O) |  |  |  |  |  |  |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |
| CA_n260(2A-3O) | - | CA_n260(2A) |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3A-2O) | - | CA_n260(3A) |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-O) | - | CA_n260(4A) |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(5A-O) | - | CA_n260(5A) |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  | 2400 | 0 |
| CA_n260(6A-O) | - | CA_n260(6A) |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | 27002 | 0 |
| CA_n260(7A-O) | - | CA_n260(7A) |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  | 26502 | 0 |
| CA_n260(8A-O) | - | CA_n260(8A) |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  | 26002 | 0 |
| CA_n260(4A-2O) | - | CA_n260(4A) |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 2000 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-4O) | - | CA_n260(4A) |  |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 2400 | 0 |
|  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |
| CA_n260(4A-2Q) | - | CA_n260(4A) |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  | 2400 | 0 |
|  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |
| CA_n260(3A-3O) | - | CA_n260(3A) |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-G-O) | - | n260 |  | CA_n260G |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260G |  |  |  | CA_n260O |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-G-2O) | - | n260 | CA_n260G |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(2A-G-O) | - | CA_n260(2A) |  |  |  | CA_n260G |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260G |  |  |  | CA_n260O |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2G-O) | - | n260 |  | CA_n260(2G) |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(2G) |  |  |  |  |  | CA_n260O |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-3O) | - | n260 |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3A-O) | - | CA_n260(3A) |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3A-O-P) | CA_n260O, CA_n260P | CA_n260(3A) |  |  |  |  | CA_n260O |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  | 1700 | 0 |
|  |  | CA_n260P |  |  |  |  | CA_n260(3A) |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n260O |  |  |  | CA_n260P |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-4O) | - | n260 |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |
| CA_n260(2A-4O) | - | CA_n260(2A) |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
|  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |
| CA_n260(3A-4O) | - | CA_n260(3A) |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2000 | 0 |
| CA_n260(4A-4O) | - | CA_n260(4A) |  |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 2400 | 0 |
| CA_n260(5A-4O) | - | CA_n260(5A) |  |  |  |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  | 26002 | 0 |
| CA_n260(A-P) | - | n260 |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n260P |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-3P) | - | n260 | CA_n260(3P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(A-4P) | - | n260 | CA_n260(4P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(A-P-Q) | CA_n260P, CA_n260Q | n260 |  | CA_n260P |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  | 1100 | 0 |
|  |  | CA_n260Q |  |  |  |  |  | n260 |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n260P |  |  |  |  | CA_n260Q |  |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-P) | - | CA_n260(2A) |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1100 | 0 |
|  |  | CA_n260P |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3A-P) | - | CA_n260(3A) |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1300 | 0 |
| CA_n260(4A-P) | - | CA_n260(4A) |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  | 1900 | 0 |
| CA_n260(5A-P) | - | CA_n260(5A) |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  | 2300 | 0 |
| CA_n260(6A-P) | - | CA_n260(6A) |  |  |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  | 2700 | 0 |
| CA_n260(A-2P) | - | n260 |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2P) | - | CA_n260(2A) |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
|  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-3P) | - | CA_n260(2A) |  |  | CA_n260(3P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1700 | 0 |
| CA_n260(2A-4P) | - | CA_n260(2A) |  |  |  | CA_n260(4P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2000 | 0 |
|  |  | CA_n260(4P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |
| CA_n260(3A-2P) | - | CA_n260(3A) |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 | 0 |
| CA_n260(4A-2P) | - | CA_n260(4A) |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  | 2200 | 0 |
| CA_n260(5A-2P) | - | CA_n260(5A) |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  | 2600 | 0 |
| CA_n260(6A-2O) | - | CA_n260(6A) |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  | 24502 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(5A-3O) | - | CA_n260(5A) |  |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  | 2600 | 0 |
| CA_n260(6A-3O) | - | CA_n260(6A) |  |  |  |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  | 26002 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(7A-3O) | - | CA_n260(7A) |  |  |  |  |  |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  | 25502 | 0 |
| CA_n260(6A-2P) | - | CA_n260(6A) |  |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  | 26502 | 0 |
|  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(8A-2O) | - | CA_n260(8A) |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  | 25502 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(8A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-Q) | - | n260 | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(A-2Q) | - | n260 | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-Q) | - | CA_n260(2A) |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-2Q) | - | CA_n260(2A) |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(3A-Q) | - | CA_n260(3A) |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(3A-2Q) | - | CA_n260(3A) |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2400 | 0 |
| CA_n260(4A-Q) | - | CA_n260(4A) |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  | 2400 | 0 |
| CA_n260(D-2G) | - | CA_n260D |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(2G) |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2D-O) | - | CA_n260(2D) |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260(2D) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-2O) | - | CA_n260D |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-I) | CA_n260I | n260 |  | CA_n260I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260I |  |  |  |  |  | n260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-G) | CA_n260D CA_n260G | CA_n260D |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260G |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-H) | CA_n260D CA_n260H | CA_n260D |  |  |  | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n260H |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-I) | CA_n260D CA_n260I | CA_n260D |  |  |  | CA_n260I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260I |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-O) | CA_n260D CA_n260O | CA_n260D |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-P) | CA_n260D CA_n260P | CA_n260D |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n260P |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(D-Q) | CA_n260D CA_n260Q | CA_n260D |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260Q |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(E-O) | CA_n260E CA_n260O | CA_n260O |  |  |  | CA_n260E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260E |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(E-P) | CA_n260E CA_n260P | CA_n260E |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8001 | 0 |
|  |  | CA_n260P |  |  |  |  | CA_n260E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(E-Q) | CA_n260E CA_n260Q | CA_n260E |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260Q |  |  |  |  |  | CA_n260E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-I) | CA_n260G CA_n260I | CA_n260G |  |  |  | CA_n260I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260I |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-O) | - | CA_n260G |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-2O) | - | CA_n260G |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2G-O) | - | CA_n260(2G) |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2G-2O) | - | CA_n260(2G) |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-3O) | - | CA_n260G |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3G-O) | - | CA_n260(3G) |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260(3G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2G-3O) | - | CA_n260(2G) |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-4O) | - | CA_n260G |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |
| CA_n260(2G-4O) | - | CA_n260(2G) |  |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
|  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |
| CA_n260(4G-O) | - | CA_n260(4G) |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260(4G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(H-O) | - | CA_n260H |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 500 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2H-O) | - | CA_n260(2H) |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n260O |  |  |  | CA_n260(2H) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(O-2P) | - | CA_n260O |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(O-2Q) | - | CA_n260O |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(O-P) | - | CA_n260O |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 500 | 0 |
| CA_n260(2O-P) | - | CA_n260(2O) |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n260(2O-2P) | - | CA_n260(2P) |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  | 1000 | 0 |
|  |  | CA_n260(2O) |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(O-Q) | - | CA_n260O |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(2O-Q) | - | CA_n260(2O) |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(2O-2Q) | - | CA_n260(2O) |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(P-Q) | - | CA_n260P |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(A-D) | - | n261 |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n261D |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-2D) | - | n261 |  | CA_n261(2D) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261(2D) |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-D-H) | - | n261 |  | CA_n261D |  |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261H |  |  |  |  | CA_n261D |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-D-O) | - | n261 | CA_n261D |  |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
| CA_n261(A-D-2O) | - | n261 |  | CA_n261D |  |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
|  |  | CA_n261(2O) |  |  |  |  |  | n261 |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n261D |  |  |  | CA_n261(2O) |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-G) | - | n261 |  | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n261G |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-G-H) | - | n261 |  | CA_n261G |  |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
|  |  | CA_n261H |  |  |  |  | CA_n261G |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-G-I) | - | n261 |  | CA_n261G |  |  | n261I |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
|  |  | n261 |  |  |  |  |  | CA_n261G |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-G-O) | - | n261 | CA_n261G |  |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
| CA_n261(A-G-2O) | - | n261 | CA_n261G |  |  |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(A-2G-O) | - | n261 | CA_n261(2G) |  |  |  |  |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(A-2G-2O) | - | n261 |  | CA_n261(2G) |  |  |  |  |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  | 6501 | 0 |
|  |  | CA_n261(2O) |  |  |  |  |  | n261 |  | CA_n261(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n261(2G) |  |  |  |  |  | CA_n261(2O) |  |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(A-3G) | - | n261 | CA_n261(3G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(A-3G-O) | - | n261 |  | CA_n261(3G) |  |  |  |  |  |  |  |  |  | CA_n261O |  |  |  |  |  |  |  | 6501 | 0 |
|  |  | CA_n261O |  |  |  | n261 | CA_n261(3G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n261(3G) |  |  |  |  |  |  |  |  |  | CA_n261O |  |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(A-2G) | - | n261 | CA_n261(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |  |
| CA_n261(A-4G) | - | n261 |  | CA_n261(4G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6501 | 0 |
|  |  | CA_n261(4G) |  |  |  |  |  |  |  |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(A-H) | - | n261 |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n261H |  |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-2H) | - | n261 |  | CA_n261(2H) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
|  |  | CA_n261(2H) |  |  |  |  |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-H-I) | - | n261 |  | CA_n261H |  |  |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261I |  |  |  |  |  | CA_n261H |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |  |
| CA_n261(A-I) | - | n261 |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n261I |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-2I) | - | n261 |  | CA_n261(2I) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261(2I) |  |  |  |  |  |  |  |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(A-O) | - | n261 | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-2O) | - | n261 | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
| CA_n261(A-3O) | - | n261 | CA_n261(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(A-4O) | - | n261 |  | CA_n261(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6501 | 0 |
|  |  | CA_n261(4O) |  |  |  |  |  |  |  |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(A-5O) | - | n261 | CA_n261(5O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6001 | 0 |
| CA_n261(A-6O) | - | n261 | CA_n261(6O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5501 | 0 |
| CA_n261(A-7O) | - | n261 |  | CA_n261(7O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5001 | 0 |
|  |  | CA_n261(7O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n261 |  |  |
| CA_n261(A-P) | - | n261 | CA_n261P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(A-2P) | - | n261 |  | CA_n261(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261(2P) |  |  |  |  |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(A-Q) | - | n261 | CA_n261Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-2Q) | - | n261 |  | CA_n261(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
|  |  | CA_n261(2Q) |  |  |  |  |  |  |  |  |  |  |  |  | n261 |  |  |  |  |  |  |  |  |
| CA_n261(D-G) | CA_n261D CA_n261G | CA_n261D |  |  |  | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n261G |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(D-H) | CA_n261D CA_n261H | CA_n261D |  |  |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n261H |  |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(D-I) | CA_n261D CA_n261I | CA_n261D |  |  |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n261I |  |  |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(D-O) | CA_n261D CA_n261O | CA_n261D |  |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n261O |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(D-2O) | - | CA_n261D |  |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
| CA_n261(D-P) | CA_n261D CA_n261P | CA_n261D |  |  |  | CA_n261P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n261P |  |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(D-Q) | CA_n261D CA_n261Q | CA_n261D |  |  |  | CA_n261Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n261Q |  |  |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(E-O) | CA_n261E CA_n261O | CA_n261E |  |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
|  |  | CA_n261O |  |  |  | CA_n261E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(E-P) | CA_n261E CA_n261P | CA_n261E |  |  |  |  | CA_n261P |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 900 | 0 |
|  |  | CA_n261P |  |  |  |  | CA_n261E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(E-Q) | CA_n261E CA_n261Q | CA_n261E |  |  |  |  | CA_n261Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8001 | 0 |
|  |  | CA_n261Q |  |  |  |  |  | CA_n261E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(G-I) | - | CA_n261G |  |  |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
|  |  | CA_n261I |  |  |  |  |  | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(G-H) | - | CA_n261G |  |  |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 500 | 0 |
|  |  | CA_n261H |  |  |  |  | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261(2G-2O) | - | CA_n261(2G) |  |  |  |  |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(G-O) | - | CA_n261G |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
| CA_n261(G-2O) | - | CA_n261G |  |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(2G-O) | - | CA_n261(2G) |  |  |  |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(3G-O) | - | CA_n261(3G) |  |  |  |  |  |  |  |  |  | CA_n261O |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(H-I) | - | CA_n261H |  |  |  |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
|  |  | CA_n261I |  |  |  |  |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The maximum bandwidth of band n261 is 850MHz and a non-contiguous gap is in between NR component carriers NOTE 2: The maximum bandwidth of band n260 is 3000MHz and a non-contiguous gap is in between NR component carriers NOTE 3: Channel bandwidth per operating band defined in Table 5.3.5-1 NOTE 4: Configurations for intra-band contiguous CA defined in Table 5.5A.1-1 NOTE 5: Configurations for intra-band non-contiguous CA defined in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

---End of changes---