3GPP TSG-RAN WG4 Meeting #92                                                                R4-1907996

# Ljubljana, Slovenia, 26th– 30th August, 2019

Source:	Verizon, Nokia, Ericsson, [Qualcomm]

Title:	Requirement of n48 uplink intra-band CA

Agenda item:			10.1.2

Document for:	Approval

- Introduction

RAN4 has introduced Band n48 in Rel.16 spec [1], including channel bandwidth and intra-band CA configurations. It is clear not all of downlink channel bandwidths would be applicable to its uplink component carrier as illustrated in Table 5.3.5-1 below.

In addition, a set of n48 intra-band contiguous and non-contiguous CA configurations are defined in Table 5.5A.1-1 and Table 5.5A.2-1 of [1] respectively, however the uplink CA configurations are not specified.

This contribution provides input for the n48 uplink CA configuration for RAN4 decision.

- Discussion

As indicated above, the maximum n48 uplink carrier bandwidth for a carrier is restricted by 40MHz. This limitation will be applied to n48 intra-band (contiguous and non-contiguous) uplink CA configurations in consistency of the requirement.

Once we look at in detail of Table 5.5A.1-1 [1], it is easy to find the CA configuration CA_n48C cannot be applied in uplink CA configuration because the carrier bandwidths have exceeded the limitation. The set of CA_n48B is one that allows UE to ensure both carrier bandwidth and SEM compliance.

In contribution [2], it analysed the CBRS spurious emission requirements and partitioned the A-MPR values into eight (8) regions based on a method to measure the emission level tighter outside of the band measured from inside the bandwidth. For a 40MHz bandwidth operation on both edges of spectrum, the A-MPR in range A7 is up to 11.5dBm for -40dBm emission level. And, RAN4 adopted the measured results and defined NS_27 for single n48 component carrier in [1] in order to meet the regulated requirement.

In our view, the uplink aggregated bandwidth should be in 20 - 40MHz ranges, and this will keep the aggregated n48 bandwidth similar to its upper limit of single carrier bandwidth. Although we could go additional A-MPR values to wider bandwidth from n48 intra-band combination, higher power reduction is expected in scenarios where simultaneous transmission on multiple carriers in scenarios where output power should be reduced to meet -40dBm emission. In consequence, the A-MPR could be so large that effectively the UE drops the transmission or transmits at such a lower power level that the base-station cannot receive the transmission.

Proposal 1: Configuration requirements for n48 uplink intra-band contiguous CA should be part of Rel.16 EN-DC specifications, corresponding to n48 downlink CA configurations.

Proposal 2: The aggregated bandwidth for n48 uplink CA will be limited in 20 - 40MHz for both contiguous and non-contiguous CA.

- Conclusion

In Rel.16, 3GPP will define uplink configuration for n48 CA. We provide our input to RAN4 along with following proposals,

Proposal 1: Configuration requirements for n48 uplink intra-band contiguous CA should be part of Rel.16 EN-DC specifications, corresponding to n48 downlink CA configurations.

Proposal 2: The aggregated bandwidth for n48 uplink CA will be limited in 20 - 40MHz for both contiguous and non-contiguous CA.

Reference

[1]	38.101-1 (16.0.0), User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[2]	R4-1900055, TP to TR 38.173: n48 A-MPR, Nokia