3GPP TSG RAN WG4 Meeting #92	R4-1908003

Ljubljana, Slovenia, 26 - 30 August 2019

Agenda item:	7.3.1

Source:	SoftBank Corp.

Title:	On UE CBW determination for initial BWP

Document for:	discussion

1	Introduction

Determination of UE CBW (Channel Bandwidth in RAN4) in RRC_idle, typically needed for Random access MSG1/3 transmission, still seems outstanding in REL-15. Without clear definition, regulatory compliance such as SEM/ACLR could be at risk in Stand-alone operation [1]. Discussion was mainly conducted in RAN2 and RAN4 received an LS on the subject [2]. This paper considers issues raised by RAN2.

As RAN2 LS uses a term “channel bandwidth” for carrierBandwidth IE [TS38.331], Channel Bandwidth in RAN4 context is expressed as CBW in this paper.

2  Summary of the LS

The LS raised three items relevant to bandwidths and in RAN2’s understanding as below.

### Background and Terminology For each of its serving cells the UE is configured with parameters defining three types of bandwidths: 1) The “channel bandwidth”, which is conveyed in SIB1 and can be optionally overridden by dedicated signalling, and is applicable to all BWPs in the serving cell 2) The “BWP bandwidth”, which is conveyed in SIB1 for the initial BWP and in dedicated signalling for dedicatedly configured BWPs. 3) The “CORESET#0 bandwidth” which is conveyed in MIB RAN2 understands that the “channel bandwidth” is used for determining the RF characteristics of transmission and reception of serving cell, whereas the “BWP bandwidth” is used by the scheduler to determine which PRBs UE uses (e.g. when scheduled) in the active BWP, and the “CORESET#0 bandwidth” is used to denote the CORESET size in MIB.

And RAN2 agreed:

Table. 1 Summary of RAN2 agreements in LS

|  | Channel bandwidth (carrierBandwidth) | Initial BWP Bandwidth (locationAndBandwidth) | CORESET#0 Bandwidth (ControlResourceSetZero) |
| --- | --- | --- | --- |
| UE to verify when camping on? | N | Y | N (all the values are supported in UE as below) |
| Supported values (both UL/DL) : Unit in [RB] | No agreement mentioned | > CORESET#0 values, > Value corresponding to CBW defined for the band > Value corresponding to CBW that UE supports with capability Q1) support of values lower than CBW needed? | > All the values supported [sec 13, 38.213]: {24, 48, 96} |

In addition, under the section of “Configuring the Channel Bandwidth”, with the understanding that “this still doesn’t solve the question of which channel bandwidth would such a UE use for initial access before network is able to indicate a UE-specific channel bandwidth via dedicated signalling”, there are question/comment on RAN4 status of UE CBW determination while UE remains in RRC_idle:

Q2) Did RAN4 specify how a UE not supporting the channel bandwidth indicated in SIB1 behaves during that time span? Or what happens if such a UE is never provided with the dedicated channel bandwidth?

C1) RAN2 considers there may be cases when the gNB has no option to configure a channel bandwidth which is supported by the UE: E.g. if the gNB that only supports channel bandwidths of X MHz or less, but the UE only supports channel bandwidths of more than X MHz.

3  CBW indication in RRC_idle

Firstly, at present, it is understood that an answer to Q2 is No (or unclear) and we need to define an appropriate method.

3.1 CBW for UL or UL/DL

Before considering a method, one thing to note is that, for regulatory compliance of a UE, CBW of UE Tx (UL) is of importance in terms of SEM/ACLR or protection, i.e. how MPR/A-MPR is to be applied for Random access MSG1/3. However, from RAN4 RF perspective, DL CBW would not have serious necessity as far as a rational initial DL BWP is configured. To simplify the situation, we’d like to confirm:

[Observation-1] In RAN4 UE RF perspective, UL CBW is essential to be defined for regulatory compliance while DL CBW may not be.

In this context, it seems that it is not useful to address “CORESET#0 bandwidth” in the first place since this is solely for DL.

[Observation-2] CORESET#0 bandwidth is not suited to CBW indication in the first place as it is solely for DL.

Note that CORESET#0 configuration is used as initial DL BWP when there is no initial DL BWP information provided in SIB1. On the contrary, in Pcell or Scell, initial UL BWP is provided. [Sec 12, TS38.213] So if observation-1 is valid, we can leave initial DL BWP related procedures as they are: there is no problem if CORESET#0 bandwidths (24, 48 or 96 RBs) are also used for initial DL BWP. If we need to indicate DL CBW via initial DL BWP bandwidth, this would have some impacts to existing procedures or RAN2 agreements mentioned in the LS.

3.2 How UE CBW is indicated

Apparently, it is destructive to try to add a new IE to REL-15 at this stage. Thus, all we can do is to do add some definition/restriction to existing IE to indicate UE CBW. RAN4 needs an IE to instruct UE CBW where values should be limited to maximum transmission bandwidth configuration (NRB) in 38.101-1/2 so that a UE can set relevant CBW without ambiguity. Note that, similar to “uplink/downlinkChannelBW-PerSCS-List”, we need not only width but also position of RBs.

[Observation-3] We need to instruct RB width and its position as UE CBW where the RB widths are to be limited to NRB.

Judging from the LS, candidates are Initial BWP bandwidth or channel bandwidth (with offsetToCarrier). Both can be used for this purpose in the senses that 1) both can be configured UL/DL per SCS independently, 2) both can indicate location and RB numbers and 3) both are configurable or can be overridden with UE-specific information later.

Logically speaking, we can select either. Given that in the LS ‘RAN2 understands that the “channel bandwidth” is used for determining the RF characteristics of transmission and reception of serving cell’, it looks straightforward for channel bandwidth to provide UE CBW. On the other hand, RAN2 agreed to put some limitations to initial BWP so it would be easier to use the IE if we need to support limited values (NRB) for UE CBW.

Then, which IE to take for UE CBW indication would be left for discussion, checking various pros and cons. Note that if a limitation is put on values of these IEs, we should note that the limitation is only valid during RRC_idle.

For DL CBW, while it does not seem essential for RAN4 RF point of view, we can use relevant DL IE corresponding to selected UL IE (i.e. initial DL CBW/CORESET#0 or DL channel bandwidth) under the same constraints. Then,

[Observation-4] Either initial UL BWP bandwidth or UL channel bandwidth can be used to convey UL UE CBW in RRC_idle. For DL UE CBW, relevant IE for DL can be used if necessary.

Concerning unknown values for a UE, since UL CBW is relevant to some regulatory aspects such as SEM, ACLR or protection, the UE should be operated if the UL CBW is unknown to the UE.

[Observation-5] A UE should not be operated if an indicated UL CBW is unknown to the UE.

It depends on how the UL CBW values are selected: there should be no concern if values are selected only from NRB of mandatory CBWs, while we need a statement to prohibit camp-on if values other than for NRB mentioned above have to be supported.

3.3 Case of unsupported CBW only?

As labelled in C1 in section 2, RAN2 commented that there might be a case that a gNB cannot set UE CBW supported: supported gNB CBWs are lower than UE CBWs. From CBW definition standpoint in RAN4, there are a set of CBWs for a UE/BS defined in band-by-band manner and the set of CBWs normally covers the smallest CBW for the band as mandatory for UE. Then it seems, as far as UE CBW is selected from the mandatory CBWs, there should be no concern as such.

[Observation-6] On C1, given that RAN4 has defined a mandatory set of UE CBWs per band including the smallest CBW, there would be no problem as indicated in the LS.

Related to this comment, if a new CBW is added to a certain band in a certain release, to guarantee backward compatibility, UE CBW for RRC_idle should be selected from CBWs mandatory supported in earlier release(s) otherwise a UE of earlier release(s) could not gain access to a cell indicating (unknown) UE CBW.

3.4 Answers to the LS

Based on the discussion above, answers to the LS would be:

Q1) Depending on UE CBW indication method (sec 3.2).

Q2) No and we need to define a method.

C1) The case is not likely (sec 3.3).

4	Conclusion

This paper is to discuss an LS from RAN2 on UE CBW indication during RRC_idle. Either initial BWP bandwidth or channel bandwidth can be used for this purpose, with some constraints on values taken to avoid ambiguity. Decision is needed to select one of these.

Reference

R4-1902867		On UE CBW determination in RRC_idle state, SoftBank, RAN4#90bis (Xi’an)

R4-1907911	   LS on supported BW for initial BWP, RAN2, Nokia (R2-1908301)