3GPP RAN WG4 Meeting #92		R4-1908007

Ljubljana, Slovenia, 26-30th August 2019

Agenda item:	11.4.2

Source:	Apple Inc.

Title:	TP to 38.820 on the regulatory overview of 7-24 GHz for ITU Regions 1 and 2

WI/SI:	FS_7to24GHz_NR

Release:	Rel-16

Document for:	Approval

# 1	Introduction

The study on the 7 – 24 GHz frequency range [1] has made progress in identifying the technological feasibility of NR operation in this frequency range.  In an effort to complement the technical feasibility aspects of TR38.820 with appropriate regulation information in order to present a holistic view of NR feasibility in the 7-24 GHz frequency range, this contribution provides the initial input with the information for ITU Regions 1 and 2.

# 2	References

- RP-182884, “New SID on 7 - 24 GHz frequency range”

# 3	Text proposal (for approval)

<< begin text proposal >>

# 4	Regulatory landscape survey

## 4.1	General

Editor’s note: Conduct a regulatory landscape survey for each geographical area. This clause was planned to be completed once the WRC-19 conclusions will become available, but not later than RAN4#93.

The regulatory landscape survey for the frequency range from 7.125 to 24.25 GHz is provided in this clause.  Tables 4.1-1 and 4.1-2 below summarize the radio regulation.

Table 4.1-1: Frequency bands in the range 7 000 to 10 000 MHz in radio regulation

| Frequency band (MHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 7 145-7 190 | Yes | Yes | EESS (passive) and SRS (passive), footnote 5.458 |
| 7 145-7 190 | Yes | Yes | SRS (deep space) (Earth-to-space). EESS (passive) and SRS (passive), footnote 5.458 |
| 7 190-7 235 | Yes | Yes | EESS (passive) and SRS (passive) (Earth-to-space), footnote 5.458. Footnote 5.460A – EESS space stations shall not claim protection from existing and future stations in the fixed and mobile services. |
| 7 235-7 250 | Yes | Yes | EESS (passive) (Earth-to-space) and SRS (passive), footnote 5.458. Footnote 5.460A – EESS space stations shall not claim protection from existing and future stations in the fixed and mobile services. |
| 7 250-7 300 | Yes | Yes |  |
| 7 300-7 375 | Yes | Yes | Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services |
| 7 375-7 450 | Yes | Yes | Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services |
| 7 450-7 550 | Yes | Yes | METSAT (space-Earth) Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services |
| 7 550-7 750 | Yes | Yes | Footnote 5.461AB MMSS shall not claim protection from, nor constrain the use and development of, stations in the fixed and mobile, except aeronautical mobile services |
| 7 750-7 900 | Yes | Yes | METSAT (Space-to-Earth) |
| 7 900-8 025 | Yes | Yes |  |
| 8 025-8 175 | Yes | Yes | EESS (space-to-Earth). Footnote 5.463 Aircraft stations are not permitted in 8 025-8 400 MHz. |
| 8 175-8 215 | Yes | Yes | EESS (space-to-Earth), METSAT (Earth-to-space) Footnote 5.463 Aircraft stations are not permitted in 8 025-8 400 MHz. |
| 8 215-8 400 | Yes | Yes | EESS (space-to-Earth) Footnote 5.463 Aircraft stations are not permitted in 8 025-8 400 MHz. |
| 8 400-8 500 | Yes | Yes | SRS (space-to-Earth), footnote 5.465 SRS is limited to deep space. |
| 8 500-8 550 | No | No | Radiolocation |
| 8 550-8 650 | No | No | EESS (active), Radiolocation, SRS (active) |
| 8 650-8 750 | No | No | Radiolocation |
| 8 750-8 850 | No | No | Radiolocation and Aeronautical Radionavigation |
| 8 850-9 000 | No | No | Radiolocation and Maritime Radionavigation |
| 9 000-9 200 | No | No | Radiolocation and Aeronautical Radionavigation |
| 9 200-9 300 | No | No | EESS (active), Radiolocation, Maritime Radionavigation, footnote 5.474 search and rescue transponders may be used. |
| 9 300-9 500 | No | No | EESS (active), Radiolocation, Radionavigation, SRS (active), footnote 5.474 search and rescue transponders may be used. |
| 9 500-9 800 | No | No | EESS (active), Radiolocation, Radionavigation, SRS (active) |
| 9 800-9 900 | No | No | Radiolocation, Fixed service secondary |
| 9 900-10 000 | No | No | EESS (active), Radiolocation. Fixed service secondary, footnote 5.478B EESS (active) shall not cause harmful interference to, nor claim protection from, stations in the fixed service. |

Table 4.1-2: Frequency bands in the range 10 to 24 GHz in radio regulation

| Frequency band (GHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 10-10.4 | No | No | EESS (active), Radiolocation, footnote 5.478B EESS (active) shall not cause harmful interference to, nor claim protection from, stations in the fixed service. Also used by weather radar on a secondary basis, footnote 5.479 |
| 10.4-10.45 | Yes excluding R2 | Yes excluding R2 | Radiolocation |
| 10.45-10.5 | No | No | Radiolocation, amateur and amateur-satellite secondary |
| 10.5-10.55 | Yes | Yes | Radiolocation secondary R1 |
| 10.55-10.6 | Yes | Yes | Radiolocation secondary |
| 10.6-10.68 | Yes | Yes | EESS (passive), RAS (footnote 5.149), SRS (passive), Radiolocation secondary. Footnote 5.482A EESS (passive) and mobile and fixed services, Res. 751 applies |
| 10.68-10.7 | No | No | EESS (passive), RAS, SRS (passive) Footnote 5.340 – all emissions prohibited |
| 10.7-10.95 | Yes | Yes | Footnote 5.441 FSS Appendix 30B band |
| 10.95-11.2 | Yes | Yes | Footnote 5.848B Res. 155 applies – unmanned aircraft systems using FSS |
| 11.2-11.45 | Yes | Yes | Footnote 5.441 FSS Appendix 30B |
| 11.45-11.7 | Yes | Yes | Footnote 5.484B Res. 155 applies – unmanned aircraft systems using FSS. |
| 11.7-12.5 | Yes, except 11.7-12.2 GHz in R2 | Yes, except 12.1-12.2 in R2 | BS and BSS Footnotes 4.587, 5.488, 5.490, and 5.493 denotes Appendix 30 and 30B planned bands |
| 12.5-12.75 | Yes, except R1 | Yes, except R1 | R3 BSS Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS. |
| 12.75-13.25 | Yes | Yes | SRS (deep space) (space-to-Earth) secondary Footnote 5.441 FSS Appendix 30B |
| 13.25-13.4 | No | No | EESS (active), ARNS, SRS (active) |
| 13.4-13.65 | No | No | EESS (active), Radiolocation, SRS, Standard frequency and time signal-satellite (Earth-to-space) |
| 13.65-13.75 | No | No | EESS (active), Radiolocation, SRS (active), standard frequency and time signal-satellite (Earth-to-space) secondary |
| 13.75-14 | No | No | Radiolocation, standard frequency and time signal-satellite (Earth-space) |
| 14-14.25 | No | No | Radionavigation Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.25-14.3 | No | No | Radionavigation Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.3-14.4 | Yes, except R2 | Yes, except R2 | Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.4-14.47 | Yes | Yes | Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.47-14.5 | Yes | Yes | Radio astronomy – Footnote 5.149 Footnote 5.484B – Res 155 applies – unmanned aircraft systems using FSS |
| 14.5-14.75 | Yes | Yes | Res 163 and 164 restriction to FSS to protect aeronautical mobile |
| 14.75-14.8 | Yes | Yes | Res 163 and 164 restriction to FSS to protect aeronautical mobile |
| 14.8-15.35 | Yes | Yes | Footnote 5.339 also allocated to EESS and SRS (passive) secondary |
| 15.35-15.4 | No | No | EESS (passive), Radio Astronomy, SRS (passive) Footnote 5.340 – all emission prohibited |
| 15.4-15.43 | No | No | Radiolocation and radionavigation |
| 15.43-15.63 | No | No | Radiolocation and radionavigation |
| 15.63-15.7 | No | No | Radiolocation and ARNS |
| 15.7-16.6 | No | No | Radiolocation Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 16.6-17.1 | No | No | Radiolocation, SRS (deep space) (Earth-to-space) Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 17.1-17.2 | No | No | Radiolocation Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 17.2-17.3 | No | No | EESS (active), Radiolocation, SRS (active) Footnote 5.513 – mobile and fixed primary, but subject to not claiming protection from or causing interference to services operating in accordance to the Table. |
| 17.3-17.7 | No | No | Footnote 5.515 Appendix 30A Footnote 5.516B High density FSS applications (in Region 1) |
| 17.7-18.1 | Yes, excluding Region 2 (17.7-17.8) | Yes | Footnote 5.515 Appendix 30A (Region 2 17.7-17.8) |
| 18.1-18.4 | Yes | Yes | Footnote 5.516B High density applications in FSS (in Region 2) |
| 18.4-18.6 | Yes | Yes | Footnote 5.516B High density applications in FSS (in Region 2) |
| 18.6-18.8 | Yes | Yes | EESS (passive), SRS (passive) Footnote 5.516B High density applications in FSS (in Region 2) |
| 18.8-19.3 | Yes | Yes | 5.516B High density FSS applications (in Region 2) |
| 19.3-19.7 | Yes | Yes |  |
| 19.7-20.1 | No | No | 5.484B Res 155 applies – unmanned aircraft systems using FSS 5.516B High density FSS applications |
| 20.1-20.2 | No | No | 5.484B Res 155 applies – unmanned aircraft systems using FSS 5.516B High density FSS applications |
| 20.2-21.2 | No | No | Standard frequency and time signal-satellite (space-to-Earth) |
| 21.2-21.4 | Yes | Yes | EESS (passive) and SRS (passive) |
| 21.4-22 | Yes | Yes | BSS in Regions 1 and 3 Footnote 5.530A any station in the fixed or mobile services shall not produce a pfd in excess of -120.4 dB(W/m2 • MHz) at 3m above the ground for more than 20% of the time. (see Rec. ITU-R BO.1898) Footnote 5.530B Admins in R1 and R3 are encouraged no to deploy stations in the mobile service and are encouraged to limit the deployment of FS ptp links |
| 22-22.21 | Yes | Yes | Footnote 5.149 Radio astronomy |
| 22.21-22.5 | Yes | Yes | EESS (passive), Radio astronomy, SRS (passive), Footnote 5.149 Footnote 5.532 EESS and SRS (passive) shall not impose constraints on the FS and MS |
| 22.5-22.55 | Yes | Yes | Not applicable |
| 22.55-23.15 | Yes | Yes | SRS (Earth-to-space) Footnote 5.149 Radio astronomy Footnote 5.338A – Res 750 applies (passive emission limits) |
| 23.15-23.55 | Yes | Yes | Footnote 5.338A – Res 750 applies (passive emission limits) |
| 23.55-23.6 | Yes | Yes | WRC-19 Agenda item 1.13 determining IMT adjacent emissions limits for protection of passive services in the 23.6-24 GHz result would most likely be applicable to this band. |
| 23.6-24 | No | No | EESS (passive), Radio astronomy, SRS (passive) Footnote 5.340 all emissions prohibited – unwanted emissions from IMT is under review WRC-19 Agenda item 1.13 |

## 4.2	ITU Region 1

### 4.2.1	Europe and CEPT

General information for usages in CEPT can be found in THE EUROPEAN TABLE OF FREQUENCY ALLOCATIONS AND APPLICATIONS IN THE FREQUENCY RANGE 8.3 kHz to 3000 GHz (ECA TABLE). When it comes to Regulation about Short Range Devices additional information can be found in Recommendation ERC/REC 70-03 "Relating to the use of Short Range Devices (SRD)"(*). Where a corresponding EC decision exists it is marked with (+).  Unwanted emission limits can be found in ECC Recommendation (02)/05 "Unwanted Emissions" and ERC/RECOMMENDATION 74-01 "Unwanted emissions in the spurious domain".

Table 4.2.1-1: Frequency bands in the range 7 to 24 GHz

| Frequency range in MHz | Allocations/Applications | ECC Dec/Rec |
| --- | --- | --- |
| 7 145 – 7 190 | Fixed PMSE Radiodetermination applications UWB | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 7 190 – 7 235 | Fixed PMSE EESS (passive) Radiodetermination applications UWB | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 7 235 – 7 250 | Fixed PMSE EESS (passive) Radiodetermination applications UWB | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 7 250 – 7 450 | Fixed PMSE Land Mobile (military) Radiodetermination applications UWB Satellite Systems (military) MSS Earth stations | ECC/REC/(02)06 ERC/REC 25-10 NATO NJFA Extract ECC/DEC/(11)02*+ ECC/DEC/(06)04+ NATO NJFA Extract |
| 7 450 – 7 550 | Fixed PMSE Land Mobile (military) Radiodetermination applications UWB Satellite Systems (military) Weather satellites | ECC/REC/(02)06 ERC/REC 25-10 NATO NJFA Extract ECC/DEC/(11)02*+ ECC/DEC/(06)04+ NATO NJFA Extract |
| 7 550 – 7 750 | Fixed PMSE Land Mobile (military) Radiodetermination applications UWB Satellite Systems (military) | ECC/REC/(02)06 ERC/REC 25-10 NATO NJFA Extract ECC/DEC/(11)02*+ ECC/DEC/(06)04+ NATO NJFA Extract |
| 7 750 – 7 900 | Fixed PMSE Radiodetermination applications UWB Weather satellites | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 7 900 – 8 025 | Fixed PMSE Land Mobile (military) Radiodetermination applications UWB Satellite Systems (military) MSS Earth stations | ECC/REC/(02)06 ERC/REC 25-10 NATO NJFA Extract ECC/DEC/(11)02*+ ECC/DEC/(06)04+ NATO NJFA Extract |
| 8 025 – 8 215 | EESS (space-Earth) Fixed PMSE Land Mobile (military) Land Mobile Radiodetermination applications UWB Satellite Systems (military) | ECC/REC/(02)06 ERC/REC 25-10 NATO NJFA Extract ECC/DEC/(11)02*+ ECC/DEC/(06)04+ NATO NJFA Extract |
| 8 215 – 8 400 | EESS (space-Earth) Fixed PMSE Land Mobile (military) Radiodetermination applications UWB Satellite Systems (military) Radio Astronomy | ECC/REC/(02)06 ERC/REC 25-10 NATO NJFA Extract ECC/DEC/(11)02*+ ECC/DEC/(06)04+ NATO NJFA Extract |
| 8400 - 8500 | Fixed PMSE Radiodetermination applications UWB Space Research | ECC/REC/(02)06 ERC/REC 25-10 ECC/DEC/(11)02*+ ECC/DEC/(06)04+ |
| 8 500 – 8 550 | Aeronautical military systems Aeronautical navigation Radiodetermination applications UWB Radiolocation (civil) Radiolocation (military) | NATO NJFA Extract *+ ECC/DEC/(06)04+ NATO NJFA Extract |
| 8 550 – 8 650 | Active sensors (satellite) Aeronautical military systems Aeronautical navigation Radiodetermination applications UWB Radiolocation (civil) Radiolocation (military) | NATO NJFA Extract *+ ECC/DEC/(06)04+ NATO NJFA Extract |
| 8 650 – 9 000 | Aeronautical military systems Aeronautical navigation Radiodetermination applications UWB Radiolocation (civil) Radiolocation (military) | NATO NJFA Extract *+ ECC/DEC/(06)04+ NATO NJFA Extract |
| 9 000 – 9 200 | Aeronautical military systems Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) | NATO NJFA Extract *+ NATO NJFA Extract |
| 9 200 – 9 300 | Aeronautical military systems Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) Synthetic aperture radar | NATO NJFA Extract *+ NATO NJFA Extract |
| 9 300 – 9 500 | Aeronautical military systems Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) Satellite systems (military) Weather radar | NATO NJFA Extract *+ NATO NJFA Extract NATO NJFA Extract |
| 9 500 – 9 800 | Active sensors (satellite) Aeronautical military systems Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) Satellite systems (military) | NATO NJFA Extract *+ NATO NJFA Extract NATO NJFA Extract |
| 9 800 – 9 900 | Aeronautical military systems Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) Satellite systems (military) | NATO NJFA Extract *+ NATO NJFA Extract NATO NJFA Extract |
| 9 900 – 10 000 | Aeronautical military systems Aeronautical navigation Radiodetermination applications Radiolocation (civil) Radiolocation (military) Satellite systems (military) Synthetic aperture radar | NATO NJFA Extract *+ NATO NJFA Extract NATO NJFA Extract |
| 10 000 – 10 400 | Aeronautical military systems Amateur FWA Fixed Land Mobile (military) Maritime Mobile (military) PMSE Radiodetermination applications Radiolocation (civil) Radiolocation (military) Synthetic aperture radar | NATO NJFA Extract ERC/REC 12-05 NATO NJFA Extract NATO NJFA Extract ERC/REC 25-10 *+ NATO NJFA Extract |
| 10 400 – 10 450 | Aeronautical military systems Amateur Land Mobile (military) Maritime Mobile (military) PMSE Radiodetermination applications Radiolocation (civil) Radiolocation (military) | NATO NJFA Extract NATO NJFA Extract NATO NJFA Extract ERC/REC 25-10 *+ NATO NJFA Extract |
| 10 400 – 10 500 | Aeronautical military systems Amateur Amateur-satellite Land Mobile (military) Maritime Mobile (military) PMSE Radiodetermination applications Radiolocation (civil) Radiolocation (military) | NATO NJFA Extract NATO NJFA Extract NATO NJFA Extract ERC/REC 25-10 *+ NATO NJFA Extract |
| 10 500 – 10 600 | Fixed PMSE Radiodetermination applications | ERC/REC 12-05 ERC/REC 25-10 *+ |
| 10 600 – 10 680 | Fixed PMSE Passive sensors (satellite) Radio astronomy | ERC/REC 12-05 ERC/REC 25-10 ECC/DEC/(10)01 ECC/DEC/(10)01 |
| 10 680 – 10 700 | Passive Sensors Radio astronomy |  |
| 10 700 – 10 950 | FSS Fixed | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 Earth Stations ERC/DEC/(00)08 HEST ECC/DEC/(06)03 LEST ECC/DEC/(06)02 NGSO ECC Decision (18)05 NGSO ECC/DEC/(17)04 ERC/REC 12-06 |
| 10 950 – 11 700 | FSS Fixed | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 NGSO ECC Decision (18)05 NGSO ECC/DEC/(17)04 ERC/REC 12-06 |
| 11 700 – 12 500 | Broadcast satellite FSS | ERC/DEC/(00)08 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 HEST ECC/DEC/(06)03 LEST ECC/DEC/(06)02 NGSO ECC Decision (18)05 NGSO ECC/DEC/(17)04 |
| 12 500 – 12 750 | FSS | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 Earth Stations ERC/DEC/(00)08 HEST ECC/DEC/(06)03 LEST ECC/DEC/(06)02 NGSO ECC Decision (18)05 NGSO ECC/DEC/(17)04 |
| 12 750 – 13 250 | FSS Fixed | ERC/REC 12-02 |
| 13 250 – 13 400 | Active sensors (satellite) Airborne doppler navigation aids Maritime radar |  |
| 13 400 – 13 650 | Active sensors (satellite) Airborne doppler navigation aids Maritime radar FSS Radiodetermination applications Radiolocation (military) | *+ NATO NJFA Extract |
| 13 650 – 13 750 | Active sensors (satellite) Airborne doppler navigation aids Maritime radar Radiodetermination applications Radiolocation (military) | * NATO NJFA Extract |
| 13 750 – 14 000 | FSS Passive sensors (satellite) Maritime radar Radiodetermination applications Radiolocation (military) | * NATO NJFA Extract |
| 14 000 – 14 250 | FSS | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 Earth Stations ERC/DEC/(00)08 HEST ECC/DEC/(06)03 LEST ECC/DEC/(06)02 NGSO ECC Decision (18)05 NGSO ECC/DEC/(17)04 |
| 14 250 – 14 300 | FSS MSS Earth stations | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 NGSO ECC/DEC/(17)04 VSAT ERC/REC 13-03 |
| 14 300 – 14 470 | FSS MSS Earth stations | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 NGSO ECC/DEC/(17)04 VSAT ERC/REC 13-03 |
| 14 470 – 14 500 | FSS MSS Earth stations Radio astronomy | AES ECC/DEC/(05)11 ESIM ECC/DEC/(18)04 ESIM ECC/DEC/(18)05 ESV ECC/DEC/(05)10 NGSO ECC/DEC/(17)04 VSAT ERC/REC 13-03 |
| 14 500 – 14 750 | Aeronautical military systems Fixed Land military systems related Maritime military systems related Radio astronomy | NATO NJFA Extract ERC/REC 12-07 NATO NJFA Extract NATO NJFA Extract |
| 14 750 – 14 800 | Aeronautical military systems Land military systems related Maritime military systems related Radio astronomy | NATO NJFA Extract NATO NJFA Extract NATO NJFA Extract |
| 14 800 – 15 350 | Aeronautical military systems Fixed Land military systems related Maritime military systems related Radio astronomy | NATO NJFA Extract ERC/REC 12-07 NATO NJFA Extract NATO NJFA Extract |
| 15 350 – 15 400 | Passive sensors (satellite) Radio astronomy |  |
| 15 400 – 15 430 | Airborne doppler navigation aids Radiolocation (civil) |  |
| 15 430 – 15 630 | Airborne doppler navigation aids FSS Radiolocation (civil) |  |
| 15 630 – 15 700 | Airborne doppler navigation aids Radiolocation (civil) |  |
| 15 700 – 17 100 | Radiolocation (military) | NATO NJFA Extract |
| 17 100 – 17 300 | Ground Based SAR Radiolocation (military) | * NATO NJFA Extract |
| 17 300 – 17 700 | FSS Radiolocation (military) | GSO ESOMP ECC/DEC/(13)01 NGSO ESOMP ECC/DEC/(15)04 Earth Stations ECC/DEC/(05)08 NATO NJFA Extract |
| 17 700 – 18 600 | Fixed FSS | ERC/DEC/(00)07 ERC/REC 12-03 Earth Stations ERC/DEC/(00)07 GSO ESOMP ECC/DEC/(13)01 NGSO ESOMP ECC/DEC/(15)04 |
| 18 600 – 18 800 | Fixed FSS Passive sensors (passive) | ERC/DEC/(00)07 ERC/REC 12-03 Earth Stations ERC/DEC/(00)07 GSO ESOMP ECC/DEC/(13)01 NGSO ESOMP ECC/DEC/(15)04 |
| 18 800 – 19 700 | Fixed FSS | ERC/DEC/(00)07 ERC/REC 12-03 Earth Stations ERC/DEC/(00)07 GSO ESOMP ECC/DEC/(13)01 NGSO ESOMP ECC/DEC/(15)04 |
| 19 700 – 20 200 | FSS MSS Earth stations | Earth Stations ERC/DEC/(00)07 GSO ESOMP ECC/DEC/(13)01 NGSO ESOMP ECC/DEC/(15)04 HEST ECC/DEC/(06)03 LEST ECC/DEC/(06)02 |
| 20 200 – 21 200 | MSS Earth stations Satellite systems (military) | NATO NJFA Extract |
| 21 200 – 21 400 | PMSE | ERC/REC 25-10 |
| 21 400 – 22 000 | Broadcasting (satellite) PMSE | ERC/REC 25-10 |
| 22 000 – 23 150 | Fixed PMSE Radio astronomy | T/R 13-02 ERC/REC 25-10 |
| 23 150 – 23 600 | Fixed PMSE | T/R 13-02 ERC/REC 25-10 |
| 23 600 – 24 000 | Passive sensors (satellite) Radio astronomy |  |

## 4.3	ITU Region 2

### 4.3.1	USA

FCC rules part(s) and additional requirements are rules and regulations that pertain to specified frequency ranges and detailed description of the rules and regulations are listed in FCC code of federal regulations Title 47 Part 2.

Table 4.2.3.1-1: USA spectrum allocation between 7 GHz and 24 GHz

| Frequency [GHz] | Allocations | Governing FCC Rule Part(s) |
| --- | --- | --- |
| 7125-7145 | FIXED | General Rules and Regulations (2.105- G116) RF Devices (15) |
| 7145-7190 | FIXED SPACE RESEARCH (deep space) (passive) | General Rules and Regulations (2.105- G116, US262, 5.548) RF Devices (15) |
| 7190-7235 | FIXED SPACE RESEARCH (Earth-to-space) | General Rules and Regulations (2.105 -G113, G134, US262, 5.458) RF Devices (15) |
| 7235-7250 | FIXED | General Rules and Regulations (2.105 – 5.458) RF Devices (15) |
| 7250-7300 | FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) Fixed | General Rules and Regulations (2.105 -G117) |
| 7300-7450 | FIXED FIXED-SATELLITE (space-to-Earth) Mobile-satellite (space-to-Earth) | General Rules and Regulations (2.105 – G117) |
| 7450-7550 | FIXED FISED-SATELLITE (space-to-Earth) METEROLOGICAL-SATELLITE (space-to-Earth) Mobile-satellite (space-to-Earth) | General Rules and Regulations (2.105 -G105, G117) |
| 7550-7750 | FIXED FIXED-SATELLITE (space-to-Earth) Mobile-satellite (space-to-Earth) | General Rules and Regulations (2.105 – G117) |
| 7750-7900 | FIXED METEROLOGICAL-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105 – 5.461B) |
| 7900-8025 | FIXED-SATELLITE (Earth-to-space) MOBILE-SATELLITE (Earth-to-space) Fixed | General Rules and Regulations (2.105 – G117) |
| 8025-8175 | EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) (no airborne transmissions) | General Rules and Regulations (2.105 -G117, US258) |
| 8175-8215 | EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) METEROLOGICAL-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) (no airborne transmissions) | General Rules and Regulations (2.105 - G104, G117, US258) |
| 8215-8400 | EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) (no airborne transmissions) | General Rules and Regulations (2.105 - G117, US258) |
| 8400-8450 | FIXED SPACE RESEARCH (deep space) (space-to-Earth) | General Rules and Regulations (2.105) |
| 8450-8500 | FIXED SPACE RESEARCH (space-to-Earth) | General Rules and Regulations (2.105) |
| 8500-8550 | RADIOLOCATION | General Rules and Regulations (2.105 -G59) Private Land Mobile (90) |
| 8550-8650 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) | General Rules and Regulations (2.105 -G59) Private Land Mobile (90) |
| 8.65-9 | RADIOLOCATION | General Rules and Regulations (2.105 -US53) Aviation (87) Private Land Mobile (90) |
| 9-9.2 | AERONAUTICAL RADIONAVIGATION RADIOLOCATION | General Rules and Regulations (2.105 -G2, G19, 5.337, 5.473A) Aviation (87) Private Land Mobile (90) |
| 9.2-9.3 | MARITIME RADIONAVIGATION Radiolocation | General Rules and Regulations (2.105 – G59, US 110, 5.472, 5.474) Maritime (80) Private Land Mobile (90) |
| 9.3-9.5 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION RADIONAVIGATION SPACE RESEARCH (active) Meteorological aids | General Rules and Regulations (2.105 -G56, US475, US71, US67, US476A, 5.427, 5.474, 5.475A, 5.475B) Maritime (80) Aviation (87) Private Land Mobile (90) |
| 9.5-9.8 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) | General Rules and Regulations (2.105) Private Land Mobile (90) |
| 9.8-9.9 | RADIOLOCATION Earth exploration-satellite (active) Space research (active) | General Rules and Regulations (2.105) Private Land Mobile (90) |
| 9.1-10 | RADIOLOCATION | General Rules and Regulations (2.105, 5.479) Private Land Mobile (90) |
| 10-10.5 | RADIOLOCATION Amateur Amateur-satellite (10.45-10.5) | General Rules and Regulations (2.105, G32, NG50, US108, US128, 5.479) Private Land Mobile (90) Amateur Radio (97) |
| 10.5-10.55 | RADIOLOCATION | General Rules and Regulations (2.105, US59) Private Land Mobile (90) |
| 10.55-10.6 | FIXED | FIXED MICROWAVE (101) |
| 10.6-10.68 | EARTH EXPLORATION-SATELLITE (passive) FIXED SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US120, US131, US482) FIXED MICROWAVE (101) |
| 10.68-10.7 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US74, US131, US246) |
| 10.7-11.7 | FIXED FIXED-SATELITE (space-to-Earth) | General Rules and Regulations (2.105, US131, US211, NG52, 5.441) Satellite Communications (25) Fixed Microwave (101) |
| 11.7-12.2 | FIXED-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, NG55, NG143, 5.458, 5.488) Satellite Communications (25) |
| 12.2-12.7 | FIXED BROADCASTING-SATELLITE | General Rules and Regulations (2.105, 5.487A, 5.488, 5.490) Satellite Communications (25) Fixed Microwave (101) |
| 12.7-12.75 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | General Rules and Regulations (2.105, NG118) TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |
| 12.75-13.25 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | General Rules and Regulations (2.105, NG52, NG53, NG57, NG118, US251, 5.441) Satellite Communications (25) TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |
| 13.25-13.4 | EARTH EXPLORATION-SATELLITE (active) AERONAUTICAL RADIONAVIGATION SPACE RESEARCH (active) | General Rules and Regulations (2.105, 5.497, 5.498A) Aviation (87) |
| 13.4-13.75 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH Standard frequency and time signal-satellite (Earth-to-space) | General Rules and Regulations (2.105, G59, 5.501A, 5.501B) Private Land Mobile (90) |
| 13.75-14 | RADIOLOCATION FIXED-SATELLITE (Earth-to-space) Standard frequency and time signal-satellite (Earth-to-space) Space research | General Rules and Regulations (2.105, G59, US337, US356, US357) Satellite Communications (25) Private Land Mobile (90) |
| 14-14.2 | FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) Space research | General Rules and Regulations (2.105, NG55, US133) Satellite Communications (25) |
| 14.2-14.4 | FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) | General Rules and Regulations (2.105, NG55) Satellite Communications (25) |
| 14.4-14.47 | FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) Fixed Mobile | General Rules and Regulations (2.105, NG55) Satellite Communications (25) |
| 14.47-14.5 | FIXED-SATELLITE (Earth-to-space) Mobile-satellite (Earth-to-space) Fixed Mobile | General Rules and Regulations (2.105, NG55, US115, US133, US342) Satellite Communications (25) |
| 14.5-14.7145 | FIXED Mobile Space research | General Rules and Regulations (2.105) |
| 14.7.145-14.8 | FIXED Mobile Space research | General Rules and Regulations (2.105) |
| 14.8-15.1365 | MOBILE SPACE RESEARCH Fixed | General Rules and Regulations (2.105, US310) |
| 15.1365-15.35 | FIXED SPACE RESEARCH Mobile | General Rules and Regulations (2.105, US211, 5.339) |
| 15.35-15.4 | EARTH EXPLORATION-SATELLITE RADIO ASTRONOMY SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US74, US246) |
| 15.4-15.43 | RADIOLOCATION AERONAUTICAL RADIONAVIGATION | General Rules and Regulations (2.105, US211, US260, US511E, 5.511E, 5.11F) Aviation (87) |
| 15.43-15.63 | RADIOLOCATION FIXED-SATELLITE (Earth-to-space) AERONAUTICAL RADIONAVIGATION | General Rules and Regulations (2.105,US211, US260, US359, US511E, 5.511C, 5.511E, 5.511F) Satellite Communications (25) Aviation (87) |
| 15.63-15.7 | RADIOLOCATION AERONAUTICAL RADIONAVIGATION | General Rules and Regulations (2.105, US211, US260, US511E, 5.511E, 5.511F) Aviation (87) |
| 15.7-16.6 | RADIOLOCATION | General Rules and Regulations (2.105, G59) Private Land Mobile (90) |
| 16.6-17.1 | RADIOLOCATION Space research (deep space) (Earth-to-space) | General Rules and Regulations (2.105, G59) Private Land Mobile (90) |
| 16.6-17.1 | RADIOLOCATION | General Rules and Regulations (2.105, G59) Private Land Mobile (90) |
| 17.1-17.2 | RADIOLOCATION | General Rules and Regulations (2.105, G59) Private Land Mobile (90) |
| 17.2-17.3 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) | General Rules and Regulations (2.105, G59) Private Land Mobile (90) |
| 17.3-17.7 | FIXED-SATELLITE (Earth-to-space) BROADCASTING-SATELLITE Radiolocation | General Rules and Regulations (2.105, G59, G117, US259, US271, US402, NG163) Satellite Communications (25) |
| 17.7-17.8 | FIXED FIXED-SATELLITE (Earth-to-space) | General Rules and Regulations (2.105, G117, US271, US334) Satellite Communications (25) TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |
| 17.8-18.3 | FIXED FIXED-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, G117, US334, US519) Satellite Communications (25) TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |
| 18.3-18.6 | FIXED-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, G117, US139, US334) Satellite Communications (25) |
| 18.6-18.8 | EARTH EXPLORATION-SATELLITE (passive) FIXED-SATELLITE (space-to-Earth) SPACE RESEARCH (passive) | General Rules and Regulations (2.105, G117, NG164 US139, US225, US254, US334) Satellite Communications (25) |
| 18.8-19.3 | FIXED-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, G117, NG165, US139, US334) Satellite Communications (25) |
| 19.3-19.7 | FIXED FIXED-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, G117, NG166, US139, US334) Satellite Communications (25) TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |
| 19.7-20.2 | FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) | General Rules and Regulations (2.105, G117, US139, US334, 5.525, 5.526, 5.527, 5.528, 5.529) Satellite Communications (25) |
| 20.2-21.2 | FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) Standard frequency and time signal-satellite (space-to-Earth) | General Rules and Regulations (2.105, G117) |
| 21.2-21.4 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US532) Fixed Microwave (101) |
| 21.4-22 | FIXED MOBILE | General Rules and Regulations (2.105, US211) Fixed Microwave (101) |
| 22-22.21 | FIXED MOBILE (except aeronautical mobile) | General Rules and Regulations (2.105) Fixed Microwave (101) |
| 22.21-22.5 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE (except aeronautical mobile) RADIO ASTRONOMY SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US342, US532) Fixed Microwave (101) |
| 22.5-22.55 | FIXED MOBILE | General Rules and Regulations (2.105, US211) Fixed Microwave (101) |
| 22.55-23.15 | FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (Earth-to-space) | General Rules and Regulations (2.105, US145, US278, US342, 5.532A) Satellite Communications (25) Fixed Microwave (101) |
| 23.15-23.55 | FIXED INTER-SATELLITE MOBILE | General Rules and Regulations (2.105, US145, US278) Satellite Communications (25) Fixed Microwave (101) |
| 23.55-23.6 | FIXED MOBILE | General Rules and Regulations (2.105) Fixed Microwave (101) |
| 23.6-24 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | General Rules and Regulations (2.105, US74, US246) |

<< end text proposal >>